| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC- H0489-v3 WG11 number: |
| --- | --- |

| Title: | Complexity analysis of high throughput CABAC Entropy codecs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alberto Dueñas, Prashant Arora , Oscar Patino and Francisco Javier Roncero 2315 N. First Street, San Jose, CA 95131 USA | Tel: Email: | +1-408-431-0635 alberto@cavium.com prashant.arora@cavium.com oscar.patino@cavium.com kroncero@cavium.com |
| Source: | Cavium Inc. |  |  |

_____________________________

# Abstract

This contribution presents the importance of defining a method to analyze the complexity of high throughput CABAC Entropy codec schemes. This contribution presents an example analysis of the current entropy codec method part of the HEVC WD5 [1]. This contribution suggests that JCT-VC establishes a breakout group (BoG) in order to analyze the complexity of different options for a high throughput CABAC Entropy codec.

- Background

The need for a high throughput CABAC entropy codec scheme was identified at the 7th JCT-VC meeting after the decision to simplify the design by having a single entropy codec method.

It has been traditionally challenging to identify a reliable method that can be used to analyze the implementation complexity of different entropy codec methods and to do it for software and hardware implementations at the same time.

This contribution outlines a method that analyzes the implementation complexity of different entropy codec methods; in addition it reaches some conclusions in relation to the current entropy scheme used in HEVC WD5.

# Creation of a method to analyze the implementation complexity of different entropy codec methods

A reliable scheme to analyze the implementation complexity of different entropy codec methods needs to focus on the worst case scenarios; it is on those circumstances where the complexity of different entropy codec methods should be evaluated. Implementations of the different entropy codec methods will need to be able to deal with those cases and have a design that will be able to perform it correctly in all cases. An implementation of an entropy codec method will need to operate satisfactory in all cases, including the most complex ones and that is why the analysis in those circumstances is most relevant from the implementation point of view.

Current common conditions and software reference configurations [2] may not be used to analyze the implementation complexity of different entropy codec methods as they are not representative of the worst case scenarios in which the entropy codec will need to operate.

# Example of the analysis of the implementation complexity of current working draft entropy codec method

As stated on the previous section it is very important to focus on the worst case scenarios when analyzing complexity of an entropy codec method and we are going to do on this analysis.

In the current design the syntax elements of a residual block are as follows:

| last_significant_coeff_x, last_significant_coeff_y |  |
| --- | --- |
| significant_coeff_flag |  |
| coeff_abs_level_greater1_flag |  |
| coeff_abs_level_greater2_flag |  |
| coeff_abs_level_minus3 |  |
| coeff_sign_flag |  |

These syntax elements will be the most commonly used in the worst case scenarios.

The highlighted ones use coded bins. The coeff_abs_level has two codec bins and one codec bin per coefficient for significant_coeff_flag. This indicates that the worst case scenarios for the current WD5 entropy codec method happens when we have 3 coded bins per coefficient (this was greatly reduced from 14 coded bins before JCTVC-E253[3] was adopted). The rest of the bins for the coefficients are by-pass bins that can be implemented in a parallel manner. The non-coefficient bins do not have a substantial contribution when we do operate at high bit rates as it is described in JCTVC-G569[4].

Identifying that these are the most difficult conditions to create an implementation for the entropy codec method under analysis, we can continue to identify some complexity aspects of a design that will have to deal with those circumstances.

The following assumptions have been used in this analysis:

*	Traditionally there are 1.5 bins per coded bit.

*	Traditionally 1 bin can be processed per clock cycle on the decoding process.

*	16x16 TU size is used for simplification.

*	4:2:0 Chrominance format is used, therefore 1.5 coefficients per pixel.

In this case, this will lead to a number of bins per TU equal to 3*16*16*1.5, which is 1152 Bins per TU.

When processing a video sequence with 1200*1920 resolution, the image will have 9000 TUs per frame and if the frame rate is 60 frames per second. This will produce the worst case for this high resolution sequence at 1152*9000*60 bins per second, which is 622 080 000 bins per second.

Using our assumption of 1.5 bins per processed bit (for pixel rate used on this example) for any bit rate that is lower than 414.7 megabits per second, the coefficient bins are actually codec bins and no by-pass bins are used. For the pixel rate used on this example and for bit rates higher than 414.7 megabits per second it is guaranteed that some of the coefficient bins will be processed as by-pass bins and the complexity will very close to the value required for processing 414.7 Mbps. It is important to note that this inflection point is quite high for the pixel rate used on this example.

We use the assumption that 1coded bin can be processed per cycle on the decoding process. In that case the current entropy codec method part of the HEVC WD5 will need 1.5 cycles per bit to be able to perform the decoding process on the worst case scenarios under analysis for bit rates below 414.7 megabits per second for the pixel rate used on this example. At the worst case bit-rate of 414.7 Mbps, this implies a clock frequency of 622 MHz to avoid any bottlenecks because of the number of coded bins on the current entropy codec method part of the HEVC WD5. Or, at a nominal frequency of 250 MHz, the maximum bitrate would be limited to 166 Mbps.

As a further example an entropy decoding method that do not process any coefficient data as codec bins could potentially have 3 times higher throughput when using the same frequency compared with the current entropy codec method part of the HEVC WD5 for bit rates below 414.7 megabits per second for the pixel rate used on this example. Or, at a nominal frequency of 250 MHz, the maximum bitrate would be limited to 500 Mbps.

# Conclusions and recommendations

It is very important to take into account the worst case scenario when analyzing the implementation complexity of different entropy codec methods. This will drive the implementation of this method.

This contribution suggests the need to create a reliable analysis methodology to evaluate different entropy codec methods.

This contribution recommends enhancing the common conditions and software reference configurations to be able to use the results to analyze the implementation complexity of different entropy codec methods.

This contribution suggests the creation of a breakout group (BoG) in order to discuss technical analysis of the complexity of different high throughput CABAC Entropy codec during the 8th JCT-VC meeting. This BoG could have the following mandates:

- Review all related inputs

- Identify the contributions that have the highest theoretical throughput for a given complexity

- Perform analysis of the advantages and disadvantages of each of those contributions

- Report the results and conclusions to the JCT-VC

# References

- [1]	B. Bross, W.-J. Han, J. R. Ohm, G. J. Sullivan, and T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- G1103, 7th Meeting: Geneva, CH, November 2011.

- F. Bossen, “Common conditions and software reference configurations,” in JCTVC-G1200, 7th Meeting: Geneva, CH, November 2011.

- T. Nguyen,“CE11: Coding of transform coefficient levels with Golomb-Rice codes,” in JCTVC-E253, 5th Meeting: Geneva, CH, March 2011.

- Jani Lainema et al. “Single entropy coder for HEVC with a high throughput binarization mode”, in JCTVC-G569, 7th Meeting: Geneva, CH, November 2011.

# Patent rights declaration(s)

Cavium Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).