| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0494 |
| --- | --- |

| Title: | CE3: Chroma interpolation filter with half pixel phase offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo Limin Wang Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-3893 limin.wang@motorola.com +1-858-404-3470 aluthra@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Motorola Mobility’s Chroma interpolation filter with half pixel offset for HEVC. The simulations were conducted using HM5.0 software with Motorola Mobility’s modifications. Two sets of fixed interpolation filters are tested.

Compared with the current interpolation filter in HM5.0, the proposed filter variant A achieves -0.02%/-0.60%/-0.68%, -0.01%/-0.81%/-0.83%/, 0.01%/-0.41%/-0.44%, -0.01%/-0.77%/-0.80%, -0.03%/1.32%/1.62%, -0.03%/-0.39%/-0.38%, -0.02%/-0.43%/-0.80%, 0.03%/2.71%/2.84%, -0.04%/-0.30%/-0.33% bitrate difference in RAHE, RALC, RAHE10, LBHE, LBLC, LBHE10, LPHE, LPLC, and LPHE10 settings; the proposed filter variant B achieves 0.00%/-0.25%/-0.29%, 0.00%/-0.29%/-0.36%, 0.01%/-0.16%/-0.17%, -0.06%/-0.34%/-0.30%, -0.04%/-0.04%/0.04%, -0.01%/-0.31%/-0.34%, 0.00%/-0.19%/-0.31%, -0.01%/0.10%/0.09%, -0.01%/-0.27%/-0.21% bitrate difference in RAHE, RALC, RAHE10, LBHE, LBLC, LBHE10, LPHE, LPLC, and LPHE10 settings. Additional experimental results show that on the proposed filter variant A achieves -0.02%/-0.84%/-1.01%, -0.04%/-1.13%/-1.12%, -0.03%/-0.79%/-1.13% bitrate difference in RALC with SAO, LBLC with SAO, and LPLC with SAO settings; the proposed filter variant B achieves 0.00%/-0.31%/-0.39%, -0.05%/-0.36%/-0.35%, -0.03%/-0.27%/-0.52% bitrate difference in RALC with SAO, LBLC with SAO, and LPLC with SAO settings.

Cross-check will be provided by Cannon, Samsung, and Toshiba. The attached spreadsheet contains detailed data of the results.

# Introduction

The target of this document is to verify the performance of Motorola Mobility’s Chroma interpolation filter [1] with half pixel offset for HEVC. The simulations were conducted using HM5.0 software [2] with Motorola Mobility’s modifications. Cross-check will be provided by Cannon, Samsung, and Toshiba [3-5].

Fig. 1 illustrates the positions of the resulting half-pel and quarter-pel pixels between full-pel pixels along a pixel line within an image, where L1, L0, R0, and R1 are full-pel pixels, and H is a half-pel pixel between L0 and R0.

Fig. 1 Sub-Pel Positions

# Proposed sub-pel interpolation filters

Variant A: {-7, 39, 39, -7}

Variant B: {-5, 37, 37, -5}

Both variant A and B have wider passband than the current HEVC half-pel Chroma filter {-4, 36, 36, -4}.

Fig. 2 Frequency Responses for Different Filters

# Simulation environment

## Software

The software used to run the simulations is HM5.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [6]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 14G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The experimental results of the rate-distortion performance and speed comparison are summarized in Table 1-4. Detailed results are included in the attachments (JCTVC-H0494(VariantA).xls, JCTVC-H0494 (VariantB).xls, JCTVC-H0494 (VariantA)LCwithSAO.xls, and JCTVC-H0494 (VariantB)LCwithSAO.xls).

Table 1 Rate-distortion performance and speed comparison of variant A

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.6% | 0.0% | -1.0% | -0.8% | 0.0% | -0.4% | -0.4% |
| Class B | 0.0% | -0.7% | -0.7% | 0.0% | -0.8% | -0.7% | 0.0% | -0.4% | -0.5% |
| Class C | 0.0% | -0.4% | -0.5% | 0.0% | -0.6% | -0.7% | 0.0% | -0.4% | -0.3% |
| Class D | 0.0% | -0.7% | -1.0% | 0.0% | -0.9% | -1.1% | 0.0% | -0.4% | -0.5% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Overall | -0.02% | -0.60% | -0.68% | -0.01% | -0.81% | -0.83% | 0.01% | -0.41% | -0.44% |
|  | 0.0% | -0.6% | -0.7% | 0.0% | -0.8% | -0.8% | 0.0% | -0.4% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -1.1% | -0.9% | 0.0% | 2.1% | 2.4% | 0.0% | -0.8% | -0.5% |
| Class C | 0.0% | -0.9% | -0.9% | 0.0% | 0.7% | 0.7% | 0.0% | -0.6% | -0.5% |
| Class D | 0.0% | -1.5% | -1.5% | 0.0% | 1.8% | 2.7% | -0.1% | -0.5% | -1.5% |
| Class E | 0.1% | 0.5% | 0.2% | 0.0% | 3.2% | 2.9% | 0.1% | 0.5% | 1.2% |
| Class F | -0.1% | -0.4% | -0.5% | -0.1% | -1.1% | -0.5% | -0.1% | -0.1% | -0.3% |
| Overall | -0.01% | -0.77% | -0.80% | -0.03% | 1.32% | 1.62% | -0.03% | -0.39% | -0.38% |
|  | 0.0% | -0.8% | -0.8% | 0.0% | 1.3% | 1.5% | 0.0% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.9% | -1.1% | 0.0% | 3.6% | 3.4% | 0.0% | -0.8% | -0.7% |
| Class C | 0.0% | -0.6% | -0.9% | 0.0% | 1.7% | 2.3% | 0.0% | -0.5% | -0.4% |
| Class D | 0.0% | -0.6% | -1.8% | 0.0% | 3.1% | 5.0% | 0.0% | -0.7% | -1.1% |
| Class E | 0.0% | -0.2% | 0.7% | 0.1% | 4.8% | 3.4% | 0.0% | 0.8% | 0.8% |
| Class F | 0.0% | 0.3% | -0.4% | 0.1% | 0.7% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall | -0.02% | -0.43% | -0.80% | 0.03% | 2.71% | 2.84% | -0.04% | -0.30% | -0.33% |
|  | 0.0% | -0.4% | -0.8% | 0.0% | 2.6% | 2.7% | 0.0% | -0.4% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 2 Rate-distortion performance and speed comparison of variant B

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.4% | -0.5% | 0.0% | -0.3% | -0.6% | 0.0% | -0.1% | -0.3% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% |
| Overall | 0.00% | -0.25% | -0.29% | 0.00% | -0.29% | -0.36% | 0.01% | -0.16% | -0.17% |
|  | 0.0% | -0.3% | -0.3% | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% | 0.4% | 0.0% | -0.4% | -0.4% |
| Class C | -0.1% | -0.3% | -0.2% | 0.0% | -0.1% | -0.3% | 0.0% | -0.4% | -0.3% |
| Class D | 0.0% | -0.8% | -0.6% | -0.1% | -0.1% | -0.2% | 0.0% | -0.6% | -1.0% |
| Class E | 0.0% | 0.2% | -0.1% | 0.0% | 0.5% | 0.9% | 0.0% | 0.0% | 0.6% |
| Class F | -0.3% | -0.3% | -0.4% | -0.1% | 0.0% | -0.4% | -0.1% | 0.0% | -0.4% |
| Overall | -0.06% | -0.34% | -0.30% | -0.04% | -0.04% | 0.04% | -0.01% | -0.31% | -0.34% |
|  | -0.1% | -0.4% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.4% | 0.0% | 0.4% | 0.0% | 0.0% | -0.3% | -0.6% |
| Class C | 0.0% | -0.3% | -0.5% | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.6% | -0.7% | -0.1% | -0.4% | 0.0% | 0.0% | -0.7% | -0.7% |
| Class E | 0.0% | 0.3% | 0.0% | 0.0% | 0.4% | 0.3% | 0.0% | -0.2% | 0.5% |
| Class F | 0.0% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| Overall | 0.00% | -0.19% | -0.31% | -0.01% | 0.10% | 0.09% | -0.01% | -0.27% | -0.21% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 3 Rate-distortion performance and speed comparison of variant A (LC settings with SAO)

|  | Random Access LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.9% | -0.9% |
| Class B | 0.0% | -0.9% | -0.8% |
| Class C | 0.0% | -0.7% | -0.8% |
| Class D | 0.0% | -0.9% | -1.5% |
| Class E |  |  |  |
| Class F | 0.0% | -0.3% | -0.3% |
| Overall | -0.02% | -0.84% | -1.01% |
|  | 0.0% | -0.9% | -1.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -1.3% | -1.5% |
| Class C | -0.1% | -1.3% | -1.4% |
| Class D | -0.1% | -1.7% | -2.6% |
| Class E | 0.0% | -0.9% | 0.6% |
| Class F | 0.0% | -0.3% | -0.2% |
| Overall | -0.04% | -1.13% | -1.12% |
|  | 0.0% | -1.1% | -1.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -1.3% | -1.1% |
| Class C | -0.1% | -1.1% | -0.9% |
| Class D | -0.1% | -1.3% | -3.1% |
| Class E | 0.2% | 0.4% | -0.1% |
| Class F | -0.1% | -0.3% | -0.3% |
| Overall | -0.03% | -0.79% | -1.13% |
|  | 0.0% | -0.8% | -1.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 4 Rate-distortion performance and speed comparison of variant B (LC settings with SAO)

|  | Random Access LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.2% | -0.5% |
| Class B | 0.0% | -0.3% | -0.3% |
| Class C | 0.0% | -0.3% | -0.3% |
| Class D | 0.0% | -0.4% | -0.6% |
| Class E |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% |
| Overall | 0.00% | -0.31% | -0.39% |
|  | 0.0% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.4% | -0.4% |
| Class C | -0.1% | -0.6% | -0.5% |
| Class D | -0.1% | -0.8% | -1.2% |
| Class E | 0.0% | 0.4% | 0.4% |
| Class F | 0.0% | -0.3% | 0.1% |
| Overall | -0.05% | -0.36% | -0.35% |
|  | 0.0% | -0.4% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.6% | -0.3% |
| Class C | 0.0% | -0.4% | -0.3% |
| Class D | -0.1% | -0.4% | -1.7% |
| Class E | 0.1% | 0.6% | -0.3% |
| Class F | -0.1% | -0.2% | 0.0% |
| Overall | -0.03% | -0.27% | -0.52% |
|  | 0.0% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Visual Quality

In CE3 description [7], it is required to check subjective visual qualities of sequence Vidyo4 (LBLC@QP32), BQTerrace (LBLC@QP32), Vidyo4 (LPLC@QP32), BQSquare (LPLC@QP32) and BQTerrace (LPLC@QP32). There is no noticeable difference.

# References

[1] J. Lou, K. Minoo, L. Wang, and A. Luthra, “On Chroma interpolation filter,” JCT-VC document, JCTVC-G0698, November 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0/

[3]

[4]

[5]

[6] F. Bossen, “Common HM test conditions and software reference configurations”, JCT-VC document, JCTVC-G1200, November 2011

[7] T. Chujoh and E. Alshina, “Description of Core Experiment 3 (CE3): Chroma Interpolation Filter,” JCT-VC document, JCTVC-G1203, November 2011

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).