| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0495 |
| --- | --- |

| Title: | Quantization matrices for 4x4 and 8x8 TUs matching HEVC integer transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Jae-Hoon Kim Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2369 jaekim@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document proposes new default quantization matrices for 4x4 and 8x8 TUs matching HEVC integer transforms.

# Introduction

Quantization matrices are designed to achieve higher compression ratio by utilizing the characteristics of human visual system. Many video coding standards offer the default quantization matrix, or matrices, designed based upon the specific transform(s) defined in those standards.

In H.264/AVC [1], there is one default quantization matrix for 4x4 Intra transform blocks (Table 1), one default quantization matrix for 4x4 Inter transform blocks (Table 2), one default quantization matrix for 8x8 Intra transform blocks (Table 3), and one default quantization matrix for 8x8 Inter transform blocks (Table 4).

Table 1 Default quantization matrix for 4x4 Intra transform blocks in H.264/AVC

| 6 | 13 | 20 | 28 |
| --- | --- | --- | --- |
| 13 | 20 | 28 | 32 |
| 20 | 28 | 32 | 37 |
| 28 | 32 | 37 | 42 |

Table 2 Default quantization matrix for 4x4 Inter transform blocks in H.264/AVC

| 10 | 14 | 20 | 24 |
| --- | --- | --- | --- |
| 14 | 20 | 24 | 27 |
| 20 | 24 | 27 | 30 |
| 24 | 27 | 30 | 34 |

Table 3 Default quantization matrix for 8x8 Intra transform blocks in H.264/AVC

| 6 | 10 | 13 | 16 | 18 | 23 | 25 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 16 | 18 | 23 | 25 | 27 | 29 |
| 13 | 16 | 18 | 23 | 25 | 27 | 29 | 31 |
| 16 | 18 | 23 | 25 | 27 | 29 | 31 | 33 |
| 18 | 23 | 25 | 27 | 29 | 31 | 33 | 36 |
| 23 | 25 | 27 | 29 | 31 | 33 | 36 | 38 |
| 25 | 27 | 29 | 31 | 33 | 36 | 38 | 40 |
| 27 | 29 | 31 | 33 | 36 | 38 | 40 | 42 |

Table 4 Default quantization matrix for 8x8 Inter transform blocks in H.264/AVC

| 9 | 13 | 15 | 17 | 19 | 21 | 22 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 13 | 17 | 19 | 21 | 22 | 24 | 25 |
| 15 | 17 | 19 | 21 | 22 | 24 | 25 | 27 |
| 17 | 19 | 21 | 22 | 24 | 25 | 27 | 28 |
| 19 | 21 | 22 | 24 | 25 | 27 | 28 | 30 |
| 21 | 22 | 24 | 25 | 27 | 28 | 30 | 32 |
| 22 | 24 | 25 | 27 | 28 | 30 | 32 | 33 |
| 24 | 25 | 27 | 28 | 30 | 32 | 33 | 35 |

In the current HEVC working draft, it is decided to keep the H.264/AVC default quantization matrices for 4x4 and 8x8 TUs. However, since the transform matrices in HEVC are different from those in H.264/AVC, the coefficients after H.264/AVC transforms might stand for different frequency components compared with the coefficients after HEVC transforms.

In H.264/AVC, the integer 4x4 transform matrix is

TAVC4×4=222221−1−22−2−221−22−12

In H.264/AVC, the integer 8x8 transform matrix is

TAVC8×8=88888888121063−3−6−10−1284−4−8−8−44810−3−12−66123−108−8−888−8−886−12310−10−312−64−88−4−48−843−610−1212−106−38

In HEVC, the integer 4x4 transform matrix is

THEVC4×4=646464648336−36−8364−64−646436−8383−36

In HEVC, the integer 8x8 transform matrix is

THEVC8×8=646464646464646489755018−18−50−75−898336−36−83−83−36368375−18−89−50508918−7564−64−646464−64−646450−891875−75−1889−5036−8383−36−3683−833618−5075−8989−7550−18

The normalized transform matrices are

TAVC4×4Norm=Normalize(TAVC4×4)

TAVC8×8Norm=Normalize(TAVC8×8)

THEVC4×4Norm=Normalize(THEVC4×4)

THEVC8×8Norm=Normalize(THEVC8×8)

The normalization process is done by dividing each coefficient in the transform matrices by the square root of the square sum of the coefficients in the same row.

The base vectors of normalized matrices are illustrated in Fig. 1 and Fig. 2.

Fig. 1 Basis vector comparison for 4x4 transforms

Fig. 2 Basis vector comparison for 8x8 transforms

The DCT distortion is calculated by the scheme mentioned in [2].

T4×4=TAVC4×4norm⋅THEVC4×4NormT

T8×8=TAVC8×8norm⋅THEVC8×8NormT

d14×4'=1444

d18×8'=1888

The first-order frequency distortion between AVC and HEVC 4x4 transforms is 0.0272 and the first-order frequenct distortion between AVC and HEVC 8x8 transforms is 0.0551.

Since the 4x4 and 8x8 transforms in AVC and HEVC are substantially different, the default quantization matrices defined in HEVC should be changed accordingly.

# Proposed quantization matrices for 4x4 and 8x8 TUs matching HEVC integer transforms

The new quantization matrices for HEVC are generated from AVC quantization matrices as follows

QMatrixHEVC4×4Intra=round(THEVC4×4Norm⋅TAVC4×4NormT⋅QMatrixAVC4×4Intra⋅TAVC4×4Norm⋅THEVC4×4NormT)

QMatrixHEVC8×8Intra=round(THEVC8×8Norm⋅TAVC8×8NormT⋅QMatrixAVC8×8Intra⋅TAVC8×8Norm⋅THEVC8×8NormT)

QMatrixHEVC4×4Inter=round(THEVC4×4Norm⋅TAVC4×4NormT⋅QMatrixAVC4×4Inter⋅TAVC4×4Norm⋅THEVC4×4NormT)

QMatrixHEVC8×8Inter=round(THEVC8×8Norm⋅TAVC8×8NormT⋅QMatrixAVC8×8Inter⋅TAVC8×8Norm⋅THEVC8×8NormT)

Tables 5-8 show the specific quantization matrices.

Table 5 Proposed quantization matrix for 4x4 Intra TUs in HEVC

| 6 | 15 | 20 | 27 |
| --- | --- | --- | --- |
| 15 | 24 | 30 | 33 |
| 20 | 30 | 32 | 35 |
| 27 | 33 | 35 | 38 |

Table 6 Proposed quantization matrix for 4x4 Inter TUs in HEVC

| 10 | 15 | 20 | 23 |
| --- | --- | --- | --- |
| 15 | 23 | 26 | 28 |
| 20 | 26 | 27 | 29 |
| 23 | 28 | 29 | 31 |

Table 7 Proposed quantization matrix for 8x8 Intra TUs in HEVC

| 6 | 11 | 14 | 16 | 18 | 23 | 24 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | 12 | 18 | 18 | 24 | 26 | 27 | 29 |
| 14 | 18 | 21 | 24 | 27 | 30 | 30 | 32 |
| 16 | 18 | 24 | 24 | 27 | 30 | 29 | 32 |
| 18 | 24 | 27 | 27 | 29 | 32 | 32 | 35 |
| 23 | 26 | 30 | 30 | 32 | 35 | 36 | 38 |
| 24 | 27 | 30 | 29 | 32 | 36 | 35 | 37 |
| 27 | 29 | 32 | 32 | 35 | 38 | 37 | 40 |

Table 8 Proposed quantization matrix for 8x8 Inter TUs in HEVC

| 9 | 13 | 16 | 17 | 19 | 22 | 21 | 23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 14 | 19 | 19 | 21 | 23 | 23 | 25 |
| 16 | 19 | 22 | 22 | 23 | 26 | 26 | 28 |
| 17 | 19 | 22 | 21 | 24 | 26 | 25 | 27 |
| 19 | 21 | 23 | 24 | 25 | 28 | 27 | 29 |
| 22 | 23 | 26 | 26 | 28 | 30 | 30 | 32 |
| 21 | 23 | 26 | 25 | 27 | 30 | 29 | 31 |
| 23 | 25 | 28 | 27 | 29 | 32 | 31 | 33 |

# References

[1] Advanced video coding for generic audiovisual services, International Telecommunication Union-Telecommunication (ITU-T) and International Standards Organization/International Electrotechnical Communication (ISO/IEC) JTC 1, Recommendation H.264 and ISO/IEC 14 496-10 (MPEG-4) AVC, 2003.

[2] M. Wien and S. Sun, “ICT Comparison for Adaptive Block Transforms”, VCEG document, VCEG-L12, January 2001.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).