| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0498 |
| --- | --- |

| Title: | Simplified coeff_abs_level_minus3 coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current CABAC of HEVC, syntax coeff_ab_level_minu3 is used to represent the absolute value of a quantized transform coefficient level minus 3. Adaptive truncated Rice code with parameter 0, 1, 2, and 3 are used. This document proposes to simplify the truncated Rice code with 8 bit codeword, so that memory could be saved if lookup table is used for implementation. Cross-check will be provided by Qualcomm. The attached spreadsheet contains detailed data of the results.

# Introduction

In the current CABAC of HEVC, syntax coeff_ab_level_minu3 is used to represent the absolute value of a quantized transform coefficient level minus 3. Adaptive truncated Golomb-Rice code with parameter 0, 1, 2, and 3 are used. For some large values, truncated Rice code and 0-th order Exp-Golomb code are used together.

The Truncated Rice code is generated by two parameters, cRiceParam and cTRMax. In the current HEVC, cRiceParam could have a value of 0, 1, 2, or 3 and it controls the flatness of the codeword distribution. For each cRiceParam value, there is a related cTRMax value which controls the truncation point for the rice code. Table 1 gives the value of cTRMax for each cRiceParam value and Table 2 gives the codewords of the Truncated Rice code with different cRiceParam and cTRMax from HM 5.0.

Table 1 Specification of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 20 |
| 2 | 42 |
| 3 | 70 |

Table 2 Codewords of the Truncated Rice code with different cRiceParam and cTRMax

| cRiceParam | 0 |  | 1 |  | 2 |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | Codeword | Bits | Codeword | Bits | Codeword | Bits | Codeword | Bits |
| 0 | 0 | 1 | 00 | 2 | 000 | 3 | 0000 | 4 |
| 1 | 10 | 2 | 01 | 2 | 001 | 3 | 0001 | 4 |
| 2 | 110 | 3 | 100 | 3 | 010 | 3 | 0010 | 4 |
| 3 | 1110 | 4 | 101 | 3 | 011 | 3 | 0011 | 4 |
| 4 | 11110 | 5 | 1100 | 4 | 1000 | 4 | 0100 | 4 |
| 5 | 111110 | 6 | 1101 | 4 | 1001 | 4 | 0101 | 4 |
| 6 | 1111110 | 7 | 11100 | 5 | 1010 | 4 | 0110 | 4 |
| 7 | 11111110 | 8 | 11101 | 5 | 1011 | 4 | 0111 | 4 |
| 8 | 11111111,EG0 | 9 | 111100 | 6 | 11000 | 5 | 10000 | 5 |
| 9 |  |  | 111101 | 6 | 11001 | 5 | 10001 | 5 |
| 10 |  |  | 1111100 | 7 | 11010 | 5 | 10010 | 5 |
| 11 |  |  | 1111101 | 7 | 11011 | 5 | 10011 | 5 |
| 12 |  |  | 11111100 | 8 | 111000 | 6 | 10100 | 5 |
| 13 |  |  | 11111101 | 8 | 111001 | 6 | 10101 | 5 |
| 14 |  |  | 111111100 | 9 | 111010 | 6 | 10110 | 5 |
| 15 |  |  | 111111101 | 9 | 111011 | 6 | 10111 | 5 |
| 16 |  |  | 1111111100 | 10 | 1111000 | 7 | 110000 | 6 |
| 17 |  |  | 1111111101 | 10 | 1111001 | 7 | 110001 | 6 |
| 18 |  |  | 11111111100 | 11 | 1111010 | 7 | 110010 | 6 |
| 19 |  |  | 11111111101 | 11 | 1111011 | 7 | 110011 | 6 |
| 20 |  |  | 11111111110 | 11 | 11111000 | 8 | 110100 | 6 |
| 21 |  |  | 11111111111,EG0 | 12 | 11111001 | 8 | 110101 | 6 |
| 22 |  |  |  |  | 11111010 | 8 | 110110 | 6 |
| 23 |  |  |  |  | 11111011 | 8 | 110111 | 6 |
| 24 |  |  |  |  | 111111000 | 9 | 1110000 | 7 |
| 25 |  |  |  |  | 111111001 | 9 | 1110001 | 7 |
| 26 |  |  |  |  | 111111010 | 9 | 1110010 | 7 |
| 27 |  |  |  |  | 111111011 | 9 | 1110011 | 7 |
| 28 |  |  |  |  | 1111111000 | 10 | 1110100 | 7 |
| 29 |  |  |  |  | 1111111001 | 10 | 1110101 | 7 |
| 30 |  |  |  |  | 1111111010 | 10 | 1110110 | 7 |
| 31 |  |  |  |  | 1111111011 | 10 | 1110111 | 7 |
| 32 |  |  |  |  | 11111111000 | 11 | 11110000 | 8 |
| 33 |  |  |  |  | 11111111001 | 11 | 11110001 | 8 |
| 34 |  |  |  |  | 11111111010 | 11 | 11110010 | 8 |
| 35 |  |  |  |  | 11111111011 | 11 | 11110011 | 8 |
| 36 |  |  |  |  | 111111111000 | 12 | 11110100 | 8 |
| 37 |  |  |  |  | 111111111001 | 12 | 11110101 | 8 |
| 38 |  |  |  |  | 111111111010 | 12 | 11110110 | 8 |
| 39 |  |  |  |  | 111111111011 | 12 | 11110111 | 8 |
| 40 |  |  |  |  | 111111111100 | 12 | 111110000 | 9 |
| 41 |  |  |  |  | 111111111101 | 12 | 111110001 | 9 |
| 42 |  |  |  |  | 111111111110 | 12 | 111110010 | 9 |
| 43 |  |  |  |  | 111111111111,EG0 | 13 | 111110011 | 9 |
| 44 |  |  |  |  |  |  | 111110100 | 9 |
| 45 |  |  |  |  |  |  | 111110101 | 9 |
| 46 |  |  |  |  |  |  | 111110110 | 9 |
| 47 |  |  |  |  |  |  | 111110111 | 9 |
| 48 |  |  |  |  |  |  | 1111110000 | 10 |
| 49 |  |  |  |  |  |  | 1111110001 | 10 |
| 50 |  |  |  |  |  |  | 1111110010 | 10 |
| 51 |  |  |  |  |  |  | 1111110011 | 10 |
| 52 |  |  |  |  |  |  | 1111110100 | 10 |
| 53 |  |  |  |  |  |  | 1111110101 | 10 |
| 54 |  |  |  |  |  |  | 1111110110 | 10 |
| 55 |  |  |  |  |  |  | 1111110111 | 10 |
| 56 |  |  |  |  |  |  | 11111110000 | 11 |
| 57 |  |  |  |  |  |  | 11111110001 | 11 |
| 58 |  |  |  |  |  |  | 11111110010 | 11 |
| 59 |  |  |  |  |  |  | 11111110011 | 11 |
| 60 |  |  |  |  |  |  | 11111110100 | 11 |
| 61 |  |  |  |  |  |  | 11111110101 | 11 |
| 62 |  |  |  |  |  |  | 11111110110 | 11 |
| 63 |  |  |  |  |  |  | 11111110111 | 11 |
| 64 |  |  |  |  |  |  | 11111111000 | 11 |
| 65 |  |  |  |  |  |  | 11111111001 | 11 |
| 66 |  |  |  |  |  |  | 11111111010 | 11 |
| 67 |  |  |  |  |  |  | 11111111011 | 11 |
| 68 |  |  |  |  |  |  | 11111111100 | 11 |
| 69 |  |  |  |  |  |  | 11111111101 | 11 |
| 70 |  |  |  |  |  |  | 11111111110 | 11 |
| 71 |  |  |  |  |  |  | 11111111111,EG0 | 12 |

In Table 2, if a symbol value is larger than the cTRMax value for one specific cRiceParam, 0-th order Exp-Golomb code is used to code the value of (symbol-cTRMax-1), which is appended to the last Truncated Rice codeword.

Generally, Truncated Rice code with smaller cRiceParam value is preferred to code the symbols with smaller values, as it needs few bits to represent.

cRiceParam for coeff_abs_level_minus3[n] is derived from Table 3.

Table 3 cRiceParam update table

|  | coeff_abs_level_minus3[n-1] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | ≥15 |
| Previous cRiceParam | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

Notice that in Table 2, the maximum number of bits for the Truncated Rice code is 12, so if lookup table is used for the implementation, each codeword needs to be stored as 2 bytes and the total memory needed for Table 2 is no less than 2*(9+22+44+72) = 294 bytes.

# Simplified coeff_abs_level_minus3 coding with 8bit lookup table

Table 4 gives for the value of cTRMax for each cRiceParam value and Table 5 gives the codewords of the Truncated Rice code.

Table 4 Specification of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 14 |
| 2 | 26 |
| 3 | 46 |

Table 5 Codewords of the Truncated Rice code with different cRiceParam and cTRMax

| cRiceParam | 0 |  | 1 |  | 2 |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | Codeword | Bits | Codeword | Bits | Codeword | Bits | Codeword | Bits |
| 0 | 0 | 1 | 00 | 2 | 000 | 3 | 0000 | 4 |
| 1 | 10 | 2 | 01 | 2 | 001 | 3 | 0001 | 4 |
| 2 | 110 | 3 | 100 | 3 | 010 | 3 | 0010 | 4 |
| 3 | 1110 | 4 | 101 | 3 | 011 | 3 | 0011 | 4 |
| 4 | 11110 | 5 | 1100 | 4 | 1000 | 4 | 0100 | 4 |
| 5 | 111110 | 6 | 1101 | 4 | 1001 | 4 | 0101 | 4 |
| 6 | 1111110 | 7 | 11100 | 5 | 1010 | 4 | 0110 | 4 |
| 7 | 11111110 | 8 | 11101 | 5 | 1011 | 4 | 0111 | 4 |
| 8 | 11111111,EG0 | 9 | 111100 | 6 | 11000 | 5 | 10000 | 5 |
| 9 |  |  | 111101 | 6 | 11001 | 5 | 10001 | 5 |
| 10 |  |  | 1111100 | 7 | 11010 | 5 | 10010 | 5 |
| 11 |  |  | 1111101 | 7 | 11011 | 5 | 10011 | 5 |
| 12 |  |  | 11111100 | 8 | 111000 | 6 | 10100 | 5 |
| 13 |  |  | 11111101 | 8 | 111001 | 6 | 10101 | 5 |
| 14 |  |  | 11111110 | 8 | 111010 | 6 | 10110 | 5 |
| 15 |  |  | 11111111,EG0 | 9 | 111011 | 6 | 10111 | 5 |
| 16 |  |  |  |  | 1111000 | 7 | 110000 | 6 |
| 17 |  |  |  |  | 1111001 | 7 | 110001 | 6 |
| 18 |  |  |  |  | 1111010 | 7 | 110010 | 6 |
| 19 |  |  |  |  | 1111011 | 7 | 110011 | 6 |
| 20 |  |  |  |  | 11111000 | 8 | 110100 | 6 |
| 21 |  |  |  |  | 11111001 | 8 | 110101 | 6 |
| 22 |  |  |  |  | 11111010 | 8 | 110110 | 6 |
| 23 |  |  |  |  | 11111011 | 8 | 110111 | 6 |
| 24 |  |  |  |  | 11111100 | 8 | 1110000 | 7 |
| 25 |  |  |  |  | 11111101 | 8 | 1110001 | 7 |
| 26 |  |  |  |  | 11111110 | 8 | 1110010 | 7 |
| 27 |  |  |  |  | 11111111,EG0 | 9 | 1110011 | 7 |
| 28 |  |  |  |  |  |  | 1110100 | 7 |
| 29 |  |  |  |  |  |  | 1110101 | 7 |
| 30 |  |  |  |  |  |  | 1110110 | 7 |
| 31 |  |  |  |  |  |  | 1110111 | 7 |
| 32 |  |  |  |  |  |  | 11110000 | 8 |
| 33 |  |  |  |  |  |  | 11110001 | 8 |
| 34 |  |  |  |  |  |  | 11110010 | 8 |
| 35 |  |  |  |  |  |  | 11110011 | 8 |
| 36 |  |  |  |  |  |  | 11110100 | 8 |
| 37 |  |  |  |  |  |  | 11110101 | 8 |
| 38 |  |  |  |  |  |  | 11110110 | 8 |
| 39 |  |  |  |  |  |  | 11110111 | 8 |
| 40 |  |  |  |  |  |  | 11111000 | 8 |
| 41 |  |  |  |  |  |  | 11111001 | 8 |
| 42 |  |  |  |  |  |  | 11111010 | 8 |
| 43 |  |  |  |  |  |  | 11111011 | 8 |
| 44 |  |  |  |  |  |  | 11111100 | 8 |
| 45 |  |  |  |  |  |  | 11111101 | 8 |
| 46 |  |  |  |  |  |  | 11111110 | 8 |
| 47 |  |  |  |  |  |  | 11111111,EG0 | 9 |

In Table 5, if a symbol value is larger than the cTRMax value for one specific cRiceParam, 0-th order Exp-Golomb code is used to code the value of (symbol-cTRMax-1), which is appended to the last Truncated Rice codeword. With Table 5, the total memory needed is 9+16+28+48 = 101 bytes.

cRiceParam for coeff_abs_level_minus3[n] is also derived from Table 3.

In [1], it is proposed to add value 4 for the truncated Rice code to improve the compression performance under high bitrate. These two scheme could be combined to reduce the complexity while achieving higher compression efficiency under high bitrate. Table 6 gives the value of cTRMax for each cRiceParam value and Table 7 gives the codewords of the Truncated Rice code with different cRiceParam and cTRMax.

Table 6 Specification of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 14 |
| 2 | 26 |
| 3 | 46 |
| 4 | 78 |

Table 7 Codewords of the Truncated Rice code with different cRiceParam and cTRMax

| cRiceParam | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | Codeword | Bits | Codeword | Bits | Codeword | Bits | Codeword | Bits | Codeword | Bits |
| 0 | 0 | 1 | 00 | 2 | 000 | 3 | 0000 | 4 | 00000 | 5 |
| 1 | 10 | 2 | 01 | 2 | 001 | 3 | 0001 | 4 | 00001 | 5 |
| 2 | 110 | 3 | 100 | 3 | 010 | 3 | 0010 | 4 | 00010 | 5 |
| 3 | 1110 | 4 | 101 | 3 | 011 | 3 | 0011 | 4 | 00011 | 5 |
| 4 | 11110 | 5 | 1100 | 4 | 1000 | 4 | 0100 | 4 | 00100 | 5 |
| 5 | 111110 | 6 | 1101 | 4 | 1001 | 4 | 0101 | 4 | 00101 | 5 |
| 6 | 1111110 | 7 | 11100 | 5 | 1010 | 4 | 0110 | 4 | 00110 | 5 |
| 7 | 11111110 | 8 | 11101 | 5 | 1011 | 4 | 0111 | 4 | 00111 | 5 |
| 8 | 11111111,EG0 | 9 | 111100 | 6 | 11000 | 5 | 10000 | 5 | 01000 | 5 |
| 9 |  |  | 111101 | 6 | 11001 | 5 | 10001 | 5 | 01001 | 5 |
| 10 |  |  | 1111100 | 7 | 11010 | 5 | 10010 | 5 | 01010 | 5 |
| 11 |  |  | 1111101 | 7 | 11011 | 5 | 10011 | 5 | 01011 | 5 |
| 12 |  |  | 11111100 | 8 | 111000 | 6 | 10100 | 5 | 01100 | 5 |
| 13 |  |  | 11111101 | 8 | 111001 | 6 | 10101 | 5 | 01101 | 5 |
| 14 |  |  | 11111110 | 8 | 111010 | 6 | 10110 | 5 | 01110 | 5 |
| 15 |  |  | 11111111,EG0 | 9 | 111011 | 6 | 10111 | 5 | 01111 | 5 |
| 16 |  |  |  |  | 1111000 | 7 | 110000 | 6 | 100000 | 6 |
| 17 |  |  |  |  | 1111001 | 7 | 110001 | 6 | 100001 | 6 |
| 18 |  |  |  |  | 1111010 | 7 | 110010 | 6 | 100010 | 6 |
| 19 |  |  |  |  | 1111011 | 7 | 110011 | 6 | 100011 | 6 |
| 20 |  |  |  |  | 11111000 | 8 | 110100 | 6 | 100100 | 6 |
| 21 |  |  |  |  | 11111001 | 8 | 110101 | 6 | 100101 | 6 |
| 22 |  |  |  |  | 11111010 | 8 | 110110 | 6 | 100110 | 6 |
| 23 |  |  |  |  | 11111011 | 8 | 110111 | 6 | 100111 | 6 |
| 24 |  |  |  |  | 11111100 | 8 | 1110000 | 7 | 101000 | 6 |
| 25 |  |  |  |  | 11111101 | 8 | 1110001 | 7 | 101001 | 6 |
| 26 |  |  |  |  | 11111110 | 8 | 1110010 | 7 | 101010 | 6 |
| 27 |  |  |  |  | 11111111,EG0 | 9 | 1110011 | 7 | 101011 | 6 |
| 28 |  |  |  |  |  |  | 1110100 | 7 | 101100 | 6 |
| 29 |  |  |  |  |  |  | 1110101 | 7 | 101101 | 6 |
| 30 |  |  |  |  |  |  | 1110110 | 7 | 101110 | 6 |
| 31 |  |  |  |  |  |  | 1110111 | 7 | 101111 | 6 |
| 32 |  |  |  |  |  |  | 11110000 | 8 | 1100000 | 7 |
| 33 |  |  |  |  |  |  | 11110001 | 8 | 1100001 | 7 |
| 34 |  |  |  |  |  |  | 11110010 | 8 | 1100010 | 7 |
| 35 |  |  |  |  |  |  | 11110011 | 8 | 1100011 | 7 |
| 36 |  |  |  |  |  |  | 11110100 | 8 | 1100100 | 7 |
| 37 |  |  |  |  |  |  | 11110101 | 8 | 1100101 | 7 |
| 38 |  |  |  |  |  |  | 11110110 | 8 | 1100110 | 7 |
| 39 |  |  |  |  |  |  | 11110111 | 8 | 1100111 | 7 |
| 40 |  |  |  |  |  |  | 11111000 | 8 | 1101000 | 7 |
| 41 |  |  |  |  |  |  | 11111001 | 8 | 1101001 | 7 |
| 42 |  |  |  |  |  |  | 11111010 | 8 | 1101010 | 7 |
| 43 |  |  |  |  |  |  | 11111011 | 8 | 1101011 | 7 |
| 44 |  |  |  |  |  |  | 11111100 | 8 | 1101100 | 7 |
| 45 |  |  |  |  |  |  | 11111101 | 8 | 1101101 | 7 |
| 46 |  |  |  |  |  |  | 11111110 | 8 | 1101110 | 7 |
| 47 |  |  |  |  |  |  | 11111111,EG0 | 9 | 1101111 | 7 |
| 48 |  |  |  |  |  |  |  |  | 11100000 | 8 |
| 49 |  |  |  |  |  |  |  |  | 11100001 | 8 |
| 50 |  |  |  |  |  |  |  |  | 11100010 | 8 |
| 51 |  |  |  |  |  |  |  |  | 11100011 | 8 |
| 52 |  |  |  |  |  |  |  |  | 11100100 | 8 |
| 53 |  |  |  |  |  |  |  |  | 11100101 | 8 |
| 54 |  |  |  |  |  |  |  |  | 11100110 | 8 |
| 55 |  |  |  |  |  |  |  |  | 11100111 | 8 |
| 56 |  |  |  |  |  |  |  |  | 11101000 | 8 |
| 57 |  |  |  |  |  |  |  |  | 11101001 | 8 |
| 58 |  |  |  |  |  |  |  |  | 11101010 | 8 |
| 59 |  |  |  |  |  |  |  |  | 11101011 | 8 |
| 60 |  |  |  |  |  |  |  |  | 11101100 | 8 |
| 61 |  |  |  |  |  |  |  |  | 11101101 | 8 |
| 62 |  |  |  |  |  |  |  |  | 11101110 | 8 |
| 63 |  |  |  |  |  |  |  |  | 11101111 | 8 |
| 64 |  |  |  |  |  |  |  |  | 11110000 | 8 |
| 65 |  |  |  |  |  |  |  |  | 11110001 | 8 |
| 66 |  |  |  |  |  |  |  |  | 11110010 | 8 |
| 67 |  |  |  |  |  |  |  |  | 11110011 | 8 |
| 68 |  |  |  |  |  |  |  |  | 11110100 | 8 |
| 69 |  |  |  |  |  |  |  |  | 11110101 | 8 |
| 70 |  |  |  |  |  |  |  |  | 11110110 | 8 |
| 71 |  |  |  |  |  |  |  |  | 11110111 | 8 |
| 72 |  |  |  |  |  |  |  |  | 11111000 | 8 |
| 73 |  |  |  |  |  |  |  |  | 11111001 | 8 |
| 74 |  |  |  |  |  |  |  |  | 11111010 | 8 |
| 75 |  |  |  |  |  |  |  |  | 11111011 | 8 |
| 76 |  |  |  |  |  |  |  |  | 11111100 | 8 |
| 77 |  |  |  |  |  |  |  |  | 11111101 | 8 |
| 78 |  |  |  |  |  |  |  |  | 11111110 | 8 |
| 79 |  |  |  |  |  |  |  |  | 11111111,EG0 | 9 |

In Table 7, if a symbol value is larger than the cTRMax value for one specific cRiceParam, 0-th order Exp-Golomb code is used to code the value of (symbol-cTRMax-1), which is appended to the last Truncated Rice codeword. With Table 7, the total memory needed is 9+16+28+48+80 = 181 bytes.

cRiceParam for coeff_abs_level_minus3[n] is derived from Table 8.

Table 8 cRiceParam update table

|  | coeff_abs_level_minus3[n-1] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Previous cRiceParam | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
|  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | coeff_abs_level_minus3[n-1] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | ≥23 |  |  |  |  |  |  |  |  |
| Previous cRiceParam | 0 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 |  |  |  |  |  |  |  |  |
|  | 1 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 |  |  |  |  |  |  |  |  |
|  | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 |  |  |  |  |  |  |  |  |
|  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 |  |  |  |  |  |  |  |  |
|  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |  |  |  |  |  |  |  |  |

In [2], it is mentioned that combination logics could be used in stead of Table 3. Following the same method, an example of combination logic representation for Table 8 is

cRicePram = cRicePram

+ (coeff_abs_level_minus3[n-1] >= 2   &&  cRicePram == 0)

+ (coeff_abs_level_minus3[n-1] >= 5   &&  cRicePram <= 1)

+ (coeff_abs_level_minus3[n-1] >= 11  &&  cRicePram <= 2);

+ (coeff_abs_level_minus3[n-1] >= 23  &&  cRicePram <= 3);

# Simulation environment

## Software

The software used to run the simulations is HM5.0 [3] for common test condition, and HM5.1rc2 for low QPs (2 7 12 17) [4]. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [5]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 14G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation results

The experimental results of the rate-distortion performance and speed comparison are summarized in Table 9-12. Detailed results are included in the attachments (JCTVC-H0498_4Param.xls, JCTVC-H0498_4ParamLowQP.xls, JCTVC-H0498_5Param.xls, and JCTVC-H0498_5ParamLowQP.xls). Cross-check will be provided by Qualcomm [6].

Table 9 Rate-distortion performance and speed comparison for QP 22 27 32 37 (4 parameters for cRicePram)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 0.00% | 0.01% | 0.00% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.00% | -0.04% | 0.14% | 0.02% | 0.02% | 0.02% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | 0.6% |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.00% | 0.00% | 0.04% | -0.02% | 0.04% | 0.00% | 0.24% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class F | -0.11% | -0.03% | -0.12% | 0.01% | -0.04% | -0.02% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.2% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | -0.2% | -0.1% | 0.6% | 0.4% |  |  |  |
| Class E | 0.1% | -0.9% | -0.2% | -0.1% | 0.2% | 0.2% |  |  |  |
| Overall | 0.02% | -0.21% | -0.03% | -0.01% | 0.19% | 0.19% |  |  |  |
|  | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | 0.2% |  |  |  |
| Class F | -0.20% | -0.09% | -0.46% | 0.15% | -0.02% | -0.19% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 10 Rate-distortion performance and speed comparison for QP 2 7 12 17(4 parameters for cRicePram)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class C | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% |  |  |  |
| Class D | 0.3% | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% |  |  |  |
| Class E | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Overall | 0.17% | 0.16% | 0.16% | 0.18% | 0.13% | 0.13% |  |  |  |
|  | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% |  |  |  |
| Class F | 0.43% | 0.41% | 0.41% | 0.45% | 0.40% | 0.41% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.03% | 0.03% | 0.06% | 0.03% | 0.02% | 0.05% | 0.02% | 0.03% | 0.03% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.02% | 0.01% | -0.67% | 0.48% | 0.53% | 0.43% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.02% | -0.01% | 0.04% | 0.02% | 0.00% | 0.03% |  |  |  |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | 0.23% | 0.36% | 0.20% | 0.41% | 0.38% | 0.40% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |

Table 11 Rate-distortion performance and speed comparison for QP 22 27 32 37 (4 parameters for cRicePram)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.00% | -0.03% | 0.05% | 0.03% | 0.04% | 0.02% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.1% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 0.00% | 0.04% | -0.02% | 0.03% | -0.11% | -0.01% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | -0.10% | -0.01% | -0.08% | 0.06% | 0.01% | 0.02% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.4% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | -0.1% | -0.2% | -0.3% | 0.0% | 0.4% | 0.3% |  |  |  |
| Class E | 0.1% | -0.5% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Overall | 0.02% | -0.18% | 0.11% | 0.00% | 0.10% | 0.08% |  |  |  |
|  | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | -0.17% | 0.06% | -0.25% | 0.16% | 0.19% | -0.27% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 12 Rate-distortion performance and speed comparison for QP 2 7 12 17(4 parameters for cRicePram)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class C | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |  |  |  |
| Class D | -0.5% | -0.4% | -0.5% | -0.5% | -0.4% | -0.4% |  |  |  |
| Class E | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Overall | -0.25% | -0.22% | -0.23% | -0.24% | -0.18% | -0.20% |  |  |  |
|  | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| Class F | -1.67% | -1.46% | -1.45% | -1.58% | -1.33% | -1.35% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class D | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.05% | -0.02% | -0.01% | -0.02% | -0.01% | -0.03% | -0.03% | -0.06% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | -0.67% | -0.62% | -0.66% | -1.05% | -0.68% | -0.76% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class E | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | -0.01% | -0.06% | 0.01% | -0.02% | -0.04% | -0.01% |  |  |  |
|  | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | -1.11% | -0.88% | -0.99% | -0.81% | -0.57% | -0.68% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |

The best gain is found on SlideEditing with AIHE settings. The bitrate saving for Y/U/V is -4.2%/-3.7%/-3.7%.

# References

[1] J. Lou, L. Wang, and X. Fang, “Improving coeff_abs_level_minus3 coding performance under high bitrate,” JCT-VC document, JCTVC-H0497, February 2012

[2] J. Lou, L. Wang, and X. Fang, “On coeff_abs_level_minus3 coding,” JCT-VC document, JCTVC-G0700, November 2011

[3] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0/

[4] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.1rc2/

[5] F. Bossen, “Common HM test conditions and software reference configurations”, JCT-VC document, JCTVC-G1200, November 2011

[6]

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).