| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 1-10 February, 2012 | Document: JCTVC-H0506 |
| --- | --- |

| Title: | CE4 Subtest 3.1.a: QP adaptation at sub-CU level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jae Hoon Kim Xue Fang Krit Panusopone Limin Wang Ajay Luthra | Tel: Email: | +1-858-404-2369 jaekim@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document describes implementation details and reports the results of Motorola Mobility’s QP adaptation at sub-CU level. Simulation results show that there is a loss of 0.7% and 1.3% for AIHE and AILC, a loss of 1.5% and 1.0% for LBHE and LBLC and a loss of 0.9%, 0.7%, and 1.5% for RAHE, RALC, and RAHE 10bit conditions respectively. The QP modulation of sub-CU is based on existing activity calculation in HM5.0.  Cross-Check will be provided by NEC.

# Introduction

QP adaptation at sub-CU level was proposed [1] to allow finer QP granularity in HEVC and is included in subtest 3.1.a of CE4 [2]. In this experiment, if depth of a leaf CU is smaller than the max_cu_qp_delta_depth, a TU can signal its own dQP if the full depth (CU depth + transform depth) of that TU is less than the given threshold. Otherwise, leaf TUs will share a single TU dQP when their full depths are over the max_cu_qp_delta_depth threshold.

# Algorithm

The delta QP for each TU (dQPTU) in a CU is signaled, if all of the following conditions are met:

- CU depth (CUdepth) is smaller than max_cu_tu_qp_delta_depth;

- TU has at least one coefficient in either Luma,Y or Chroma U or V;

- The full depth, which is CU depth plus transform depth (Trdepth) of a TU, is smaller than or equal to max_cu_tu_qp_delta_depth. Or the full depth of a TU is larger than max_cu_tu_qp_delta_depth, but it’s the first TU which has at least one nonzero coefficient in either Luma or Chroma.

The final QP for the TU, QPTU, is derived from dQPTU and predicted QPp-TU as follows;

QPTU=QPP−TU+dQPTU                                                                        (1)

The method for QPp-TU prediction is the same used in current HM5.0.

# Simulation Results

The proposed method was implemented in the HM-5.0 and tested to evaluate its coding performance and complexity. The binaries were compiled using Microsoft Visual Studio 2010 64 bits. The coding conditions used for the simulations are specified in [2]. Detailed results are included in the attachment (JCTVC-H506.xls).

Table 1: Coding performance and complexity of the proposed method with MaxCUdQPdepth set to 3

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 0.8% | 2.9% | 3.0% | 10.5% | 1.9% | 0.8% | 0.4% | 24.9% |
| Class B | 1.0% | 2.4% | 2.7% | 14.6% | 1.8% | 0.9% | 0.8% | 30.7% |
| Class C | 0.5% | 1.6% | 1.7% | 7.1% | 0.9% | 0.7% | 0.6% | 12.5% |
| Class D | 0.4% | 1.1% | 1.2% | 6.1% | 0.6% | 0.4% | 0.3% | 9.5% |
| Class E | 1.0% | 3.5% | 3.1% | 9.3% | 1.7% | -0.1% | 0.4% | 17.1% |
| Overall | 0.7% | 2.2% | 2.2% | 9.7% | 1.3% | 0.5% | 0.5% | 19.0% |
|  | 0.7% | 2.2% | 2.2% |  | 1.3% | 0.6% | 0.6% |  |
| Class F | 0.5% | 1.6% | 1.7% | 5.5% | 1.4% | 0.2% | 0.4% | 10.7% |
| Enc Time[%] | 107% |  |  |  | 109% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 96% |  |  |  |

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 1.0% | 2.5% | 2.5% | 10.5% | 1.0% | 0.3% | 0.0% | 13.4% |
| Class B | 1.2% | 2.4% | 2.9% | 14.4% | 1.5% | 0.9% | 0.9% | 18.0% |
| Class C | 0.7% | 1.4% | 1.3% | 9.0% | 0.4% | 0.1% | 0.3% | 8.6% |
| Class D | 0.5% | 1.0% | 1.0% | 8.5% | 0.1% | -0.3% | 0.0% | 7.3% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.9% | 1.8% | 1.9% | 10.8% | 0.7% | 0.3% | 0.4% | 12.0% |
|  | 0.9% | 1.8% | 1.9% |  | 0.7% | 0.3% | 0.4% |  |
| Class F | 0.6% | 1.7% | 1.8% | 7.8% | 0.8% | 0.6% | 0.9% | 9.0% |
| Enc Time[%] | 158% |  |  |  | 116% |  |  |  |
| Dec Time[%] | 77% |  |  |  | 399% |  |  |  |

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | 1.7% | 0.7% | 0.9% | 22.2% |
| Class B | 1.3% | 2.4% | 3.3% | 14.9% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | 1.5% | 1.7% | 2.2% | 18.2% |
|  | 1.4% | 1.7% | 2.2% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 153% |  |  |  |
| Dec Time[%] | 77% |  |  |  |

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.8% | 4.1% | 5.0% | 13.7% | 1.6% | 2.4% | 2.9% | 16.0% |
| Class C | 1.2% | 2.0% | 2.1% | 9.6% | 0.4% | 1.1% | 1.4% | 8.4% |
| Class D | 0.9% | 2.1% | 2.6% | 10.8% | 0.1% | 0.6% | 0.3% | 7.9% |
| Class E | 2.2% | 5.4% | 4.9% | 9.0% | 1.8% | 0.9% | 0.7% | 7.6% |
| Overall | 1.5% | 3.3% | 3.6% | 11.1% | 1.0% | 1.4% | 1.5% | 10.5% |
|  | 1.5% | 3.4% | 3.5% |  | 0.9% | 1.4% | 1.4% |  |
| Class F | 1.0% | 1.8% | 1.8% | 12.9% | 1.0% | 1.3% | 1.8% | 12.0% |
| Enc Time[%] | 90% |  |  |  | 88% |  |  |  |
| Dec Time[%] | 77% |  |  |  | 72% |  |  |  |

# References

- K. Panusopone, A. Luthra, X. Fang, L. Wang, “QP adaptation at sub_CU level,” JCTVC-F577

- K. Sato, M. Budagavi, M. Coban, H. Aoki, X. Li, “Description of Core Experiment 4: Quantization,” JCTVC-G1204

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).