| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0507 |
| --- | --- |

| Title: | Referencing different APS’s in the same picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Vidyo, Inc. 433 Hackensack Ave., 7th floor Hackensack, NJ 07601 USA Ye-Kui Wang In Suk Chong Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | Tel: +1-415-713-5473 Email: stewe@stewe.org 1-858-651-8345 yekuiw@qualcomm.com 1-858-845-5477 ichong@qualcomm.com |
| Source: | Vidyo Inc. and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Proposed is the option to activate more than one Adaptation Parameter Set (APS) per coded picture.  As a result, at least some of the coded tools whose parameters are carried by the APS (such as: loop and deblocking filters, SAO, quantization matrixes) can be adapted to different content in a picture, by appropriately placing slice headers into the picture referencing different APSs.

# Proposal

The content of the current APS is understood by many as a per-picture property that must remain unchanged per picture.  However, in Torino, JCTVC specifically declined to put into the working draft (i.e. section 7.4.1.2.1) language that restricts the use of more than one APS per picture.

To refresh everyone’s memory, the key argument in Torino against per slice activation has been that this would open areas of technology not fully studied and understood, as most (if not all) work towards adaptive loop filters and sample adaptive offset—the two technologies for which the APS was introduced—was at the time picture-based.  The key arguments in favour of slice based activation had been enabling lower latency encoding, and more flexibility when coding large (multi-slice) pictures.

The key reason against slice-based activation of different ALF and SAO parameters seem to have gone away (mostly), as of this meeting.  A number of proposals appear to address CU-based changes of SAO and ALF parameters.  If one can change ALF and SAO parameters per CU, one sure can also do so per slice.

The key reasons in favour of slice based APS activation appear to us to remain the same.  We see no grounds that the reason will go away, unless the CU based activation of all tools whose parameters are carried in the APS have (in addition, or as a replacement) a CU-based activation mechanism.

There is no need to change the working draft.  As the activation of different APSs per picture (through the slice header) is not explicitly forbidden, we conclude that it is allowed as of today.  The decoder software may need updating, though.

# Patent rights declaration(s)

Vidyo, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.