| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 1-10 February, 2012 | Document: JCTVC-H0508-r4 |
| --- | --- |

| Title: | CE4 Subtest 3.1.b: Additional constraint for QP adaptation at sub-CU level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jae Hoon Kim Xue Fang Krit Panusopone Peter Borgwardt Limin Wang Ajay Luthra | Tel: Email: | +1-858-404-2369 jaekim@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document describes implementation details and reports the results of Motorola Mobility’s QP adaptation at sub-CU level with additional PU level constraint to limit amount of dQP overhead. Simulation results show that there is a loss of 0.3% and 0.5% for RA-HE and LB-HE and a gain of 0.6% for LB-LC. The QP modulation of sub-CU is based on existing activity calculation in HM5.0. Cross-Check will be provided by NEC.

# Introduction

QP adaptation at sub-CU level was proposed [1] to allow finer QP granularity in HEVC and is included in subtest 3.1.b of CE4 [2]. In this proposal, noting that each PU in a CU may point to different objects, PU based QP adaptation scheme is investigated.

# Implemented dQP Signaling Method

## Algorithm

Because QP is used when reconstructing non-zero coefficient, QP does not need to be signalled if all coefficients in TU are zero. Thus, PU QP signalling is also considered with TU. Therefore, the delta QP for each PU (dQPPU) in a CU is signaled, if all of the following conditions are met:

- CU depth is smaller than max_qp_delta_depth;

- At least one TU within a PU has at least one non-zero coefficient in Luma or Chroma;

- The QP of the current PU was not signaled or the current TU in PU is the first TU in PU with non-zero coefficient.

If the size of TU is equal to CU, CU QP is used for PU QP. Thus, if there are more than one PU’s, QP for those PU’s will be the same to CU QP.

The final QP for the PU, QPPU, is derived from dQPPU and predicted QPP-PU as follows;

QPPU=QPP−PU+dQPPU                               (1)

The method for QPP-PU prediction is the same used in current HM5.0.

## Syntax

We tried to minimize the code change of current HM5.0. The flag cu_qp_delta_enabled_flag in seq_parameter_set_rbsp() is re-used as qp_delta_enabled_flag in Table 1 with the setting in Table 2. The modification is highlighted yellow.

Table 1. Example of proposed syntax in Sequence parameter set RBSP syntax in HM

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| … |  |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| adaptive_loop_filter_enabled_flag | u(1) |
| qp_delta_enabled_flag | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2. Example of qp_delta_enabled_flag

| qp_delta_enabled_flag | level of delta QP |
| --- | --- |
| 0 | no delta QP |
| 1 | delta QP at sub-LCU (CU) |
| 2 | delta QP at sub-CU (PU) |

A new pu_qp_delta() function unit in Table 3 is added for PU dQP signaling. In this CE test version, the position of pu_qp_delta() is located between transform_tree() and transform_coeff(). Note that pu_qp_delta() function can be located after transform_tree() and transform_coeff() or even before transform_tree() and transform_coeff() if transform and cbf information is available. Also it is possible to merge the functionality of pu_qp_delta() into transform_coeff().

Table 3. Example of proposed syntax of pu_qp_delta in HM

| pu_qp_delta ( x0, y0, log2TrafoSize, trafoDepth ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| pu_qp_delta( x0, y0, log2TrafoSize − 1, trafoDepth + 1) |  |
| pu_qp_delta ( x1, y0, log2TrafoSize − 1, trafoDepth + 1) |  |
| pu_qp_delta ( x0, y1, log2TrafoSize − 1, trafoDepth + 1) |  |
| pu_qp_delta ( x1, y1, log2TrafoSize − 1, trafoDepth + 1) |  |
| } else { |  |
| if(IsPuDQPNeedToCode) { |  |
| pu_qp_delta | se(v) ¦ ae(v) |
| } |  |
| } |  |
| } else { |  |
| pu_qp_delta = QPP-PU |  |
| } |  |
| } |  |

# Simulation Results

The simulation results under the CE4 common test condition [3] are provided in Table 4. From the average of the results in Class A (only in RA), B, C, D and E (only in LB), a loss – 0.3% and 0.5% for RA-HE and LB-HE is observed, respectively. A gain – 0.6% for LB-LC is observed. Also note that the Enc/Dec Time provided may not be accurate because the simulations were distributed over different machines.

To further analyze the reason for coding gain achieved for LB-LC, BD-rate performance of selected sequences with coding gain over 1% are shown in Table 5.

Table 4. Coding performance of the proposed method with MaxCUdQPdepth set to 3

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 0.3% | 0.9% | 0.8% | 2.8% | 0.1% | 0.8% | 0.8% | 2.3% |
| Class B | 0.2% | 0.6% | 0.5% | 0.2% | 0.1% | 0.4% | 0.4% | 0.9% |
| Class C | 0.2% | 0.5% | 0.6% | 2.4% | 0.1% | 0.5% | 0.5% | 2.4% |
| Class D | 0.1% | 0.5% | 0.4% | 2.2% | 0.1% | 0.3% | 0.3% | 2.3% |
| Class E | 0.3% | 0.9% | 0.9% | 0.6% | 0.2% | 1.2% | 1.1% | 1.6% |
| Overall | 0.2% | 0.6% | 0.6% | 1.5% | 0.1% | 0.6% | 0.6% | 1.8% |
|  | 0.2% | 0.6% | 0.6% |  | 0.1% | 0.6% | 0.6% |  |
| Class F | 0.1% | 0.3% | 0.4% | 0.8% | 0.1% | 0.2% | 0.2% | 1.1% |
| Enc Time[%] | 87% |  |  |  | 90% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 84% |  |  |  |

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 0.3% | 0.9% | 0.7% | 1.7% | 0.1% | 0.7% | 0.7% | 3.1% |
| Class B | 0.3% | 0.3% | 0.5% | -1.7% | 0.0% | 0.6% | 0.3% | 0.0% |
| Class C | 0.3% | 0.5% | 0.3% | 1.4% | 0.1% | 0.3% | 0.2% | 1.5% |
| Class D | 0.3% | 0.2% | 0.0% | 3.3% | 0.0% | 0.2% | 0.1% | 2.8% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.3% | 0.4% | 0.3% | 0.9% | 0.0% | 0.4% | 0.3% | 1.6% |
|  | 0.3% | 0.4% | 0.3% |  | 0.0% | 0.4% | 0.3% |  |
| Class F | 0.0% | 0.0% | 0.2% | -0.2% | -0.4% | -0.4% | -0.2% | 0.1% |
| Enc Time[%] | 109% |  |  |  | 120% |  |  |  |
| Dec Time[%] | 92% |  |  |  | 98% |  |  |  |

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | 0.1% | 0.1% | 0.2% | -4.6% |
| Class B | 0.3% | 0.1% | 0.3% | -2.0% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | 0.2% | 0.1% | 0.2% | -3.2% |
|  | 0.3% | 0.1% | 0.2% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 110% |  |  |  |
| Dec Time[%] | 89% |  |  |  |

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.5% | 0.9% | 1.3% | -3.6% | -1.0% | 0.0% | 0.0% | -0.3% |
| Class C | 0.6% | 0.7% | 0.9% | 1.7% | 0.0% | 0.5% | 0.9% | 2.3% |
| Class D | 0.6% | 0.8% | 1.1% | 4.9% | -0.2% | 1.0% | 0.8% | 4.3% |
| Class E | 0.4% | 0.4% | 1.7% | -6.4% | -1.3% | -0.7% | 0.0% | -2.3% |
| Overall | 0.5% | 0.7% | 1.2% | -0.7% | -0.6% | 0.2% | 0.4% | 1.1% |
|  | 0.5% | 0.7% | 1.1% |  | -0.6% | 0.2% | 0.4% |  |
| Class F | 0.3% | 0.4% | 0.4% | 0.2% | -0.4% | 0.7% | 0.6% | 0.1% |
| Enc Time[%] | 87% |  |  |  | 114% |  |  |  |
| Dec Time[%] | 75% |  |  |  | 207% |  |  |  |

Table 5. Selected BD-rate performance for LB-LC condition

|  | QPISlice | kbps | Y psnr | dQP kbps | kbps | Y psnr | dQP kbps |  | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cactus | 22 | 25608.69 | 38.43 | 567.04 | 26084.86 | 38.47 | 599.85 |  | -2.1% |
|  | 27 | 7966.35 | 36.39 | 256.53 | 7957.16 | 36.44 | 269.29 |  |  |
|  | 32 | 3462.11 | 34.43 | 129.58 | 3451.57 | 34.47 | 129.99 |  |  |
|  | 37 | 1652.19 | 32.22 | 70.70 | 1634.32 | 32.23 | 65.71 |  |  |
| BQTerrace | 22 | 47512.79 | 37.51 | 958.06 | 48039.26 | 37.53 | 1051.90 |  | -1.5% |
|  | 27 | 7945.19 | 35.11 | 240.00 | 7961.40 | 35.15 | 253.83 |  |  |
|  | 32 | 2283.43 | 33.32 | 83.77 | 2268.80 | 33.33 | 83.38 |  |  |
|  | 37 | 872.21 | 31.17 | 34.35 | 864.54 | 31.15 | 32.61 |  |  |
| BlowingBubbles | 22 | 2086.54 | 37.38 | 71.14 | 2093.52 | 37.42 | 76.20 |  | -1.1% |
|  | 27 | 890.72 | 33.74 | 36.99 | 894.40 | 33.81 | 39.36 |  |  |
|  | 32 | 380.80 | 30.44 | 18.26 | 382.28 | 30.49 | 19.00 |  |  |
|  | 37 | 161.42 | 27.51 | 8.50 | 161.90 | 27.56 | 8.53 |  |  |
| Vidyo1 | 22 | 2282.94 | 42.96 | 104.80 | 2269.37 | 43.03 | 111.07 |  | -2.8% |
|  | 27 | 854.68 | 40.84 | 42.78 | 843.80 | 40.88 | 43.15 |  |  |
|  | 32 | 404.81 | 38.44 | 21.07 | 400.68 | 38.50 | 20.06 |  |  |
|  | 37 | 208.65 | 35.79 | 10.55 | 206.20 | 35.84 | 9.35 |  |  |
| Vidyo4 | 22 | 2413.99 | 42.79 | 110.90 | 2411.29 | 42.82 | 114.25 |  | -1.2% |
|  | 27 | 799.52 | 40.55 | 43.51 | 798.31 | 40.57 | 42.87 |  |  |
|  | 32 | 358.97 | 38.16 | 20.98 | 354.78 | 38.16 | 18.98 |  |  |
|  | 37 | 183.73 | 35.60 | 10.54 | 179.78 | 35.59 | 8.89 |  |  |
| ChinaSpeed | 22 | 6833.31 | 41.56 | 186.78 | 6850.68 | 41.61 | 190.85 |  | -1.4% |
|  | 27 | 3291.93 | 37.58 | 107.04 | 3301.99 | 37.66 | 107.85 |  |  |
|  | 32 | 1513.79 | 33.93 | 56.14 | 1520.87 | 34.03 | 55.50 |  |  |
|  | 37 | 673.48 | 30.74 | 28.67 | 677.21 | 30.82 | 27.29 |  |  |

# Visual Quality Improvement

Fig 1. BQMall LBLC CU Frame 324 QP37

Fig 2. BQMall LBLC PU Frame 324 QP37

The following two figures are zoom (2x) to the specific area of the frame number 324.

Fig 3. Area of BQMall LBLC CU QP37            Fig 4. Area of BQMall LBLC PU QP37

# Fig 5. RaceHorsesC LBLC CU Frame 161 QP37

# Fig 6. RaceHorsesC LBLC PU Frame 161 QP37

# The following two pictures are the area of RaceHorsesC LBLC CU vs. PU for QP37 at frame 161 which shows visible difference.

# Fig 7. RaceHorsesC CU DQP                         Fig 8. RaceHorsesC PU DQP

# Conclusions

In this contribution, syntax for PU-level of QP granularity is proposed and the performance of QP adaptation at sub-CU level with additional constraint is evaluated.

Because activity based model targets to improve visual quality, not R-D performance, a visual quality testing should be done to see the impact. As shown in associated input contributions, this approach does provide better visual quality. Also note that in addition to subjective quality improvement, proposed solution achieved 1% BD-rate gains in LB-LC.

Furthermore, this approach allow finer granularity of rate control resulting in more accurate bit rates at the output of an encoder which is very important in many applications.

Therefore, we recommend adoption of PU-level QP granularity with additional constraint as implemented in the released software to the next HEVC WD.

# References

- K. Panusopone, A. Luthra, X. Fang, L. Wang, “QP adaptation at sub_CU level,” JCTVC-F577

- K. Sato, M. Budagavi, M. Coban, H. Aoki, X. Li, “Description of Core Experiment 4: Quantization,” JCTVC-G1204

- Madhukar Budagavi, Minhua Zhou, “Delta QP signaling at sub-LCU level” JCTVC-D038

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).