| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0509-r3 |
| --- | --- |

| Title: | Simplification of significance map coding for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document describes implementation details and reports the results of Motorola Mobility’s simplification of significance map coding. Simulation results show that there is a loss of 0.1% and 0.1% for AIHE and AILC, a loss of 0.1% and 0.0% for LBHE and LBLC and a loss of 0.0%, 0.1%, and 0.2% for RAHE, RALC, and RAHE 10bit conditions respectively. Cross-Check will be provided by MediaTek.

# Introduction

The significance map of a TU (transform unit) indicates the positions of non-zero coefficients in the TU. In HM5.0, a two–level significance map (level zero L0 and level one L1) [1] is adopted for large transform units (LTU). Different contexts are applied to L1 and L0 significance maps based on different rules.

In [1], context for a current element (subblock) in L1 significance map depends on the element’s (subblock’s) immediate right and bottom neighbors, as shown in Fig 1 (a):

- 2 contexts for luma: a first context is used if neither the immediate right nor immediate bottom subblock neighbors contain non-zero coefficients, otherwise, a second context is used.

- 2 contexts for chroma: the same derivation rule as luma.

In [2], L0 significance map of luma LTU is separated to three regions, as shown in Fig. 2 (a) where there are three difference colors, one for each of three regions. L0 significance map of chroma LTU is separated to two regions (DC region and AC region), where DC region only contains one DC element and AC region contains the remaining elements.  Context for a current element in L0 significance map depends on the element’s four or five neighbors, as shown in Fig 1 (b) and (c), respectively.  In HM5.0, the element in top-left corner of a sub-block and the element right above the right-bottom corner of a sub-block derive their contexts based on their four neighbors while other elements other than the DC bit determine their contexts by their five neighbors [3], as shown in Fig 3.  The detailed rules of context decision are:

- 1 context for region 0 in luma and 1 context for DC region in chroma.

- 3 contexts for region 1 in luma: a first context is used if there is no non-zero coefficient in neighborhood; a second context is used if there are 1 or 2 non-zero coefficients in neighborhood; a third context is used if there are more than 3 non-zero coefficients in neighborhood.

- 3 contexts for region 2 in luma: the same derivation rule as with (2), but these 3 contexts are different from those in (2).

- 3 contexts for all the elements except DC component in chroma: the same derivation rule as with (2), but these 3 contexts are different from those in (2) and (3).

- Threshold for partition of region 1 and region 2 (thred) is determined based on size of the TU (Tsize): thred =Tsize>>2

(a) L1		(b) L0 - four neighbor 	(c) L0 – five neighbors

Fig. 1. Neighbors of significant bits, where “x” is the current element and “o” are the neighbors.

# Simplification of significant map coding for CABAC

We propose to harmonize luma and chroma contexts to simplify the procedure of context determination as well as to reduce number of contexts. Specifically, some context models and context values for significance map coding can be shared between luma TU and chroma TU.

We also propose to use two regions (region 0 and region 1), as shown in Fig. 2 (b), to determine context models for L0 significance map coding of both luma TU and chroma TU, when TU sizes are 16x16, 32x32, 4x16, 16x4, 8x32, 32x8. Region 0 contains DC coefficient in the TU, and region 1 contains the remaining coefficients.

In addition, we propose to derive the context for L0 significance map coding for every element in region 1 based on their four neighbors, as shown in Fig. 1 (b).  By this way, there is no need to differentiate the general or special positions in a subblock.

The detailed proposed harmonization is given as follows.

For L1 significance map coding, we propose to merge 2 luma and 2 chroma contexts so that only 2 contexts are used for both luma and chroma TU. That is, for a current subblock (in either luma or chroma TU), a first context is used if none of the subblock’s immediate right and immediate bottom neighbors contains non-zero coefficient; otherwise, a second context is used.

For L0 significance map coding, we propose the following context modeling for two regions of a TU (Fig 2 (b)).

- 1 context for region 0 both in luma and chroma. The context is the same for both luma and chroma.

- 4 contexts for region 1 in luma: given a current element, a first context is used if all the coded neighbors of the element are zero coefficients; a second context is used if one of the coded neighbors of the element is non-zero coefficient; a third context is used if two of the coded neighbors of the element are non-zero coefficients; a fourth context is used if more than 3 of the coded neighbors of the element are non-zero coefficients.

- 4 contexts for region 1 in chroma: the same derivation rule as with (2). The 4 contexts are the same as those in (2).

(a) Original	                    			         (b) Proposed

Fig 2. Contexts for significance map coding of large transform unit, where there are three regions in (a) and two regions in (b).

Fig 3. Context determination for special significant bit positions based on less neighbors.

With all our schemes together, the contexts for L1 significance map is reduced from 4 to 2. The contexts for L0 significance map of LTU is reduced from 11 to 5. When TU sizes are 16x16, 32x32, 4x16, 16x4, 8x32, 32x8, operations required for significance map coding can be simplified with the proposed modifications in four following ways.

- Common significance map coding operations apply to both luma and chroma TU.

- Two zones, with only DC component in the region 0, are used for luma TU, thereby eliminating region 2 or 3 checking.

- Common 4 neighbor context model is used for all location in region 1 in each subblock, thereby eliminating two special locations check in each subblock.

- The proposed 4 contexts mapping requires a simpler logic to determine context of element in region 1 for L0 significance map.

# Simulation Results

We have two sets of simulation results. One is with all proposed tools enabled, the experimental results of the rate-distortion and speed performance comparison are summarized in Table 1 and 2.  The other is with all tools enabled except the tool with only 4 neighbors for every significance bit, the experimental results of the rate-distortion and speed performance comparison are summarized in Table 3 and 4.  Detailed results are included in the attachment (JCTVC-H0509.xls, JCTVC-H0509-LowQP.xls, JCTVC-H509modified.xls and JCTVC-H509modified_LowQP.xls).

Table 1 QP (22 27 32 37)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.3% | 0.4% | 0.2% | 0.3% | 0.3% |  |  |  |
| Class C | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class E | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.3% |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.3% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 105% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | 0.2% | -0.4% | 0.5% |
| Class B | 0.1% | 0.2% | 0.4% | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.1% |
| Class C | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class D | 0.1% | 0.0% | 0.2% | 0.0% | 0.4% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.2% | 0.1% | 0.2% | 0.1% | 0.2% | -0.1% | 0.2% |
|  | 0.1% | 0.0% | 0.2% | 0.1% | 0.2% | 0.0% | 0.1% | 0.0% | 0.3% |
| Class F | -0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 143% |  |  | 142% |  |  |
| Dec Time[%] | 97% |  |  | 89% |  |  | 92% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.4% | 0.1% | -0.2% | 0.5% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class D | 0.2% | 0.0% | -0.1% | 0.0% | 0.8% | 0.3% |  |  |  |
| Class E | 0.2% | -0.6% | -0.4% | 0.0% | -0.6% | -0.3% |  |  |  |
| Overall | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.2% |  |  |  |
|  | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.2% |  |  |  |
| Class F | -0.2% | -0.1% | -0.4% | 0.3% | -0.5% | -0.3% |  |  |  |
| Enc Time[%] | 138% |  |  | 186% |  |  |  |  |  |
| Dec Time[%] | 92% |  |  | 140% |  |  |  |  |  |

Table 2 QP (12 17 22 27)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class E | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 87% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 116% |  |  | 116% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.5% | 0.8% |
| Class B | 0.0% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% | 0.0% | 0.3% | 0.3% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.4% | 0.5% |
|  | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.5% | 0.5% |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 152% |  |  | 117% |  |  | 131% |  |  |
| Dec Time[%] | 106% |  |  | 114% |  |  | 118% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class C | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class E | 0.0% | -0.1% | 0.1% | 0.0% | -0.3% | -0.1% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.2% | 0.3% | 0.0% |  |  |  |
| Enc Time[%] | 132% |  |  | 118% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 117% |  |  |  |  |  |

Table 3 QP (22 27 32 37)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.4% |  |  |  |
| Class B | 0.3% | 0.6% | 0.7% | 0.3% | 0.7% | 0.8% |  |  |  |
| Class C | 0.1% | 0.3% | 0.2% | 0.1% | 0.3% | 0.3% |  |  |  |
| Class D | 0.1% | 0.3% | 0.2% | 0.1% | 0.3% | 0.3% |  |  |  |
| Class E | 0.3% | 0.5% | 0.5% | 0.2% | 0.7% | 0.7% |  |  |  |
| Overall | 0.2% | 0.4% | 0.4% | 0.2% | 0.5% | 0.5% |  |  |  |
|  | 0.2% | 0.4% | 0.4% | 0.2% | 0.5% | 0.5% |  |  |  |
| Class F | 0.1% | 0.0% | 0.1% | 0.1% | 0.4% | 0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 137% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.3% | 0.2% | 0.1% | 0.3% | 0.3% | 0.4% | 0.8% | 1.2% |
| Class B | 0.2% | 0.4% | 0.5% | 0.2% | 0.5% | 0.5% | 0.2% | 0.4% | 0.4% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class D | 0.1% | -0.1% | 0.5% | 0.1% | 0.5% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.3% | 0.1% | 0.4% | 0.2% | 0.3% | 0.6% | 0.7% |
|  | 0.1% | 0.1% | 0.3% | 0.1% | 0.4% | 0.1% | 0.2% | 0.7% | 0.9% |
| Class F | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 55% |  |  | 81% |  |  | 79% |  |  |
| Dec Time[%] | 84% |  |  | 84% |  |  | 85% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.5% | 0.2% | 0.2% | -0.1% | 0.5% |  |  |  |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class D | 0.1% | -0.5% | -0.2% | 0.1% | 0.7% | 0.0% |  |  |  |
| Class E | 0.2% | 1.0% | -0.5% | 0.1% | -0.3% | -0.2% |  |  |  |
| Overall | 0.1% | 0.2% | -0.1% | 0.1% | 0.1% | 0.2% |  |  |  |
|  | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class F | 0.0% | 0.2% | -0.2% | 0.1% | -0.7% | -0.5% |  |  |  |
| Enc Time[%] | 80% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 115% |  |  | 98% |  |  |  |  |  |

Table 4 QP (12 17 22 27)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class B | 0.1% | 0.4% | 0.4% | 0.1% | 0.3% | 0.4% |  |  |  |
| Class C | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E | 0.1% | 0.2% | 0.3% | 0.2% | 0.3% | 0.3% |  |  |  |
| Overall | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
|  | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class F | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 71% |  |  | 85% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 114% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 0.1% | 1.0% | 1.3% |
| Class B | 0.1% | 0.4% | 0.3% | 0.1% | 0.3% | 0.4% | 0.1% | 0.4% | 0.4% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.7% | 0.8% |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.7% | 0.7% |
| Class F | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 88% |  |  | 93% |  |  | 86% |  |  |
| Dec Time[%] | 94% |  |  | 137% |  |  | 138% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.3% | 0.1% | 0.2% | 0.3% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E | 0.1% | 0.4% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Overall | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
|  | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class F | 0.2% | 0.0% | 0.2% | 0.4% | 0.4% | 0.1% |  |  |  |
| Enc Time[%] | 88% |  |  | 93% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 94% |  |  |  |  |  |

# Conclusions

In this document, several simplifications of significance map coding were proposed and its overall performance was evaluated. The proposed simplification reduces complexity by harmonizing the coding process between luma TU and chroma TU, and between subblock within a TU. Hence, it is recommended to adopt the proposed simplifications into the next HEVC WD.

# Reference

[1] 	N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song, “Multi-level Significant Maps for Large Transform Units,” JCTVC-G644.

[2] 	C. Auyeung, J. Xu, G. Korodi, J. Zan, D. He, Y. Piao, E. Alshina, J. Min, J. Park, “A combined proposal from JCTVC-G366, JCTVC-G657, and JCTVC-G768 on context reduction of significance map coding with CABAC,” JCTVC-G1015.

[3] J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” JCTVC-G323.

# Patent Right Declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text for Working Draft

###### 9.2.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_group_flag

Inputs to this process are the current coefficient group scan position ( xCG, yCG ), the previously decoded bins of the syntax element significant_coeff_group_flag, the width of the current transform block, log2TrafoWidth and the height of the current transform block, log2TrafoHeight.

Output of this process is ctxIdxInc, which is derived using the current position ( xCG, yCG ), two previously decoded bins of the syntax element significant_coeff_group_flag in scan order, and the width of the current transform block, log2TrafoWidth and the height of the current transform block, log2TrafoHeight as follows.

- ctxIdxInc is initialized with 0 as follows.

ctxIdxInc =  0		(9-1)

- When xCG is less than ( 1 << ( log2TrafoWidth − 2 ) ) − 1, ctxIdxInc is modified as follows.

ctxIdxInc +=  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-2)

- When yCG is less than ( 1 << ( log2TrafoHeight − 2 ) ) − 1, ctxIdxInc is modified as follows.

ctxIdxInc +=  significant_coeff_group_flag[ xCG ][ yCG + 1 ]	(9-3)

NOTE – The values of ctxIdxInc are in the range of 0 to 2, inclusive. A ctxIdxInc value of 3 does not occur because significant_coeff_group_flag[ xCG ][ yCG ] is inferred when significant_coeff_group_flag[ xCG + 1 ][ yCG ] and significant_coeff_group_flag[ xCG ][ yCG + 1 ] are both equal to 1.

###### 9.2.3.1.1.5 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Table 9-1 as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-4)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Table 9-2 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-5)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-6)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  20		(9-7)

- Otherwise (xC + yC is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-8)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-9)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-10)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-11)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ] 	(9-12)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-13)

- The variable sigCtx is modified as follows.

sigCtx =  21 + min(3, sigCtx)		(9-14)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0 or max(log2TrafoWidth, log2TrafoHeight)>3, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-15)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-16)

Table 9-1 – Specifcation of ctxIdxMap4x4[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap4x4[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |
| i | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| ctxIdxMap4x4[ i ] | 0 | 1 | 2 | 4 | 1 | 1 | 2 | 4 | 3 | 3 | 5 | 5 | 4 | 4 | 5 |

Table 9-2 – Specifcation of ctxIdxMap8x8[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap8x8[ i ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 3 | 8 | 6 | 6 | 7 | 9 | 9 | 7 | 7 |

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]