| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0510 WG11 Number: m23391 |
| --- | --- |

| Title: | Non-CE1: High Throughput Binarization (HTB) method with modified level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra, Louis Kerofsky Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

- In this contribution, a new efficient high throughput binarization (HTB) method for CABAC is presented.

- The intention of this approach is to reduce the worst-case complexity of CABAC for low complexity use cases, keeping easy compatibility with the existing CABAC. In addition, the proposed method provides the flexibility between coding performance and throughput efficiency by selectively applying HTB mode for each 4x4 coefficient block. It is reported that the proposed method reduces the number of context adaptively coded bins about 24 % under the common test conditions, and about 46 % in the low QP range where the complexity of CABAC is reportedly more problematic. It is further reported that the proposed approach can provide more flexible HTB method by employing selective activation of HTB mode in each sub-block level between coding performance and throughput efficiency.

# Introduction

In HM5.0, the only entropy coder CABAC provides even better coding efficiency than CAVLC because of its arithmetic coding engine and sophisticated context modeling. However, those efficient coding techniques require large amount of computational complexity and degrade the throughput efficiency especially at higher bitrates condition. It was also noticed that on higher bitrates the portion of transform coefficient data in the generated bitstream has a dominant role. Thus, in order to improve the worst case throughput we need to provide the codec with a higher throughput alternative for low-complexity use cases.

# High Throughput Binarization Coding Method

- The current CABAC consists of three main coding processes (last position, significance map, level coding) for residual coding. Here, last position coding part dose not require much computational complexity and significant throughput problem even in high bitrate coding. For the significance map, it contributes significantly to the coding performance. Hence, considering the coding performance and throughput efficiency, the level coding part shows room for an improvement. Specifically, current level coding consists of four sub-coding processes Greater_than_1, Greater_than_2, Sign, and Absolute-3 and also requires many context model update for the first two processes (Greater_than_1 and Greather_than_2) in high bitrate coding because many large coefficients are likely to be observed.

Figure 1 Proposed HTB Structure for CABAC

Therefore, we introduce a new HTB coding mode with simple structure as shown in Figure 1. In order to provide flexible functionality between coding performance and high throuput performance, the proposed HTB coding structure selectively activate the HTB coding mode. Basically, proposed HTB CABAC structure is the same as those of CABAC in last position coding and significance map coding. For the level coding, HTB mode is selectively activated based on the number of significant coefficients as follows.

If (Number_of_significant_coeffs < = TH)

HTB =0;

Else HTB=1;

where ‘TH’ represent a given threshold value. For improvement of the coding performance of the level coding, we combine sign and level information as indicated in eq. (1) and Fig.2, and generate new level information which is to be binarized with corresponding binarizer. Here, we introduce two binarization methods. One is based on VLC tables used in CAVLC and the other is the based on the Golomb-Rice code in HM5.0

Input = (abs (coeff[i]-1) <<1 + sign                 (1)

Figure 2 Mapping Table for ‘Input’

For the first one, predefined 5 vlc tables used in CAVLC are adaptively updated. Here, table number ‘vlc’ is first set to zero and it is monotonically increased based on the following condition;

If (input > Table [vlc]) vlc ++       where Table [vlc] = {3, 5, 13, 27}

Table update is terminated when ‘vlc’ is equal to four.

In our second binarization, the Golomb-Rice code in HM5.0 is basically employed. Here, the range of Golomb-Rice parameter is extended to {0, 1, 2, 3, 4}. The Golomb-Rice parameter is initially set to one when the numGreater1 is greater than 3; otherwise, it is set to zero. numGreater1 is defined as in HM working draft [5] which is used for context set selection for greaten_than_1 and greaten_than_2 syntax coding. Golomb-Rice parameter (cRiceParam) is monotonically increase based on the following condition;

If (cRiceParam > 0)

cRiceParam = g_aauiGoRiceUpdate [cRiceParam-1][min(15, input>>1 ]+1;

else if (input>0)

cRiceParam = 1;

where g_aauiGoRiceUpdate is defined in HM5.0 and listed in Table 1.

| cRiceParam | min(15, input>>1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
| 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

# Results

We implemented the HTB mode technology into the HM5.0 software and tested with the recommended configuration on all sequences. The following tables below show the summary results of various HTB methods vs. HM5.0 anchor.

M1: CAVLC based binarization, TH=0

M2+TH_2: Golomb-rice binarization, TH=2

M2+TH_8: Golomb-rice binarization, TH=8

## Test 1: common conditions BD-Rate performance

Table 1. BD-Rate performance at common test conditions Qp= [22, 27, 32, 37]

| Configuration | M1 |  |  | M2+TH_2 |  |  | M2+TH_8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| AI-HE | 2.2% | 2.7% | 2.6% | 1.2% | 1.3% | 1.3% | 0.2% | 0.3% | 0.3% |
| RA-HE | 1.9% | 2.4% | 2.4% | 1.2% | 1.0% | 1.0% | 0.1% | 0.1% | 0.1% |
| LB-HE | 2.2% | 3.6% | 3.2% | 1.6% | 0.9% | 0.7% | 0.2% | 0.0% | 0.2% |
| AI-LC | 2.3% | 2.7% | 2.7% | 1.0% | 1.1% | 1.1% | 0.0% | 0.1% | 0.0% |
| RA-LC | 1.5% | 1.8% | 1.6% | 0.6% | 0.6% | 0.5% | 0.0% | 0.0% | -0.1% |
| LB-LC | 1.8% | 2.5% | 2.3% | 0.7% | 0.2% | 0.0% | 0.0% | -0.2% | -0.1% |
| RA-HE10 | 2.8% | 3.5% | 3.4% | 2.0% | 1.4% | 1.6% | 0.0% | -0.2% | -0.1% |

Table 2. BD-Rate performance at low QP test conditions Qp= [1, 5, 9, 13]

| Configuration | M1 |  |  | M2+TH_2 |  |  | M2+TH_8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| AI-HE | 1.3% | 1.2% | 1.2% | 0.9% | 1.3% | 1.4% | 0.6% | 0.8% | 0.8% |
| RA-HE | 2.7% | 3.4% | 3.2% | 3.1% | 3.5% | 3.3% | 1.9% | 1.9% | 1.7% |
| LB-HE | 2.9% | 3.4% | 3.4% | 1.6% | 0.9% | 0.7% | 0.3% | 0.4% | 0.4% |
| AI-LC | 1.3% | 1.0% | 1.0% | 0.7% | 0.9% | 0.9% | 0.3% | 0.4% | 0.4% |
| RA-LC | 2.1% | 2.3% | 2.1% | 1.8% | 2.1% | 1.9% | 0.6% | 0.7% | 0.6% |
| LB-LC | 2.1% | 2.3% | 2.3% | 2.0% | 2.1% | 2.0% | 0.8% | 0.7% | 0.7% |
| RA-HE10 | 2.9% | 4.4% | 4.4% | 3.7% | 4.2% | 3.9% | 2.0% | 2.0% | 1.8% |

It is reported that the proposed method reduces the number of context adaptively coded bins up to 39 % under the common test conditions, and up to 56 % in the low QP range (QP 1 ,5, 9, 13) where the complexity of CABAC is reportedly more problematic. The average Y-BD bitrate change in common condition is observed to be LD-HE: 2.2%, RA-HE: 1.9%, and AI-HE: 2.2%.

Table 3. Average savings of context coded bin at each configuration

| Configuration | M1 |  | M2+TH_2 |  | M2+TH_8 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Common | Low QP | Common | Low QP | Common | Low QP |
| AI-HE | 29% | 45% | 25% | 44% | 11% | 37% |
| RA-HE | 19% | 38% | 16% | 37% | 5% | 27% |
| LB-HE | 17% | 37% | 12% | 38% | 3% | 28% |
| AI-LC | 25% | 43% | 20% | 42% | 8% | 35% |
| RA-LC | 15% | 33% | 11% | 32% | 3% | 19% |
| LB-LC | 13% | 32% | 8% | 30% | 1% | 19% |
| RA-HE10 | 23% | 39% | 9% | 39% | 9% | 28% |

Table 4. Average savings of maximum context coded bin at each configuration

| Configuration | M1 |  | M2+TH_2 |  | M2+TH_8 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Common | Low QP | Common | Low QP | Common | Low QP |
| AI-HE | 31% | 51% | 29% | 50% | 15% | 48% |
| RA-HE | 25% | 46% | 21% | 49% | 8% | 37% |
| LB-HE | 22% | 43% | 18% | 43% | 6% | 37% |
| AI-LC | 28% | 51% | 25% | 49% | 11% | 41% |
| RA-LC | 19% | 41% | 14% | 43% | 5% | 36% |
| LB-LC | 15% | 41% | 10% | 40% | 4% | 31% |
| RA-HE10 | 27% | 43% | 11% | 44% | 11% | 36% |

Table 5 BD-Rate changes and throughput efficiency (common condition)

|  | M1 | M2+TH_2 | M2+TH_8 |
| --- | --- | --- | --- |
| BD-Rate | 2.1 % | 1.2 % | 0.1 % |
| AVG-Saving | 20 % | 15 % | 6 % |
| AVG-W_saving | 24 % | 18 % | 9 % |

Table 6 BD-Rate changes and throughput efficiency (low QP)

|  | M1 | M2+TH_2 | M2+TH_8 |
| --- | --- | --- | --- |
| BD-Rate | 2.2% | 2.0% | 1.0 % |
| AVG-Saving | 38 % | 37 % | 28 % |
| AVG-W_saving | 45 % | 45 % | 38 % |

# Conclusion

Introduced high throughput binarization (HTB) with modified level coding has been verified to improve the throughput performance in common condition and high bitrate (low QP) environment. It also provides a more flexible coding structure by selectively employing the HTB coding mode in CABAC.

Given this level of performance and the importance of anticipating a wide variety of applications in the HEVC standard, we propose the JCT-VC adopt the proposed HTB method into the HM.

# Reference

- JCTVC-G1201, “Description of Core Experiment 1 (CE1): Entropy coding investigation”, R. Joshi, E. Alshina, H. Sasai, H. Kirchhofer, J. Lainema, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011

- JCTVC-G569, “Single entropy coder for HEVC with a high throughput binarization mode“, J. Lainema, K. Ugur, A. Hallapuro (Nokia), 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011

- JCTVC-G1200, “Common HM test conditions and software reference configurations”, F. Bossen, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011

- HM 5.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-5.0

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

In 7.3.2.2 Picture parameter set RBSP syntax

changes are marked in yellow

1) Activation of HTB mode

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software, depends on reference list construction decision ] |  |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| [Ed. (BB): not present in HM software, signaled in slice header as absolute value slice_qp, should be implemented to be used for slice_qp_delta] |  |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| [Ed. (BB): not present in HM software, should be implemented ] |  |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| tile_info_present_flag | u(1) |
| if( tile_info_present_flag = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| HTB_mode_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- 9.2.2.10 binrization process for syntax element cn

If HTB mode is activated the absolute-1 and sign information are combined and generates syntax elements cn as follows.

cn [pos] = (abs (absolute-1[pos]-1) <<1 + sign

The syntax element cn is binarized based on extended golomb-rice code as specified in subclause 9.2.2.9.

The variable cRiceParam is derived from cLastSE and cLastRiceParam as given in Table 9-7.

Table 9-37 – Specifcation of cRiceParam depending on cLastSE and cLastRiceParam

| cLastRiceParam | cLastSE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | >10 |
| 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

The variable cTRMax is derived from cRiceParam as given in Error: Reference source not found8.

Table 9-38 – Specifcation of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 20 |
| 2 | 42 |
| 3 | 70 |
| 4 | 174 |

The cRiceParam is initialized as follows

- The extended cRiceParam is initially set to one when the numGreater1 is greater than 3;

- Otherwise, it is set to zero.

cRiceParam is monotonically increased based on the following condition;

If (cRiceParam > 0)

cRiceParam = g_aauiGoRiceUpdate [cRiceParam -1][min(15, cn[pos]>>1 ]+1;

Else if (cn[pos]>0)

cRiceParam = 1;

### Residual coding syntax

| residual_coding ( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| cn[ n ] | ae(v) |
| } |  |
|  | ae(v) |
|  | ae(v) |
|  | ae(v) |
| } |  |
| } |  |

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Error: Reference source not found | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found |  |  |  |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found |  |  |  |