| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0511-r2 |
| --- | --- |

| Title: | Entropy Coding for Non Square Blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document describes implementation details and reports the results of Motorola Mobility’s entropy coding for non square blocks. Simulation results show that there is a loss of 0.0% for LBHE and a loss of 0.0%, and 0.1% for RAHE, and RAHE 10bit conditions respectively. Cross-Check will be provided by HiSilicon.

# Introduction

In HM-5.0, transform coefficient coding for CABAC is able to handle non-square TU directly without mapping non-square TU into square TU first. Additionally, a two–level significance map (level zero L0 and level one L1) [1] is adopted for large TU (16x16 and 32x32) and non square TU (16x4, 32x8, 4x16 and 8x32). This contribution investigates the impact of coding only L0 significance map for 16x4 TU and 4x16 TU since 8x8 TU, which has the same total number of coefficient, does not utilize L1 significance map coding. Moreover, this contribution also tests shape adaptive scan and a reduced number of context for significance map coding as described in [2].

# Entropy Coding for Non Square Blocks

We propose a generalized transform coefficient coding process for the TU block (square or non square). The detailed changes to the current HM transform coefficient coding process are listed as follows:

- Scanning order is set according to the TU block shape.  Fig 1 shows wave front scan for 16x4 and 4x16 TU blocks. For 32x8 or 8x32 blocks, we make it an option to set shape adaptive single level scan as Fig. 1 or multilevel scan as HM5.0. In the experiment, single level scan is used for 16x4 and 4x16 TU blocks and multilevel scan is used for 32x8 and 8x32 TU blocks.

- For L1 significance map coding, we propose to merge 2 luma and 2 chroma contexts as described in [2] so that only 2 contexts are used for both luma and chroma TU. That is, for a current subblock (in either luma or chroma TU), a first context is used if none of the subblock’s immediate right and immediate bottom neighbors contains non-zero coefficient; otherwise, a second context is used.

- For L0 significance map coding, we propose to separate both luma and chroma TUs into two regions, as described in [2] and shown in Fig. 2. For region 0, which only contains DC element, a fixed context is assigned. For region 1, we propose to derive the context for every element based on their four neighbors, as shown in Fig. 3. 4 contexts are used in region 1.  By this way, there is no need to differentiate the general or special positions in a subblock. In addition, Both luma and chroma are sharing the same contexts.

Fig. 1 Wave front scan for 16x4 and 4x16 TU

(a) Original	                    			         (b) Proposed

Fig 2. Contexts for significance map coding of large transform unit, where there are three regions in (a) and two regions in (b).

Fig. 3. Neighbors of L0 significant bits, where “x” is the current element and “o” are the neighbors.

# Simulation Results

We have two sets of simulation results. One is with all proposed tools enabled, the experimental results of the rate-distortion and speed performance comparison are summarized in Table 1.  The other is with all tools enabled except the tool which is using 4 neighbors for every significance bit, the experimental results of the rate-distortion and speed performance comparison are summarized in Table 2.  Detailed results are included in the attachment (JCTVC-H0511.xls and JCTVC-H0511_v2.xls).

Table 1 Simulation results

|  | Random Access HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.2% | 0.2% | 1.3% |
| Class B | 0.1% | 0.4% | 0.5% | 0.1% | 0.4% | 0.4% |
| Class C | 0.0% | 0.0% | 0.2% |  |  |  |
| Class D | 0.1% | -0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.2% | 0.1% | 0.3% | 0.8% |
|  | 0.0% | 0.1% | 0.2% | 0.1% | 0.4% | 0.8% |
| Class F | -0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | #NUM! |  |  | 142% |  |  |
| Dec Time[%] | 98% |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.5% |  |  |  |
| Class C | 0.1% | 0.2% | 0.2% |  |  |  |
| Class D | 0.0% | -0.2% | 0.2% |  |  |  |
| Class E | 0.1% | 0.0% | -0.4% |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% |  |  |  |
|  | 0.0% | 0.3% | 0.2% |  |  |  |
| Class F | 0.0% | 0.0% | -0.2% |  |  |  |
| Enc Time[%] | #NUM! |  |  |  |  |  |
| Dec Time[%] | 147% |  |  |  |  |  |

Table 2 Simulation results of all proposed tools on

|  | Random Access HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.3% | 0.2% | 0.4% | 0.8% | 1.2% |
| Class B | 0.2% | 0.4% | 0.5% | 0.2% | 0.4% | 0.4% |
| Class C | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | -0.2% | 0.4% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.4% | 0.3% | 0.6% | 0.8% |
|  | 0.1% | 0.1% | 0.3% | 0.2% | 0.7% | 0.9% |
| Class F | 0.0% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | #NUM! |  |  | 83% |  |  |
| Dec Time[%] | 178% |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.4% | 0.5% |  |  |  |
| Class C | 0.1% | 0.1% | 0.2% |  |  |  |
| Class D | 0.1% | -0.3% | -0.1% |  |  |  |
| Class E | 0.1% | 1.1% | -0.4% |  |  |  |
| Overall | 0.1% | 0.3% | 0.1% |  |  |  |
|  | 0.1% | 0.3% | 0.1% |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | #NUM! |  |  |  |  |  |
| Dec Time[%] | 86% |  |  |  |  |  |

# Conclusions

In this document, L0 only coding of significance map and shape adaptive scan for 16x4 TU and 4x16 TU were proposed and its overall performance was evaluated. Simulation results show that the simplifications can be integrated to HM-5.0 at little coding loss.

# Reference

[1] 	N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song, “Multi-level Significant Maps for Large Transform Units,” JCTVC-G644.

[2] W.-Y. Kung, K. Panusopone, L. Wang, “Simplification of Significance Map Coding for CABAC,” JCTVC-H509.

# Patent Rights Declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).