| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0514-r7 |
| --- | --- |

| Title: | Chroma Context reduction for Last Coefficient Position Coding in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xue Fang Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2723 X.Fang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document describes the implementation details and reports the results of Motorola Mobility’s chroma context reduction for last coefficient position coding. Two context index assignments are proposed. The first version uses same contexts for 4x4 and 8x8 chroma TUs and the second version uses same contexts for all chroma TUs. Cross-Check will be provided by Huawei.

# Introduction

Last coefficient coding process codes the position (X, Y) of last nonzero transform coefficient of a TU with respect to a pre-determining scan pattern/coding order, so decoder can skip consecutive number of zero coefficients at the end of TU.

In the current HEVC WD [1], there are a total of 12 contexts used for either X or Y in chroma per picture type. Specifically, 4x4 TU uses 3 contexts for X or Y ranging from 0 to 2, 8x8 TU uses 4 contexts for X or Y ranging from 3 to 6, and 16x16 TU has 5 contexts for X or Y ranging from 7 to 11. Context indices are assigned to  each bin of X or Y in chroma as follows;

4x4: 	 0,  1,  2,  2

8x8:	 3,  4,  5,  5,  6,   6

16x16:	 7,  8,  9,  9, 10, 10, 11, 11

# Proposal

We propose to reduce the total number of contexts used for last significance coefficient coding of a chroma TU. This can be achieved by assigning the same contexts to a few consecutive bins of X or Y per TU size. For example, the first and the second bins of X or Y for a TU size may be assigned a same context. The context reduction can also be achieved by assigning the same contexts to certain bins of X or Y for different TU sizes. These certain bins may be in relatively similar significant bit positions of X or Y for different TU sizes. For example, the first bin of X or Y in 4x4 TU and 8x8 TU may use a same context.

Two examples of context index assignments are proposed as described below.

The first proposal uses 3 contexts for 4x4 TU and 8x8 TU, and 3 different contexts for 16x16 TU, thereby reducing the total number of contexts from 12 to 6. Context index assignment for each bin of X or Y in chroma is as follows;

4x4: 	 0,  1,  2,

8x8:	 0,  0,  1,  1,  2,

16x16:	 3,  3,  4,  4, 5, 5, 5,

The second proposal uses 3 contexts for 4x4 TU, 8x8 TU and 16x16 TU, thereby reducing the total number of contexts from 12 to 3. Context index assignment for each bin of X or Y in chroma is shown as follows;

4x4: 	 0,  1,  2

8x8:	 0,  0,  1,  1,  2

16x16:	 0,  0,  0,  0, 1, 1, 1

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison of the first context index assignment and the second context index assignment are summarized in Tables 1 and 2, respectively. Detailed results are included in the attachment (JCTVC-H0514.xls).

Table 1 Simulation results for the first context index assignment

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 96% |  |  | 108% |  |  |
| Dec Time[%] | 87% |  |  | 108% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.4% | 1.1% |
| Class B | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.3% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.3% | 0.0% | 0.5% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.0% | 0.0% | 0.3% | 0.7% |
|  | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.0% | 0.0% | 0.3% | 0.6% |
| Class F | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 85% |  |  | 106% |  |  | 79% |  |  |
| Dec Time[%] | 83% |  |  | 103% |  |  | 78% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.5% | 0.7% | 0.0% | 0.3% | 0.4% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.9% | 0.2% |
| Class E | 0.1% | 0.0% | 0.1% | 0.0% | -2.3% | -0.2% |
| Overall | 0.0% | 0.2% | 0.3% | 0.0% | -0.1% | 0.2% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | -0.1% | 0.1% |
| Class F | -0.1% | 0.1% | 0.1% | 0.1% | 0.1% | -0.6% |
| Enc Time[%] | 102% |  |  | 107% |  |  |
| Dec Time[%] | 437% |  |  | 96% |  |  |

Table 2 Simulation results for the second context index assignment

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 67% |  |  | 87% |  |  |
| Dec Time[%] | 86% |  |  | 87% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.3% | 0.1% | 0.2% | -0.1% | 0.0% | 0.6% | 0.6% |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.2% | 0.0% | 0.3% | 0.5% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.5% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% | 0.0% | 0.4% | 0.5% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.0% | 0.0% | 0.4% | 0.6% |
| Class F | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 79% |  |  | 105% |  |  | 99% |  |  |
| Dec Time[%] | 79% |  |  | 92% |  |  | 93% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.7% | 0.6% | 0.0% | 0.2% | 0.4% |
| Class C | 0.0% | 0.0% | 0.3% | 0.0% | 0.4% | 0.3% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.7% | 0.4% |
| Class E | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.2% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | -0.1% | 0.1% | -0.2% | -0.1% | -0.4% | -0.2% |
| Enc Time[%] | 80% |  |  | 85% |  |  |
| Dec Time[%] | 76% |  |  | 75% |  |  |

# Conclusions

This contribution proposes to reduce the number of context used for last coefficient position coding of chroma TU. Two specific proposals were given and their simulation results were reported. The second proposal reduces the number of contexts from 12 to 3, or by 75%, with only minor loss in coding efficiency. Therefore, it is recommended to adopt the proposed context reduction method into the next HEVC WD.

# Draft Text

9.2.1.1 Initialisation process for context variables

Table 5-1 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 71 | 88 | 89 | 59 | 73 | 86 | 89 | 106 | 60 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 59 | 43 | 55 | 54 | 70 | 53 | 53 | 87 | 71 | 69 | 54 | 88 | 73 | 72 | 53 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| initValue | 57 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 102 | 103 | 73 | 89 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 73 | 57 | 87 | 54 | 70 | 54 | 101 | 71 | 55 | 70 | 116 | 103 | 72 | 72 | 119 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 118 | 103 | 73 | 89 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| initValue | 73 | 57 | 87 | 54 | 70 | 69 | 85 | 71 | 55 | 70 | 85 | 103 | 72 | 72 | 119 |

Table 5-2 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coeff_y_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 71 | 88 | 89 | 59 | 73 | 86 | 89 | 106 | 60 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 59 | 43 | 55 | 54 | 70 | 53 | 53 | 87 | 71 | 69 | 54 | 88 | 73 | 72 | 53 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| initValue | 57 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 102 | 103 | 73 | 89 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 73 | 57 | 87 | 54 | 70 | 54 | 101 | 71 | 55 | 70 | 116 | 103 | 72 | 72 | 119 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 118 | 103 | 73 | 89 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| initValue | 73 | 57 | 87 | 54 | 70 | 69 | 85 | 71 | 55 | 70 | 85 | 103 | 72 | 72 | 119 |

9.2.2 Binarization process

| Table 5-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| alf_cu_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| split_coding_unit_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| skip_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| cu_qp_delta | I | as specified in subclause Error: Reference source not found | 2 | Error: Reference source not found | 0 |
|  | P |  | 2 | Error: Reference source not found | 3 |
|  | B |  | 2 | Error: Reference source not found | 6 |
| pred_mode_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| part_mode | I | as specified in subclause Error: Reference source not found | 0 | Error: Reference source not found | 2 |
|  | P |  | 3 | Error: Reference source not found | 3 |
|  | B |  | 3 | Error: Reference source not found | 6 |
| pcm_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| mpm_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | all | as specified in subclause Error: Reference source not found | na | na | na, (uses Decode Bypass) |
| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| merge_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| merge_idx | P | TU, cMax = MaxNumMergeCand − 1 | 3 | Error: Reference source not found | 0 |
|  | B |  | 3 | Error: Reference source not found | 4 |
| inter_pred_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| ref_idx_lc | B | TU, cMax = num_ref_idx_lC_active_minus1 | 2 | Error: Reference source not found | 3 |
| ref_idx_l0 | P | TU, cMax = num_ref_idx_l0_active_minus1 | 2 | Error: Reference source not found | 0 |
|  | B |  | 2 | Error: Reference source not found | 3 |
| ref_idx_l1 | B | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Error: Reference source not found | 3 |
| abs_mvd_greater0_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| abs_mvd_greater1_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| abs_mvd_minus2[ ] | P/B | EG1 | na | na | na, (uses Decode Bypass) |
| mvd_sign_flag[ ] | P/B | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| mvp_lc_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| mvp_l0_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| mvp_l1_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| no_residual_data_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| split_transform_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 |
|  | B |  | 0 | Error: Reference source not found | 8 |
| cbf_luma | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 4 |
| cbf_cb, cbf_cr | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| last_significant_coeff_x_prefix | I | TU, cMax = ( log2TrafoWidth << 1 ) − 1 | 8 | Error: Reference source not found | 0 |
|  | P |  | 8 | Error: Reference source not found | 30 |
|  | B |  | 8 | Error: Reference source not found | 60 |
| last_significant_coeff_y_prefix | I | TU, cMax = ( log2TrafoHeight << 1 ) − 1 | 8 |  | 0 |
|  | P |  | 8 |  | 30 |
|  | B |  | 8 |  | 60 |
| last_significant_coeff_x_suffix | all | FL, cMax = (last_significant_coeff_x_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| last_significant_coeff_y_suffix | all | FL, cMax = (last_significant_coeff_y_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| significant_coeff_group_flag | I | FL, cMax = 1 | 0 | Table 9-26 | 0 |
|  | P |  | 0 | Table 9-26 | 4 |
|  | B |  | 0 | Table 9-26 | 8 |
| significant_coeff_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 48 |
|  | B |  | 0 | Error: Reference source not found | 96 |
| coeff_abs_level_greater1_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 32 |
|  | B |  | 0 | Error: Reference source not found | 64 |
| coeff_abs_level_greater2_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 24 |
|  | B |  | 0 | Error: Reference source not found | 48 |
| coeff_abs_level_minus3 | all | prefix and suffix as specified in subclause Error: Reference source not found | prefix: na suffix: na | prefix: na suffix: na | prefix: na, (uses Decode Bypass) suffix: na, (uses Decode Bypass) |
| coeff_sign_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |

##### 9.2.3.1.1.3. Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the color component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

Table 5-4 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 10 | 10 | 11 | 11 | 12 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 | 17 |

The vector lastCtx[ i ] is specified in Table5-4 and ctxIdxInc is derived as follows.

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth – 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-1)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight – 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-2)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  18		(9-3)

Changes For The Second Proposal!!

##### 9.2.3.1.1.3. Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

The vector lastCtx[ i ] is specified in Table5-4 and ctxIdxInc is derived as follows.

For Luma (cIdx == 0)

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth – 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-4)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight – 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-5)

For Chroma (cIdx >0)

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  binIdx >> (log2TrafoWidth – 2) + 18                              (9-6-chroma)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  binIdx >> (log2TrafoHeight – 2) + 18                              (9-3-chroma)

Note: The number 18 is the offset from luma context beginning position to the beginning position of chroma. If the total number of luma contexts used is changed, the offset will be changed accordingly.

# H0537 Luma Combined With H0514 Chroma Second Proposal Context Reduction Results and Working Draft

## Results

The combined H0537 (luma context reduction part) and H0514 (chroma context reduction) as described in section 6.2 were implemented and tested under common condition. Its simulation results are summarized in Table 3. Detailed results are included in the attachment (JCTVC-H0537-H0514.xls).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.6% | 0.5% | 0.1% | 0.2% | 0.2% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.6% | 0.6% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.4% | 0.3% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.4% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 113% |  |  |
| Dec Time[%] | 100% |  |  | 119% |  |  |

|  | Random Access HE |  |  |  |  | Random Access LC |  |  |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | Y |  | U | V |  | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% |  | -0.1% |  | 0.1% |  | 0.3% | 0.0% |  | 0.0% | 0.4% | 0.6% |
| Class B | 0.0% | 0.3% |  | 0.4% |  | 0.0% |  | 0.2% | 0.1% |  | 0.0% | 0.5% | 0.4% |
| Class C | 0.0% | 0.1% |  | 0.3% |  | 0.0% |  | 0.1% | 0.1% |  |  |  |  |
| Class D | 0.0% | 0.1% |  | 0.1% |  | 0.0% |  | 0.5% | 0.0% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% |  | 0.2% |  | 0.0% |  | 0.3% | 0.1% |  | 0.0% | 0.4% | 0.5% |
|  | 0.0% | 0.2% |  | 0.2% |  | 0.0% |  | 0.3% | 0.0% |  | 0.0% | 0.4% | 0.5% |
| Class F | 0.0% | 0.0% |  | 0.2% |  | 0.0% |  | 0.2% | 0.0% |  |  |  |  |
| Enc Time[%] | 72% |  |  |  |  | 98% |  |  |  |  | 86% |  |  |
| Dec Time[%] | 172% |  |  |  |  | 229% |  |  |  |  | 98% |  |  |
|  | Low delay B HE |  |  |  |  | Low delay B LC |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | Y | U |  |  | V |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% |  | 0.4% |  | 0.4% | 0.0% | 0.3% |  |  | 0.5% |  |  |  |
| Class C | 0.0% |  | 0.2% |  | 0.3% | 0.0% | 0.4% |  |  | 0.2% |  |  |  |
| Class D | 0.0% |  | 0.0% |  | -0.3% | 0.0% | 0.7% |  |  | -0.1% |  |  |  |
| Class E | 0.0% |  | 1.4% |  | 0.2% | 0.0% | -0.3% |  |  | -0.2% |  |  |  |
| Overall | 0.0% |  | 0.4% |  | 0.2% | 0.0% | 0.3% |  |  | 0.1% |  |  |  |
|  | 0.0% |  | 0.4% |  | 0.2% | 0.0% | 0.4% |  |  | 0.2% |  |  |  |
| Class F | -0.1% |  | 0.2% |  | 0.0% | -0.1% | -0.6% |  |  | 0.0% |  |  |  |
| Enc Time[%] | 104% |  |  |  |  | 121% |  |  |  |  |  |  |  |
| Dec Time[%] | 206% |  |  |  |  | 273% |  |  |  |  |  |  |  |

Please note that timing results are unreliable since the simulations were conducted across multiple computing platforms.

## Draft Text

Table 9-5 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 71 | 88 | 89 | 59 | 73 | 86 | 89 | 106 | 60 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 59 | 43 | 55 | 54 | 70 | 53 | 53 | 87 | 71 | 69 | 54 | 88 | 73 | 72 | 53 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| initValue | 57 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 102 | 103 | 73 | 89 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 73 | 57 | 87 | 54 | 70 | 54 | 101 | 71 | 55 | 70 | 116 | 103 | 72 | 72 | 119 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 118 | 103 | 73 | 89 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| initValue | 73 | 57 | 87 | 54 | 70 | 69 | 85 | 71 | 55 | 70 | 85 | 103 | 72 | 72 | 119 |

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 9-6 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coeff_y_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 71 | 88 | 89 | 59 | 73 | 86 | 89 | 106 | 60 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 59 | 43 | 55 | 54 | 70 | 53 | 53 | 87 | 71 | 69 | 54 | 88 | 73 | 72 | 53 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| initValue | 57 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 102 | 103 | 73 | 89 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 73 | 57 | 87 | 54 | 70 | 54 | 101 | 71 | 55 | 70 | 116 | 103 | 72 | 72 | 119 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 102 | 88 | 73 | 73 | 72 | 118 | 103 | 73 | 89 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| initValue | 73 | 57 | 87 | 54 | 70 | 69 | 85 | 71 | 55 | 70 | 85 | 103 | 72 | 72 | 119 |

| Initialisation variable | last_significant_coefficient_y_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

| Table 9-7 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| alf_cu_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| split_coding_unit_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| skip_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| cu_qp_delta | I | as specified in subclause Error: Reference source not found | 2 | Error: Reference source not found | 0 |
|  | P |  | 2 | Error: Reference source not found | 3 |
|  | B |  | 2 | Error: Reference source not found | 6 |
| pred_mode_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| part_mode | I | as specified in subclause Error: Reference source not found | 0 | Error: Reference source not found | 2 |
|  | P |  | 3 | Error: Reference source not found | 3 |
|  | B |  | 3 | Error: Reference source not found | 6 |
| pcm_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| mpm_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | all | as specified in subclause Error: Reference source not found | na | na | na, (uses Decode Bypass) |
| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| merge_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| merge_idx | P | TU, cMax = MaxNumMergeCand − 1 | 3 | Error: Reference source not found | 0 |
|  | B |  | 3 | Error: Reference source not found | 4 |
| inter_pred_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| ref_idx_lc | B | TU, cMax = num_ref_idx_lC_active_minus1 | 2 | Error: Reference source not found | 3 |
| ref_idx_l0 | P | TU, cMax = num_ref_idx_l0_active_minus1 | 2 | Error: Reference source not found | 0 |
|  | B |  | 2 | Error: Reference source not found | 3 |
| ref_idx_l1 | B | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Error: Reference source not found | 3 |
| abs_mvd_greater0_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| abs_mvd_greater1_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 3 |
| abs_mvd_minus2[ ] | P/B | EG1 | na | na | na, (uses Decode Bypass) |
| mvd_sign_flag[ ] | P/B | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| mvp_lc_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| mvp_l0_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| mvp_l1_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
| no_residual_data_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| split_transform_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 |
|  | B |  | 0 | Error: Reference source not found | 8 |
| cbf_luma | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 4 |
| cbf_cb, cbf_cr | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| last_significant_coeff_x_prefix | I | TU, cMax = ( log2TrafoWidth << 1 ) − 1 | 8 | Table 9-25 | 0 |
|  | P |  | 8 | Table 9-25 | 30 18 |
|  | B |  | 8 | Table 9-25 | 60 36 |
| last_significant_coeff_y_prefix | I | TU, cMax = ( log2TrafoHeight << 1 ) − 1 | 8 |  | 0 |
|  | P |  | 8 |  | 30 18 |
|  | B |  | 8 |  | 60 36 |
| last_significant_coeff_x_suffix | all | FL, cMax = (last_significant_coeff_x_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| last_significant_coeff_y_suffix | all | FL, cMax = (last_significant_coeff_y_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| significant_coeff_group_flag | I | FL, cMax = 1 | 0 | Table 9-26 | 0 |
|  | P |  | 0 | Table 9-26 | 4 |
|  | B |  | 0 | Table 9-26 | 8 |
| significant_coeff_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 48 |
|  | B |  | 0 | Error: Reference source not found | 96 |
| coeff_abs_level_greater1_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 32 |
|  | B |  | 0 | Error: Reference source not found | 64 |
| coeff_abs_level_greater2_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 24 |
|  | B |  | 0 | Error: Reference source not found | 48 |
| coeff_abs_level_minus3 | all | prefix and suffix as specified in subclause Error: Reference source not found | prefix: na suffix: na | prefix: na suffix: na | prefix: na, (uses Decode Bypass) suffix: na, (uses Decode Bypass) |
| coeff_sign_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |

| Table 9-8 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 |  | >=4 |
| alf_cu_flag | Error: Reference source not found | 0 | 0 | na | na | na |  | na |
|  |  | 1 | 0 | na | na | na |  | na |
|  |  | 2 | 0 | na | na | na |  | na |
| split_coding_unit_flag | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 3 | 0,1,2 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 6 | 0,1,2 (subclause Error: Reference source not found) | na | na | na |  | na |
| skip_flag | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 3 | 0,1,2 (subclause Error: Reference source not found) | na | na | na |  | na |
| cu_qp_delta | Error: Reference source not found | 0 | 0 | na (uses Decode Bypass) | 1 | 2 |  | 2 |
|  |  | 3 | 0 | na (uses Decode Bypass) | 1 | 2 |  | 2 |
|  |  | 6 | 0 | na (uses Decode Bypass) | 1 | 2 |  | 2 |
| pred_mode_flag | Error: Reference source not found | 0 | 0 | na | na | na |  | na |
|  |  | 1 | 0 | na | na | na |  | na |
| part_mode | Error: Reference source not found | 2 | 0 | na | na | na |  | na |
|  |  | 3 | 0 | 1 | 0 | 1 |  | na |
|  |  | 6 | 0 | 1 | 0 | 1 |  | na |
| part_mode (cLog2CUSize = = Log2MinCUSize && cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag) | Error: Reference source not found | 3 | 0 | 1 | 2 | na |  | na |
|  |  | 6 | 0 | 1 | 2 | na |  | na |
| prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 0 | na | na | na |  | na |
|  |  | 1 | 0 | na | na | na |  | na |
|  |  | 2 | 0 | na | na | na |  | na |
| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 |  | na |
|  |  | 2 | 0 | 1 | 1 | 1 |  | na |
|  |  | 4 | 0 | 1 | 1 | 1 |  | na |
| merge_flag | Error: Reference source not found | 0 | 0 | na | na | na |  | na |
|  |  | 1 | 0 | na | na | na |  | na |
| merge_idx | Error: Reference source not found | 0 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 |  | na |
|  |  | 4 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 |  | na |
| inter_pred_flag | Error: Reference source not found | 0 | cuDepth | na | na | na |  | na |
| ref_idx_l0 | Error: Reference source not found | 0 | 0 | 1 | 2 | 2 |  | 2 |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Error: Reference source not found | 3 | 0 | 1 | 2 | 2 |  | 2 |
| abs_mvd_greater0_flag[ ] | Error: Reference source not found | 0 | 0 | na | na | na |  | na |
|  |  | 1 | 0 | na | na | na |  | na |
| abs_mvd_greater1_flag[ ] | Error: Reference source not found | 2 | 0 | na | na | na |  | na |
|  |  | 3 | 0 | na | na | na |  | na |
| mvp_l0_flag | Error: Reference source not found | 0 | 0 | na | na | na |  | na |
| mvp_l0_flag, mvp_l1_flag, mvp_lc_flag | Error: Reference source not found | 1 | 0 | na | na | na |  | na |
| no_residual_data_flag | Error: Reference source not found | 0 | 0 | na | na | na |  | na |
|  |  | 1 | 0 | na | na | na |  | na |
| split_transform_flag | Error: Reference source not found | 0 | cuDepth + trafoDepth | na | na | na |  | na |
|  |  | 4 | cuDepth + trafoDepth | na | na | na |  | na |
|  |  | 8 | cuDepth + trafoDepth | na | na | na |  | na |
| cbf_luma | Error: Reference source not found | 0 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na |  | na |
|  |  | 2 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na |  | na |
|  |  | 4 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na |  | na |
| cbf_cb, cbf_cr | Error: Reference source not found | 0 | trafoDepth | na | na | na |  | na |
|  |  | 3 | trafoDepth | na | na | na |  | na |
|  |  | 6 | trafoDepth | na | na | na |  | na |
| last_significant_coeff_x_prefix | Table 9-25 | 0 | 0..29 0-17 (subclause ) |  |  |  |  |  |
|  |  | 30 18 | 0..290-17 (subclause ) |  |  |  |  |  |
|  |  | 60 36 | 0..290-17 (subclause ) |  |  |  |  |  |
| last_significant_coeff_y_prefix |  | 0 | 0..290-17 (subclause ) |  |  |  |  |  |
|  |  | 30 18 | 0..290-17 (subclause ) |  |  |  |  |  |
|  |  | 60 36 | 0..290-17 (subclause ) |  |  |  |  |  |
| significant_coeff_group_flag | Table 9-26 | 0 | 0..3 (subclause Error: Reference source not found) |  |  |  |  |  |
|  |  | 4 | 0..3 (subclause Error: Reference source not found) |  |  |  |  |  |
|  |  | 8 | 0..3 (subclause Error: Reference source not found) |  |  |  |  |  |
| significant_coeff_flag | Error: Reference source not found | 0 | 0..47 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 48 | 48..95 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 96 | 96..143 (subclause Error: Reference source not found) | na | na | na |  | na |
| coeff_abs_level_greater1_flag | Error: Reference source not found | 0 | 0..31 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 32 | 0..63 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 64 | 0..95 (subclause Error: Reference source not found) | na | na | na |  | na |
| coeff_abs_level_greater2_flag | Error: Reference source not found | 0 | 0..23 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 24 | 0..47 (subclause Error: Reference source not found) | na | na | na |  | na |
|  |  | 48 | 0..71 (subclause Error: Reference source not found) | na | na | na |  | na |

###### 9.2.3.1.1.3.  Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the color component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

Table 9-9 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 10 | 10 | 11 | 11 | 12 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 | 17 |

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| I | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 | 2 |

The vector lastCtx[ i ] is specified in Error: Reference source not found and ctxIdxInc is derived as follows.

When cIdx is equal to 0,

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth – 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-7)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight – 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-8)

When cIdx is greater than 0, ctxIdxInc is derived as follows

ctxIdxInc =  ctxIdxInc  +  18		(9-9)

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth – 2 ) )+ 15

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoHeight – 2 )) + 15

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 9,” JCTVC-G1103.

[2] L. Guo, W.-J. Chien, X. Wang, M. Karczewicz (Qualcomm), “CABAC Contexts Reduction for Last Position Coding, JCTVC-H0537”.

# Patent Rights Declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).