| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0516-r3 |
| --- | --- |

| Title: | On MPM determination and Planar mode signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2251 yyu@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document describes implementation details and reports the results of Motorola Mobility’s MPM determination and Planar mode signaling. The proposed method separates Planar mode coding from angular prediction mode coding so that Planar mode processing can start earlier and eliminates the needs to perform MPM determination when Planar mode is used.

# Introduction

In the current HEVC WD [1], the Luma component of a current PU in intra CU employs up to 35 prediction modes; Planar prediction, DC prediction, and 33 angular direction predictions. The selected prediction mode is coded as follows. First, a flag is coded, indicating determining whether the most probable mode (MPM) is used or not. If MPM is used, another flag is coded, indicating used to determine whether mode of the PU’s left neighbor or the mode of the PU’s right above neighbor is used as MPM., Otherwise, if MPM is not used, the selected remaining modes are coded. For example, a pseudocode for encoder of [1] is listed below.

if (MPM)

{

codebit (MPM);

if (Left)  // Planar or DC if unavailable

codebit (Left);

else if (Top)  // Planar or DC if unavailable

codebit (Top);

}

else ()  // !MPM; i.e., code remaining modes

{

codebit (!MPM);

codebit (remModes);

}

To ensure that there are 2 available MPM for a PU is determined from two possible modes. The two possible modes can be the modes of the left and the above neighbors of the PU. MPM is determined using from the mode of its left neighbor and its right above neighbor. If either (or both) of the left or the above neighbor is unavailable, then Planar mode will be included in the two possible modes for MPM. If none both of the left and the above neighbors are available, Planar and DC mode will be the two possible modes for MPM.

In Geneva meeting, [2] was proposed to modify intra prediction mode coding. It proposes to code Planar mode flag first when MPM is not used. One benefit of this method is to remove Planar mode from the remaining modes. Hence, up to 3 modes can be removed.

# Proposal

It has been observed that Planar mode is the most often selected mode statistically. Logically, the most often selected mode should use the least number of bits as compared to other possible modes. This contribution proposes to code Planar mode with less overhead and lower complexity. Specifically, we propose to code a flag to indicate if Planar mode is used first, even before MPM mode. In this way, Planar mode is coded with only 1 bit. In addition, this approach removes 3 modes from the remaining modes. Pseudocode for encoder of the proposed method is listed below.

if (Planar)

{

codebit (Planar);

}

else ()  // !Planar

{

codebit (!Planar);

if (MPM)

{

codebit (MPM);

if (Left)  // DC or a predetermined mode if unavailable

codebit (Left);

else if (Top)  // DC or a predetermined mode if unavailable

codebit (Top);

}

else () // !MPM and !Planar

{

codebit (!MPM);

codebit (remModes);

}

}

We also further proposed to modify MPM mode determination process of two possible modes for MPM. Given a PU, if one or both of the left and the above neighbors is not available, we fill in the missing mode using pre-determined modes. For example, we may fill in the first missing mode using DC mode, and the second missing mode using horizontal or vertical prediction mode. In case that one or both of the left and the above neighbors is Planar mode, we will treat the neighbor(s) with Planar mode as unavailable.

In order to improve coding efficiency, we propose to use a mapping table as shown in Table 1 to determine mode number of neighbor PU, which is larger than the total number of prediction modes allowed for the current PU, instead of using pre-determined modes as described earlier. For example, neighbor PU of size 8x8 with mode number 22 will be assigned mode number 1 for MPM determination purpose when current PU is of size 4x4.

Table 1. Mapping table when mode number of neighbor PU is larger than the total number of prediction modes allowed for the current PU

| Old mode | New mode |
| --- | --- |
| 19 | 11 |
| 20 | 5 |
| 21 | 12 |
| 22 | 1 |
| 23 | 13 |
| 24 | 6 |
| 25 | 14 |
| 26 | 7 |
| 27 | 15 |
| 28 | 8 |
| 29 | 16 |
| 30 | 2 |
| 31 | 17 |
| 32 | 9 |
| 33 | 18 |
| 34 | 10 |

10

27

11

28

12

29

13

30

14

31

15

32

16

33

34

17

18

1:

Intra_DC

0:

Intra_Planar

26

9

25

8

24

7

23

6

22

5

21

4

20

3

19

2

Fig. 1 Proposed intra mode numbering

Alternatively, coding efficiency can be improved by assigning the mode of the adjacent angle to the mode of the available MPM candidate to mode of MPM candidate that is unavailable to be. To avoid extra mapping tables, we propose a modified intra mode numbering as shown in Fig. 1. In this figure, similar mode numbers are assigned to the proximity of the angular prediction directions and the 18 smallest mode numbers for the allowed angular prediction directions of the PU of 4x4 are kept. Using Fig.1, mode of the MPM candidate that is unavailable is determined by adding offset to the mode of the MPM candidate that is available. The offset can be set as the following 3 rules:

Rule 1, 19->19 (4x4): delta can be either 1 or -1. For example, if the current PU is of size 4x4 and the available PU (of size 4x4) uses mode number 5, then mode number 4 or 6 is assigned to the unavailable PU.

Rule 2A, 35->35 (8x8, 16x16, 32x32): if mode is less than 19, delta can be either 1 or -1. For example, if the current PU is of size 8x8 and the available PU (of size 16x16) uses mode number 5, then mode number 4 or 6 is assigned to the unavailable PU.

Rule 2B, 35->35 (8x8, 16x16, 32x32): if mode is larger than 18, delta can be either 1 or -1. For example, if the current PU is of size 8x8 and the available PU (of size 16x16) uses mode number 32, then mode number 31 or 33 is assigned to the unavailable PU.

Rule 3A, 35->19: if mode is less than 19, delta can be either 1 or -1. For example, if the current PU is of size 4x4 and the available PU (of size 16x16) uses mode number 5, then mode number 4 or 6 is assigned to the unavailable PU.

Rule 3B, 35->19: if mode is larger than 18, delta can be either -16 or -17. For example, if the current PU is of size 4x4 and the available PU (of size 16x16) uses mode number 32, then mode number 15 or 16 is assigned to the unavailable PU.

The proposed modifications reduce complexity by treating Planar mode separately from other angular prediction modes. This way, usage of Planar mode can be determined faster with only 1 bit parsing, instead of two. Moreover, there is no need to perform MPM determination when Planar is used. The proposed method may also improve coding efficiency since currently there are two possible ways to signal planar mode; i.e., through MPM branch and through remaining modes branch. By using pre-determined modes, instead of Planar mode, to fill the missing mode in the MPM determination process, this redundancy can be eliminated.

# Simulation Results

The proposed method was implemented on top of HM-5.0. In the experiments, vertical (mode number 1) and horizontal (mode number 2) are used as the first two pre-determined replacement modes. The test configurations as specified by the common test conditions are followed [2]. Tests were conducted on the AIHE and AILC configurations. The results of the proposed method without mapping table are shown in Table 2 and the results of the proposed method with the mapping table 1 are shown in Table 3. Please note that the timing results are inaccurate since the simulations were performed across multiple PC configurations.

Table 2. Simulation results of using Planar mode flag and pre-determined replacement modes

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | 0.1% | 0.1% | -0.2% | 0.0% | 0.2% |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class E | -0.3% | -0.3% | -0.3% | -0.3% | 0.0% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class F | -0.2% | -0.5% | -0.3% | -0.2% | -0.4% | -0.4% |
| Enc Time[%] | 68% |  |  | 259% |  |  |
| Dec Time[%] | 86% |  |  | 410% |  |  |

Table 3. Simulation results of using Planar mode flag, pre-determined replacement modes, and mapping Table 1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | 0.0% | 0.1% | -0.2% | 0.0% | 0.2% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class E | -0.3% | -0.3% | -0.3% | -0.3% | 0.0% | -0.1% |
| Overall | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
|  | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
| Class F | -0.2% | -0.4% | -0.3% | -0.2% | -0.5% | -0.5% |
| Enc Time[%] | 68% |  |  | 117% |  |  |
| Dec Time[%] | 86% |  |  | 112% |  |  |

# Conclusions

This contribution proposes to code Planar mode first for intra PU. This proposed modification reduces complexity for Planar mode operation since there is no need to perform MPM determination when Planar mode is used. Simulation results of the proposed method with a mapping Table shows a minor gain of the proposed method in intra coding condition. Hence, it is recommended to adopt this modification to the next HEVC WD.

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” JCTVC-G1103.

[2] C. Yeo, H. L. Tan, Y. H. Tan, Z. Li, “Non CE-6: On Intra Prediction Mode Coding,” JCT-VC document, JCTVC-G153, Nov. 2011.

# Patent Rights Declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).