| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0518 |
| --- | --- |

| Title: | CE4: cross check report for subtest 3.2 (JCTVC-H0260) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Krit Panusopone 6420 Sequence Drive San Diego, CA 92121 | Tel: Email: | +1-858-404-3618 krit@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution presents a summary of a cross-check performed by Motorola Mobility on Samsung’s contribution (JCTVC-H0260) for subtest 3.2 of CE4. The cross-check performance results reportedly match those provided by Samsung.

# Introduction

In JCTVC-H0260 [1], Samsung proposed implicit signalling of QP offset based on TU size. Under assumptions that static region has a higher chance of being used as reference and the static region is more likely to use large TU size, relatively smaller QP value should be assigned to the larger TU and vice versa.

# Simulation results

The test configurations as specified by the common test conditions are followed [2]. Tests were conducted on the RAHE, RALC, RAHE-10, LBHE, LBLC configurations. The results are shown below:

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.4% | 0.3% | 0.0% | -0.3% | -0.4% | -1.0% | 0.1% | 1.9% | 1.7% |
| Class B | 0.1% | 0.5% | 0.3% | 0.4% | -0.3% | -0.9% | 0.1% | 0.4% | 0.3% |
| Class C | -0.3% | -0.2% | 0.0% | -0.1% | -1.1% | -1.3% |  |  |  |
| Class D | -0.1% | -0.4% | -0.4% | 0.3% | -1.7% | -1.9% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | 0.1% | -0.9% | -1.3% | 0.1% | 1.1% | 0.9% |
|  | -0.1% | 0.1% | 0.1% | 0.1% | -0.9% | -1.3% | 0.3% | 1.3% | 1.1% |
| Class F | -0.4% | -0.6% | -0.6% | -0.1% | -1.1% | -1.1% |  |  |  |
| Enc Time[%] | 72% |  |  | 95% |  |  | 99% |  |  |
| Dec Time[%] | 198% |  |  | 268% |  |  | 104% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.6% | 1.2% | 0.3% | 0.7% | 0.9% |
| Class C | -0.5% | -0.3% | -0.1% | -0.6% | -0.8% | -1.0% |
| Class D | -0.5% | -0.2% | 0.1% | -0.1% | -0.3% | -0.3% |
| Class E | 0.0% | 0.1% | 0.1% | 0.2% | -0.6% | 0.2% |
| Overall | -0.2% | 0.1% | 0.4% | 0.0% | -0.2% | 0.0% |
|  | -0.3% | 0.2% | 0.6% | 0.0% | 0.0% | 0.0% |
| Class F | -0.6% | -1.3% | -1.3% | 0.3% | -1.2% | -1.7% |
| Enc Time[%] | 102% |  |  | 122% |  |  |
| Dec Time[%] | 261% |  |  | 316% |  |  |

Please note that the timing results are unreliable since the simulations were performed across multiple PC configurations.

# Conclusions

In this contribution, the cross-verified results on Samsung’s proposal regarding implicit signalling of QP offset based on TU size are reported. It is confirmed that the results perfectly match with the ones provided by Samsung.

# References

[1] C. Kim, Y. Park, K. Choi, J. Kim, J. Park, “CE4 : 3.2 Implicit signalling of QP offset based on TU size,” JCTVC-H0260, San Jose, USA, Feb. 2012.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, Switzerland, Nov. 2011.