| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-7 Jan, 2012 | Document: JCTVC-H0522 WG11 Number: m23403 |
| --- | --- |

| Title: | Weighted predictive coding for quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianhua Zheng, HiSilicon Jianwen Chen, UCLA Q20, Shi Chuang Ke Ji Shi Fan Yuan No.156 Beiqing Rd.,Hai-Dian District Beijing 100095, P.R. China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies University of California, Los Angeles |  |  |

_____________________________

# Abstract

It has been agreed to use AVC-like approach as start point to compress the quantization matrices. In the current HEVC design, quantization matrices can go up to 32x32. AVC-like approach is lack useful quantization matrices prediction tools in inner frame matrix prediction and cross-frame matrix prediction. In practice application, the whole set of matrices may be loaded when scene cut occurs or in scene changes. It is also possible to change one or more sizes of quantization matrices within one scene. A weighted predictive method is investigated this proposal for quantization matrices coding to reduce the generated matrices bits in bitstream. Compared with the AVC quantization matrix compression method, the proposed method can achieved 3.87x compression ratio in lossless quantization matrix coding.

# Description

In the current HEVC design the transform and quantization block sizes can go up to 32x32. The large block transforms can provide improved coding efficiency, but also lead to larger overhead for carrying the perceptual quantization matrices in the picture. In practice application, quantization matrices may update partially. If large block size quantization matrices update, it is still a larger bit overhead to the video data, especially for small frame size sequences. Since the matrices in neighboring frame may have similar characteristic, cross-frame prediction can be used to reduce the matrices bits. Weighted predictive method can improve the cross-frame prediction precision.

## Predictive Coding for Quantization Matrix

Based on the fact that the content in one video stream is always continuously changing, using one set of quantization matrices cannot efficiently adapt to the changes and cannot achieve the optimal performance. The quantization matrices should be updated adaptively within one sequence. Since the contents in temporally adjacent frames are similar, the correlation among the frames are already used in the conventional coding scheme, such as P frame coding and B frame coding. For these similar video frames, the differences among the quantization matrices should be very small. Using the temporally adjacent quantization matrices as references, the quantization matrices in current frame can be efficiently compressed. Based on this concept, the quantization matrix predictive coding is proposed.

### Encoding and Decoding

The diagrams of decoder and encoder for the predictive based quantization matrix compression are illustrated in Figure 1 and Figure 2 separately. In the encoder, the matrix coefficients are predicted firstly.

Figure 1: Block diagram of decoder

Figure 2: Block diagram of encoder

We propose to use weighted predictive methods for QM prediction as described in the above diagrams. The quantization matrices that have been previously encoded are used to predict quantization matrices in current frame. The quantization matrix prediction method can be formulated as following, two reference QMs assumed in the example.

QMcp  =αQMp1 + βQMp2

In which QMcp is the predicted quantization matrix of current QM, QMp1 and QMp2 are two referenced QMs.  α, β are the weighting factors of the references.

If multiple QMs are used as references, the above formulation can be extended to

QMcp  = α1QMp1 +α2QMp2 + ….. +  αnQMpn

This formulation can be explained in the following difference cases.

- QM forward prediction:  If all reference QMs are from the previous frames, e.g. QMp1 and QMp2 are both from the previous frames;

- QM backward prediction:  If all reference QMs are from the subsequent frames, e.g. QMp1 and QMp2 are both after the current frames;

- QM bi-direction prediction:  For reference QMs in prediction, if some of reference QMs are obtained from the previous frames and others reference QMs are obtained from the subsequent frame, e.g. QMp1 is from previous frame and QMp2 is from subsequent frame.

The QM forward prediction, backward prediction and bi-direction prediction are illustrated in Fig.3 (a)~Fig.3(c) separately.

(a)                      (b)                              (c)

Figure 3

The reference QMs can be obtained from temporal neighboring frames; the weighting factors of the reference QMs in reference frames can be determined by the frame distance between the current frame and the references. Apply the weighting factor to the reference QMs to generate reference matrices for improving the prediction precision of current frame. The weighting factors are hardly coded in bitstream.

## Test software

The simulation software is implemented on the top of HM5 QM branch software. To simulate different partial updating scheme, we add QM_FRAME_UPDATE macro in software. Set QM_FRAME_UPDATE to 1 to enable the partial updating scheme. The update interval is determined in encoder configure file.

#define SCALING_LIST                          1 //JCTVC-G880/JCTVC-G1016

#define SCALING_LIST_OUTPUT_RESULT     1 //JCTVC-G880/JCTVC-G1016

#define QM_FRAME_UPDATE                   1 //scaling list updates enable

#define QM_WEIGHTING                         0 //using Sony’s compression method

1 //using proposed weighting QM encoding method

Here is an example to set updating every 5 frames in encoder configure file.

UpdateInterval                : 5                      # ScalingList update every UpdateInteval frame

In this implementation, if updating occurs, only one size of quantization matrices need be changed.

The experiments of this contribution will be based on the proposed testing platform.

# Experimental Results

The experiments use 8 sets of quantization matrices available in CE4 test condition, in which 4 cases are of symmetry matrices and 4 cases are of asymmetry matrices.

Since in lossless coding, the PSNR are not changed, the BD rate can be used in evaluated the performance of this tool.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
| Class B | -1.4% | -1.2% | -1.2% | -1.1% | -1.0% | -1.0% |
| Class C | -3.6% | -3.3% | -3.3% | -3.1% | -2.9% | -2.9% |
| Class D | -9.4% | -8.7% | -8.8% | -8.6% | -8.0% | -8.0% |
| Class E | -4.5% | -4.0% | -4.0% | -3.6% | -3.4% | -3.4% |
| Overall | -4.1% | -3.7% | -3.7% | -3.6% | -3.3% | -3.3% |
|  | -4.1% | -3.8% | -3.8% | -3.6% | -3.3% | -3.3% |
| Class F | -2.9% | -2.7% | -2.7% | -2.6% | -2.4% | -2.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -4.2% | -3.8% | -3.8% | -4.0% | -3.5% | -3.5% |
| Class B | -10.5% | -9.3% | -8.9% | -10.1% | -8.7% | -8.5% |
| Class C | -19.9% | -18.5% | -18.5% | -19.8% | -18.3% | -18.4% |
| Class D | -37.5% | -35.7% | -35.8% | -37.9% | -36.1% | -36.3% |
| Class E |  |  |  |  |  |  |
| Overall | -19.4% | -18.1% | -18.0% | -19.3% | -17.9% | -17.9% |
|  | -19.4% | -18.1% | -18.1% | -19.3% | -17.9% | -18.0% |
| Class F | -18.9% | -17.7% | -17.8% | -18.5% | -17.2% | -17.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 109% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -11.0% | -9.5% | -9.2% | -11.0% | -9.3% | -9.0% |
| Class C | -19.7% | -18.2% | -18.2% | -19.8% | -18.1% | -18.1% |
| Class D | -36.7% | -34.9% | -35.0% | -37.2% | -35.6% | -35.7% |
| Class E | -41.1% | -38.9% | -38.5% | -40.9% | -38.5% | -38.6% |
| Overall | -25.2% | -23.5% | -23.4% | -25.4% | -23.5% | -23.5% |
|  | -25.2% | -23.6% | -23.5% | -25.4% | -23.6% | -23.6% |
| Class F | -26.8% | -25.1% | -25.4% | -26.0% | -24.3% | -24.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 110% |  |  |

### Compression ratio against HM5-QM branch software

The compression ratio against current HM method is also investigated. This is conducted to evaluate the quantization matrices compression ratio improvement over than current HM5-QM method. The compression ratio against HM5-QM is estimated by the ratio of HM5-QM generated matrix bits and those of the proposed method.

|  | proposed method |
| --- | --- |
|  | Average compression ratio against HM5-QM |
| Intra_LC | 3.87 |
| Intra_HE | 3.87 |
| RA_LC | 3.87 |
| RA_HE | 3.87 |
| LB_LC | 3.87 |
| LB_HE | 3.87 |
| LP_LC | 3.87 |
| LP_HE | 3.87 |

# Conclusion

The proposed weighted predictive method for quantization matrices prediction and coding achieved 3.87x improvement comparing to current method in HM5-QM. It is proposed to include cross-frame QM prediction and weighted predictive method into HM design to improve the QM compression performance.

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies, UCLA may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).