| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0524 |
| --- | --- |

| Title: | Extending 4×4 sub-block diagonal scan to 8×8 TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Chulkeun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea Jianle Chen, Joel Sole, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com +1-858-651-8028 cjianle@qualcomm.com |
| Source: | LG Electronics, Qualcomm Inc. |  |  |

_____________________________

# Abstract

In current HM, sub-block based diagonal scanning is applied for coefficients coding in 16×16 and 32×32 TU [1]. This contribution proposes to extend 4×4 sub-block based diagonal scan to 8x8 TU for unification. Experiment results show no significant coding efficiency impact (0.1% gain in LB_LC, and 0.0% in other configuration) with the proposed method.

# Introduction

In current HM, 4×4 sub-block based scanning is applied for for coefficients coding in 16×16 and 32×32 TU. The scanning order of sub-block in the TU also follows the diagonal scanning pattern. Each sub-block is treated as a subset to signal coefficient level information. However 8x8 TU still applied the diagonal scan as show in Fig. 1, and 64 coefficients are divided into 4 subsets according to the diagonal scan order through the entire 8x8 TU.

Fig. 1 The diagonal scanning in 8×8 TU

# Proposed Method

This contribution proposes to apply 4x4 sub-block based scanning to 8x8 TU with diagonal scanning order to keep the consistency and to enhance parallelization. Fig. 1 shows the proposed scanning order applied 8x8 blocks. As done in 16×16 and 32×32 TU, a groupCoeffFlag syntax is signaled for a 4x4 sub-block in 8x8 TU when the sub-block based diagonal scan is applied.

Fig. 2 4x4 sub-block based diagonal scanning for 8×8 TU

# Results

The proposed method was implemented in HM5.0 and tested under the common condidtion [1]. Experimental results are shown in Tab.1.

Tab. 1 The BD rate of the proposed method against HM5.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.4% | -0.1% | -0.1% | 0.5% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | 0.0% | 0.0% | 0.2% | -0.1% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.2% | -0.1% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | 0.0% | 0.1% | -0.1% | 0.2% | 0.1% |
| Class D | -0.1% | -0.2% | -0.2% | -0.1% | 0.8% | -0.2% |
| Class E | -0.1% | 0.9% | -0.7% | 0.0% | -1.3% | 0.1% |
| Overall | 0.0% | 0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
|  | 0.0% | 0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class F | -0.2% | -0.1% | -0.2% | 0.0% | -0.1% | 0.1% |

# Conclusions

This contribution proposes to apply 4×4 sub-block based diagonal scan to 8x8 TU to keep the consistency on TU with variant size and enhance parallelization. Experiment results show no significant coding efficiency impact (0.1% gain in LB_LC, and 0.0% in other configuration) with the proposed method. We recommend adopting proposed method in next version of HM.

# References

- J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual”, JCTVC-G323, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

# Text for Working Draft

# Patent rights declaration

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).