| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0525 |
| --- | --- |

| Title: | Non-CE4: Cross-verification report for spatial QP prediction based on motion vector predictors (JCTVC-H0289) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a study and cross-verification results of spatial QP prediction methods based on motion vector predictors, which are presented in JCTVC-H0289. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-H0289 and results exactly match those presented in JCTVC-H0289.

# Analysis

Three methods are presented in JCTVC-H0289 [1]. First two (test 1 and test 2) are a spatial QP prediction method based on motion vector prediction. A block from which motion vector predictor of the current block is derived is likely to belong to the same object and to have similar perceptual sensitivities, in the same way as in the case of intra prediction [2]. In that sense, AMVP-based QP prediction presented in [1] is logical. The third method is a simple non-mode-dependent average prediction, which can be regarded as a modified version of CE4 test 2.3.e at the 6th JCT-VC meeting [3], where only above and left QGCUs are employed and it is prohibited to refer outside of LCU.

Software was provided by the proponent and it was successfully compiled by Microsoft Visual C++ 2008. It was confirmed that the software implementation conforms to the description in JCTVC-H0289. The implementation is simple and straightforward, and there have been no problems found in the software.

# Simulation results

The experiment to verify simulation results provided by the proponent was conducted under the CE4 Subtest 1 test conditions described in JCTVC-G1204 [4]. The computing platform used for the verification is shown below. MinCUDQPSize was set to 8x8, which is the mandatory condition in CE4 Subtest1.

| Computing platform | Usage |
| --- | --- |
| OS: Windows 7 64-bit Professional CPU: Xeon X5680 3.33GHz (12 cores) Memory: 32GiB | Up to 12 processes for encoding and decoding ran at the same time. |

Tables 1-3 show the summary of results. Attached spreadsheets contain the detailed results. It was confirmed that BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent.

Table 1: Summary results of test 1 of JCTVC-H0289 against test 1.1 of CE4 (anchor).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.2% | -4.1% | -0.1% | -0.1% | -0.2% | -3.9% |  |  |  |  |
| Class B | -0.3% | -0.3% | -0.3% | -7.2% | -0.2% | -0.2% | -0.2% | -7.3% |  |  |  |  |
| Class C | -0.4% | -0.4% | -0.4% | -9.6% | -0.4% | -0.4% | -0.4% | -9.8% |  |  |  |  |
| Class D | -0.4% | -0.4% | -0.4% | -10.2% | -0.4% | -0.3% | -0.3% | -10.3% |  |  |  |  |
| Class E | -0.4% | -0.4% | -0.4% | -6.7% | -0.4% | -0.4% | -0.4% | -7.8% |  |  |  |  |
| Overall | -0.4% | -0.3% | -0.3% | -8.0% | -0.3% | -0.3% | -0.3% | -8.2% |  |  |  |  |
|  | -0.4% | -0.3% | -0.3% |  | -0.3% | -0.3% | -0.3% |  |  |  |  |  |
| Class F | -0.1% | -0.1% | -0.2% | -4.5% | -0.1% | -0.1% | -0.1% | -4.5% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | 0.0% | -3.0% | -0.3% | 0.0% | -0.2% | -2.8% | -0.1% | 0.1% | 0.1% | -2.7% |
| Class B | -0.2% | -0.1% | -0.2% | -4.5% | -0.2% | -0.1% | -0.2% | -4.2% | -0.2% | -0.2% | -0.1% | -4.5% |
| Class C | -0.3% | -0.1% | -0.4% | -6.5% | -0.3% | -0.2% | 0.0% | -6.3% |  |  |  |  |
| Class D | -0.3% | -0.1% | -0.4% | -6.4% | -0.3% | -0.3% | -0.6% | -6.3% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.1% | -0.3% | -5.3% | -0.3% | -0.2% | -0.2% | -5.1% | -0.2% | -0.1% | 0.0% | -3.7% |
|  | -0.2% | -0.1% | -0.2% |  | -0.3% | -0.1% | -0.2% |  | -0.2% | -0.1% | 0.0% |  |
| Class F | 0.0% | 0.0% | -0.1% | -3.4% | -0.2% | -0.1% | 0.0% | -3.3% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | 0.0% | -3.4% | -0.2% | -0.2% | -0.7% | -2.6% |  |  |  |  |
| Class C | -0.3% | -0.2% | 0.0% | -5.2% | -0.3% | -0.3% | 0.1% | -4.8% |  |  |  |  |
| Class D | -0.2% | -0.6% | 0.1% | -5.0% | -0.3% | -0.8% | -0.5% | -4.8% |  |  |  |  |
| Class E | -0.2% | -1.2% | -0.8% | -2.4% | -0.2% | 0.3% | -0.4% | -3.1% |  |  |  |  |
| Overall | -0.2% | -0.4% | -0.1% | -4.1% | -0.2% | -0.3% | -0.4% | -3.8% |  |  |  |  |
|  | -0.2% | -0.4% | -0.2% |  | -0.2% | -0.3% | -0.3% |  |  |  |  |  |
| Class F | -0.1% | 0.0% | 0.1% | -2.3% | 0.1% | -0.3% | 0.3% | -1.5% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 103% |  |  |  |  |  |  |  |

Table 2: Summary results of test 2 of JCTVC-H0289 against test 1.1 of CE4 (anchor).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.4% | -0.4% | -0.4% | -8.1% | -0.3% | -0.3% | -0.3% | -8.4% |  |  |  |  |
| Class B | -0.4% | -0.4% | -0.4% | -10.7% | -0.3% | -0.3% | -0.3% | -11.1% |  |  |  |  |
| Class C | -0.6% | -0.5% | -0.5% | -12.5% | -0.5% | -0.5% | -0.5% | -12.9% |  |  |  |  |
| Class D | -0.5% | -0.4% | -0.5% | -12.4% | -0.4% | -0.4% | -0.4% | -12.6% |  |  |  |  |
| Class E | -0.7% | -0.7% | -0.7% | -12.4% | -0.8% | -0.7% | -0.8% | -14.5% |  |  |  |  |
| Overall | -0.5% | -0.5% | -0.5% | -11.5% | -0.5% | -0.4% | -0.5% | -12.1% |  |  |  |  |
|  | -0.5% | -0.5% | -0.5% |  | -0.5% | -0.4% | -0.4% |  |  |  |  |  |
| Class F | -0.2% | -0.4% | -0.3% | -8.2% | -0.2% | -0.3% | -0.3% | -9.2% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.1% | -0.2% | -4.6% | -0.4% | -0.2% | -0.3% | -4.7% | -0.2% | -0.2% | -0.2% | -3.7% |
| Class B | -0.3% | -0.2% | -0.2% | -5.8% | -0.3% | -0.1% | -0.1% | -5.6% | -0.2% | -0.3% | -0.3% | -5.8% |
| Class C | -0.4% | -0.2% | -0.3% | -7.7% | -0.3% | -0.1% | -0.2% | -7.4% |  |  |  |  |
| Class D | -0.3% | -0.7% | -0.5% | -7.3% | -0.3% | -0.6% | -0.3% | -7.1% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.3% | -6.5% | -0.3% | -0.2% | -0.2% | -6.4% | -0.2% | -0.2% | -0.3% | -4.9% |
|  | -0.3% | -0.3% | -0.3% |  | -0.3% | -0.2% | -0.2% |  | -0.2% | -0.2% | -0.2% |  |
| Class F | -0.2% | -0.3% | -0.3% | -5.9% | -0.3% | -0.4% | -0.2% | -6.2% |  |  |  |  |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -3.7% | -0.2% | -0.2% | -0.2% | -3.0% |  |  |  |  |
| Class C | -0.3% | -0.4% | -0.3% | -5.6% | -0.3% | -0.5% | 0.1% | -5.2% |  |  |  |  |
| Class D | -0.3% | -1.1% | 0.3% | -5.1% | -0.3% | -0.2% | 0.0% | -5.0% |  |  |  |  |
| Class E | -0.1% | -0.9% | -0.3% | -2.8% | -0.3% | -0.3% | -1.5% | -3.7% |  |  |  |  |
| Overall | -0.2% | -0.6% | -0.1% | -4.3% | -0.3% | -0.3% | -0.3% | -4.2% |  |  |  |  |
|  | -0.2% | -0.6% | -0.1% |  | -0.3% | -0.3% | -0.4% |  |  |  |  |  |
| Class F | -0.1% | 0.3% | 0.1% | -3.0% | 0.0% | 0.3% | 0.6% | -3.2% |  |  |  |  |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |

Table 3: Summary results of test 3 of JCTVC-H0289 against test 1.1 of CE4 (anchor).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.2% | -4.1% | -0.1% | -0.1% | -0.2% | -3.9% |  |  |  |  |
| Class B | -0.3% | -0.3% | -0.3% | -7.2% | -0.2% | -0.2% | -0.2% | -7.3% |  |  |  |  |
| Class C | -0.4% | -0.4% | -0.4% | -9.6% | -0.4% | -0.4% | -0.4% | -9.8% |  |  |  |  |
| Class D | -0.4% | -0.4% | -0.4% | -10.2% | -0.4% | -0.3% | -0.3% | -10.3% |  |  |  |  |
| Class E | -0.4% | -0.4% | -0.4% | -6.7% | -0.4% | -0.4% | -0.4% | -7.8% |  |  |  |  |
| Overall | -0.4% | -0.3% | -0.3% | -8.0% | -0.3% | -0.3% | -0.3% | -8.2% |  |  |  |  |
|  | -0.4% | -0.3% | -0.3% |  | -0.3% | -0.3% | -0.3% |  |  |  |  |  |
| Class F | -0.1% | -0.1% | -0.2% | -4.5% | -0.1% | -0.1% | -0.1% | -4.5% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.3% | -0.1% | -3.2% | -0.3% | 0.0% | -0.3% | -3.1% | -0.1% | 0.1% | 0.1% | -2.9% |
| Class B | -0.2% | -0.2% | -0.2% | -4.7% | -0.2% | 0.0% | -0.2% | -4.4% | -0.2% | -0.2% | -0.2% | -4.8% |
| Class C | -0.3% | -0.1% | -0.4% | -6.8% | -0.3% | -0.2% | -0.1% | -6.6% |  |  |  |  |
| Class D | -0.3% | -0.3% | -0.4% | -6.8% | -0.3% | -0.3% | -0.6% | -6.8% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.3% | -5.6% | -0.3% | -0.1% | -0.3% | -5.4% | -0.1% | 0.0% | -0.1% | -3.9% |
|  | -0.3% | -0.2% | -0.2% |  | -0.3% | -0.1% | -0.3% |  | -0.1% | -0.1% | -0.1% |  |
| Class F | 0.0% | -0.1% | -0.1% | -3.6% | -0.2% | -0.2% | 0.0% | -3.4% |  |  |  |  |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | 0.3% | -3.7% | -0.2% | -0.4% | -0.5% | -2.9% |  |  |  |  |
| Class C | -0.3% | -0.3% | -0.1% | -5.5% | -0.3% | -0.4% | 0.2% | -5.2% |  |  |  |  |
| Class D | -0.3% | -0.2% | 0.3% | -5.5% | -0.3% | -0.8% | -0.3% | -5.4% |  |  |  |  |
| Class E | -0.2% | -0.7% | -1.2% | -2.8% | -0.2% | 1.1% | -0.5% | -3.5% |  |  |  |  |
| Overall | -0.2% | -0.3% | -0.1% | -4.4% | -0.3% | -0.2% | -0.3% | -4.2% |  |  |  |  |
|  | -0.2% | -0.3% | -0.1% |  | -0.3% | -0.3% | -0.2% |  |  |  |  |  |
| Class F | -0.2% | -0.3% | -0.2% | -2.4% | 0.1% | 1.2% | 1.2% | -1.9% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 102% |  |  |  |  |  |  |  |

# Conclusion

This contribution has presented a study and cross-verification results of spatial QP prediction methods based on motion vector predictors, which are presented in JCTVC-H0289. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-H0289 and results exactly match those presented in JCTVC-H0289.

# References

[1] P. Andrivon, et al., “Non-CE4: Spatial QP prediction based on motion vector predictors (AMVP),” JCT-VC document JCTVC-H0289, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th meeting, San Jose, February 2012.

[2] H. Aoki, et al., “CE4 Subtest 1: Spatial QP prediction based on intra prediction (test 1.3.b),” JCT-VC document JCTVC-G066, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

[3] K. Sato, “CE4: Result of 2.3.d and 2.3.e,” JCT-VC document JCTVC-F420, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[4] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-G1204, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.