| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0526 |
| --- | --- |

| Title: | Multi-level Significance Maps for Small Transform Units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nguyen Nguyen, Tianying Ji, Dake He, Gaëlle Martin-Cocher 185 Columbia St. W. Waterloo, ON, Canada N2L 5Z5 | Tel: Email: | +1-519-8887465 nnguyen@rim.com tiji@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

A multi-level approach to significant coefficient flag coding for large TUs provides gains in terms of both coding efficiency and coding complexity [3].  In this proposal, multi-level significance maps are extended to small transform units (TUs), specifically, 8x8 TUs.  To accommodate this change, diagonal, horizontal and vertical sub-block scans are proposed for 8x8 TUs.  This unifies significant coefficient coding and sub-block scan across all applicable TU sizes.

For LC settings, the proposal reduces the total number of coded bins for significance maps by 0.5% for AI, 1.5% for RA, and 1.6% for LB.  The average BD-rate difference against HM-5.0 under common test conditions is 0.0%.

For HE settings, the proposal reduces the total number of coded bins for significance maps by 0.1% for AI, 0.5% for RA, and 0.6% for LB.  The average BD-rate difference against HM-5.0 under common test conditions is 0.0%, except for the RA-HE10 configuration, where the BD-rate difference is 0.1%.

# Proposal

## Multi-level Significance Maps for Small TUs

Multi-level significance maps were proposed for large TUs of size 16x16 and 32x32 in JCTVC-G644 [3].  That proposal was adopted and also applied to non-square transform units.  This contribution proposes extending multi-level significance maps to 4x4 and 8x8 TUs.  This unifies significant coefficient coding across all TU sizes and simplifies the implementation by removing checks (and the associated execution paths) for different TU sizes.

Applying multi-level significance maps to 4x4 TUs is a mostly cosmetic change.  The coded bitstream itself does not change because a 4x4 TU is exactly one 4x4 coefficient group and JCTVC-G644 [3] stipulates that the significant coefficient flag of the first coefficient group in a TU is not coded using multiple levels.  For 8x8 TUs, multi-level significance maps can be applied by extending sub-block scan.

## Horizontal and Vertical Sub-block Scan

JCTVC-G323 [2], which was adopted, introduced a diagonal sub-block scan for 16x16 and 32x32 TUs.  To accommodate multi-level significance maps for 8x8 TUs, this contribution proposes extending the diagonal sub-block scan to 8x8 TUs.  

Because 8x8 TUs can also be scanned in horizontal or vertical order, Method A proposes sub-block scan versions of the 8x8 horizontal and vertical scan as well.  Figures 1 and 2 below illustrate the proposed sub-block horizontal and vertical scan orders, respectively.  The ascending numbers in the positions define forward scanning order.

| 0 | 1 | 2 | 3 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 5 | 6 | 7 | 20 | 21 | 22 | 23 |
| 8 | 9 | 10 | 11 | 24 | 25 | 26 | 27 |
| 12 | 13 | 14 | 15 | 28 | 29 | 30 | 31 |
| 32 | 33 | 34 | 35 | 48 | 49 | 50 | 51 |
| 36 | 37 | 48 | 39 | 52 | 53 | 54 | 55 |
| 40 | 41 | 42 | 43 | 56 | 56 | 58 | 59 |
| 44 | 45 | 46 | 47 | 60 | 61 | 62 | 63 |

Figure 1: Sub-block horizontal scan for 8x8 TU (Method A)

| 0 | 4 | 8 | 12 | 32 | 36 | 40 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 9 | 13 | 33 | 37 | 41 | 45 |
| 2 | 6 | 10 | 14 | 34 | 38 | 42 | 46 |
| 3 | 7 | 11 | 15 | 35 | 39 | 43 | 47 |
| 16 | 20 | 24 | 28 | 48 | 52 | 56 | 60 |
| 17 | 21 | 25 | 29 | 49 | 53 | 57 | 61 |
| 18 | 22 | 26 | 30 | 50 | 54 | 58 | 62 |
| 19 | 23 | 27 | 31 | 51 | 55 | 59 | 63 |

Figure 2: Sub-block vertical scan for 8x8 TU (Method A)

In Method B of the proposal, we define a coefficient group in an 8x8 TU to be a 8x2 (width x height) slice for horizontal scan, a 2x8 slice for vertical scan, and a 4x4 sub-block for diagonal scan.  This provides a simple syntactic solution because no new scan orders are needed: the existing 8x8 horizontal and vertical scans in HM-5.0 are used as, and only diagonal sub-block scan is extended to 8x8 TUs.  Because Method B also provides the best performance in terms of BD-rate, we recommend it as the one to be adopted.

# Results

In this section, we report experimental results of the proposal (integrated into HM-5.0) against HM-5.0 anchor under common test conditions [1].

## Method A

Table 2.1 below compares the number of bins (decoded by BAC including bypass mode) in Method A against HM-5.0 anchor.  The overall results exclude Class F.  An entry in the column entitled “Total” is calculated as follows:

(Total bins in the proposal – Total bins in HM-5.0 anchor)/ Total bins in HM-5.0 anchor x 100%

An entry in the column entitled “decodeBin()” is calculated only for bins decoded by BAC using decodeBin() in Method A and HM-5.0 anchor.  An entry in the column entitled “Sig” is calculated only for bins decoded by BAC in Method A and HM-5.0 anchor for significance maps. An entry in the column entitled “Sig_8x8” is calculated for bins decoded by BAC in Method A and HM-5.0 anchor for significance maps of small TUs of size 8x8.

Table 2.1: Numbers of bins decoded by BAC in Method A against those in HM-5.0 anchor

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | -0.4% | -0.7% | -1.9% | -7.6% |
| Class B | 0.0% | 0.5% | 0.9% | 0.1% |
| Class C | 0.1% | 0.1% | 0.3% | -5.3% |
| Class D | 0.1% | 0.1% | 0.3% | -3.0% |
| Class E | 0.5% | 0.7% | 2.2% | -9.4% |
| Class F | 0.2% | 0.3% | 0.5% | -9.8% |
| Overall |  |  |  |  |
|  | 0.0% | 0.2% | 0.5% | -1.4% |
|  | Random Access HE |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | -0.3% | -0.4% | -1.6% | -4.5% |
| Class B | -0.4% | -0.6% | -1.4% | -3.4% |
| Class C | -0.3% | -0.5% | -1.4% | -3.4% |
| Class D | -0.4% | -0.5% | -1.6% | -3.9% |
| Class E |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | -2.0% |
| Overall |  |  |  |  |
|  | -0.4% | -0.6% | -1.5% | -3.5% |
|  | Low delay B HE |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A |  |  |  |  |
| Class B | -0.6% | -0.8% | -1.7% | -3.6% |
| Class C | -0.5% | -0.7% | -1.9% | -3.4% |
| Class D | -0.6% | -0.7% | -2.1% | -3.2% |
| Class E | -0.1% | -0.1% | -0.8% | -1.2% |
| Class F | -0.2% | -0.2% | -0.7% | -1.6% |
| Overall |  |  |  |  |
|  | -0.5% | -0.7% | -1.7% | -3.5% |
|  | Random Access HE-10 bit |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | 0.3% | 0.3% | 1.0% | -3.7% |
| Class B | -0.4% | -0.6% | -1.4% | -3.4% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Class F |  |  |  |  |
| Overall |  |  |  |  |
|  | 0.0% | 0.0% | -1.5% | -3.5% |
|  | All Intra LC |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | -0.4% | -0.6% | -1.7% | -6.4% |
| Class B | -0.5% | -0.3% | -0.8% | -2.3% |
| Class C | 0.0% | 0.0% | -0.1% | -3.5% |
| Class D | 0.0% | 0.0% | 0.0% | -1.8% |
| Class E | 0.8% | 1.0% | 2.5% | -0.1% |
| Class F | 0.1% | 0.2% | 0.2% | -5.3% |
| Overall |  |  |  |  |
|  | -0.3% | -0.2% | -0.6% | -2.7% |
|  | Random Access LC |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | -0.4% | -0.6% | -2.3% | -5.6% |
| Class B | -1.6% | -2.0% | -4.5% | -8.6% |
| Class C | -0.7% | -1.0% | -2.8% | -7.3% |
| Class D | -0.9% | -1.1% | -3.3% | -8.1% |
| Class E |  |  |  |  |
| Class F | 0.0% | -0.1% | -0.3% | -3.0% |
| Overall |  |  |  |  |
|  | -1.2% | -1.6% | -4.0% | -8.2% |
|  | Low delay B LC |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A |  |  |  |  |
| Class B | -1.7% | -2.0% | -4.0% | -7.7% |
| Class C | -1.1% | -1.4% | -3.6% | -8.4% |
| Class D | -1.2% | -1.4% | -3.8% | -7.5% |
| Class E | -0.7% | -0.8% | -2.9% | -4.6% |
| Class F | -0.4% | -0.5% | -1.7% | -5.9% |
| Overall |  |  |  |  |
|  | -1.5% | -1.8% | -3.9% | -7.7% |

Table 2.2 below shows that the BD-rates of Method A against HM-5.0 anchor (a negative value indicates gain; and a positive value indicates loss).  Note that the encoding time results of Method A and the HM-5.0 anchor are obtained on the same computing platform; and the decoding time results of Method A and the HM-5.0 anchor are obtained on the same computer.

Table 2.2: Method A BD-rate vs. HM-5.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | -0.1% | -0.2% | 0.2% | 0.1% | 0.0% |
| Class B | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class C | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class D | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E | 0.6% | -0.1% | -0.1% | 0.4% | 0.1% | 0.1% |
| Overall | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class F | 0.2% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | -0.1% | -0.3% | 0.1% | 0.0% | 0.0% | 0.2% | -0.3% | -0.1% |
| Class B | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% | -0.1% |
| Class C | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.2% | -0.2% | -0.1% |
|  | 0.1% | -0.2% | -0.1% | 0.0% | 0.1% | 0.0% | 0.2% | -0.2% | -0.1% |
| Class F | 0.1% | -0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.4% | 0.3% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | -0.2% |
| Class D | 0.0% | -0.2% | 0.1% | -0.1% | 0.4% | -0.6% |
| Class E | 0.1% | -0.2% | -0.9% | 0.1% | -1.6% | 0.2% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.3% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.0% |
| Class F | -0.2% | -0.3% | -0.1% | 0.1% | -0.8% | -0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

## Method B

Table 2.3 below compares the number of bins (decoded by BAC including bypass mode) in Method B against HM-5.0 anchor.  The overall results exclude Class F.  An entry in the column entitled “Total” is calculated as follows:

(Total bins in the proposal – Total bins in HM-5.0 anchor)/ Total bins in HM-5.0 anchor x 100%

An entry in the column entitled “decodeBin()” is calculated only for bins decoded by BAC using decodeBin() in Method B and HM-5.0 anchor.  An entry in the column entitled “Sig” is calculated only for bins decoded by BAC in Method B and HM-5.0 anchor for significance maps. An entry in the column entitled “Sig_8x8” is calculated for bins decoded by BAC in Method B and HM-5.0 anchor for significance maps of small TUs of size 8x8.

Table 2.3: Numbers of bins decoded by BAC in Method B against those in HM-5.0 anchor

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | 0.1% | 0.1% | 0.2% | 0.0% |
| Class B | -0.3% | 0.0% | 0.0% | -0.3% |
| Class C | -0.1% | -0.2% | -0.5% | -0.8% |
| Class D | 0.0% | -0.1% | -0.1% | 2.5% |
| Class E | 0.2% | 0.3% | 0.9% | -0.4% |
| Class F | -0.1% | -0.1% | -0.3% | -0.2% |
| Overall |  |  |  |  |
|  | -0.1% | 0.0% | 0.0% | -0.3% |
|  | Random Access HE |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | 0.0% | 0.0% | -0.1% | -1.3% |
| Class B | -0.6% | -0.8% | -1.7% | -3.7% |
| Class C | -0.4% | -0.5% | -1.5% | -2.7% |
| Class D | -0.4% | -0.6% | -1.7% | -2.9% |
| Class E |  |  |  |  |
| Class F | -0.2% | -0.2% | -0.5% | -0.6% |
| Overall |  |  |  |  |
|  | -0.5% | -0.6% | -1.5% | -3.5% |
|  | Low delay B HE |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A |  |  |  |  |
| Class B | -0.6% | -0.8% | -1.7% | -3.6% |
| Class C | -0.5% | -0.7% | -1.8% | -3.5% |
| Class D | -0.6% | -0.7% | -2.2% | -3.2% |
| Class E | -0.1% | -0.1% | -0.9% | -0.9% |
| Class F | -0.2% | -0.3% | -0.9% | -0.4% |
| Overall |  |  |  |  |
|  | -0.6% | -0.7% | -1.7% | -3.6% |
|  | Random Access HE-10 bit |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | 0.3% | 0.4% | 1.2% | -2.0% |
| Class B | -0.6% | -0.7% | -1.6% | -3.7% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Class F |  |  |  |  |
| Overall |  |  |  |  |
|  | 0.0% | 0.0% | -1.5% | -3.5% |
|  | All Intra LC |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | 0.1% | 0.1% | 0.1% | -0.3% |
| Class B | -0.8% | -0.6% | -1.2% | -3.3% |
| Class C | -0.3% | -0.5% | -1.1% | -3.6% |
| Class D | -0.2% | -0.2% | -0.5% | -0.1% |
| Class E | 0.2% | 0.3% | 0.7% | 0.1% |
| Class F | -0.2% | -0.3% | -0.8% | -4.8% |
| Overall |  |  |  |  |
|  | -0.5% | -0.4% | -0.9% | -2.9% |
|  | Random Access LC |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A | -0.1% | -0.2% | -0.6% | -2.0% |
| Class B | -1.7% | -2.1% | -4.6% | -9.0% |
| Class C | -0.8% | -1.0% | -2.9% | -7.8% |
| Class D | -0.9% | -1.1% | -3.3% | -7.6% |
| Class E |  |  |  |  |
| Class F | -0.3% | -0.4% | -1.2% | -4.8% |
| Overall |  |  |  |  |
|  | -1.2% | -1.6% | -3.9% | -8.2% |
|  | Low delay B LC |  |  |  |
|  | Total | decodeBin() | Sig | Sig_8x8 |
| Class A |  |  |  |  |
| Class B | -1.8% | -2.1% | -4.1% | -7.8% |
| Class C | -1.1% | -1.4% | -3.7% | -8.3% |
| Class D | -1.2% | -1.4% | -3.9% | -7.7% |
| Class E | -0.7% | -0.8% | -3.1% | -4.7% |
| Class F | -0.5% | -0.6% | -2.1% | -6.8% |
| Overall |  |  |  |  |
|  | -1.6% | -1.9% | -4.0% | -7.8% |

Table 2.4 below shows that the BD-rates of Method B against HM-5.0 anchor (a negative value indicates gain; and a positive value indicates loss).  Note that the encoding time results of Method B and the HM-5.0 anchor are obtained on the same computing platform; and the decoding time results of Method B and the HM-5.0 anchor are obtained on the same computer.

Table 2.4 Method B BD-rate vs. HM-5.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.0% | 0.2% | 0.3% | 0.4% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class D | 0.0% | -0.3% | -0.1% | -0.1% | 0.4% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% | 0.1% | 0.1% | 0.2% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.2% | 0.1% | 0.1% | 0.2% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | -0.4% | -0.1% | -0.1% | 1.0% | -0.3% |
| Class E | 0.1% | 0.7% | -0.5% | 0.0% | -0.7% | 0.1% |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.2% | -0.1% | -0.2% | 0.0% | -0.4% | -0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this proposal, multi-level significance maps are extended to 8x8 TUs. Sub-block scan is also extended. This unifies significant coefficient coding, and sub-block scan across all applicable TU sizes, while providing a reduction in coding complexity in terms of the number of coded bins.  Under common test conditions against HM-5.0 anchor, the proposal achieves overall bin savings of up to 1.6% on average, with minimal impact on coding efficiency.

# Change to the HEVC text spec.

## Proposed changes to “7.3.10 Residual Coding CABAC Syntax”

| residual_coding_cabac( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Proposed changes to “6.5.2 Scanning order array initialisation process”

[Note: Only Method A requires the changes in this section]

Input to this process is a block widthsize blkWidth and a block height blkHeight.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies the up-right diagonal scan as specified in subclause Error: Reference source not found with blkWidth and blkHeight as inputs, scanIdx equal to 1 specifies a horizontal scan, and scanIdx equal to 2 specifies a vertical scan. The array index pos specifies the scan position ranging from 0 to ( blkWidth * blkHeight ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.  Let ScanOrderX denote the ScanOrder array for blkWidth == X.

ScanOrder[0] = DiagScan

i  =  0
y  =  0
while( y  < blkHeight ) {
	x  =  0
	while( x  <  blkWidth ) {
		ScanOrder[ 1 ][ i ][ 0 ]  = x
		ScanOrder[ 1 ][ i ][ 1 ]  = y
		x++
		i++
	}
	y++
}

i  =  0
x  =  0
while( x  < blkWidth ) {
	y  =  0
	while( y  <  blkHeight ) {
		ScanOrder[ 2 ][ i ][ 0 ]  = x
		ScanOrder[ 2 ][ i ][ 1 ]  = y
		y++
		i++
	}
	x++
}

if( blkWidth == 8 ) {

HOffset[2] = {0, 0}

VOffset[2] = {0, 0}

for (CGIdx= 0; CGIdx < 4;  CGIdx++) {

for (i = 0; i < 16;  i++) {

ScanOrder8[1][i+CGIdx*16][0] = ScanOrder4[1][i][0] + HOffset[0]

ScanOrder8[1][i+CGIdx*16][1] = ScanOrder4[1][i][1] + HOffset[1]

ScanOrder8[2][i+CGIdx*16][0] = ScanOrder4[2][i][0] + VOffset[0]

ScanOrder8[2][i+CGIdx*16][0] = ScanOrder4[2][i][0] + VOffset[1]

}

if( CGIdx == 0 ) {

HOffset = {4, 0}

VOffset = {0, 4}

}

else if( CGIdx == 1 ) {

HOffset = {0, 4}

VOffset = {4, 0}

}

else {

HOffset = {4, 4}

VOffset = {4, 4}

}

}

}

## Proposed changes to “7.4.10 Residual Coding Semantics”

[Note: Only Method B requires the changes in this section]

significant_coeff_group_flag[ xCG ][ yCG ] specifies for the coefficient group position ( xCG, yCG ) within the current transform block whether the corresponding coefficient group at location ( xCG, yCG ) has non-zero transform coefficient level. A coefficient group at location ( xCG, yCG ) is a 2x8 (height x width) array in a 8x8 TU scanned horizontally, a 8x2 array in a 8x8 TU scanned vertically, and a 4x4 array otherwise, of 16 transform coefficient level at locations ( xC, yC ).

When significant_coeff_group_flag[ xCG ][ yCG ] is not present, it is inferred to be equal to 0.

# References

[1] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting, Torino, July 2011.

[2] J. Sole, R. Joshi, and M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” JCTVC-G323.

[3] N. Nguyen, T. Ji, D. He, G. Martin-Cocher and L. Song, “Multiple Level Significance Maps for Large Transform Units”, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 7th Meeting, Geneva, November 2011.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).