| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0531 WG11 Number: m23412 |
| --- | --- |

| Title: | AHG21: Comments on Signaling of Reference Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document provides comments on signaling of reference pictures. This document proposes syntax to signal long-term reference pictures.

# Introduction

JCTVC-F493 [1] proposed absolute signaling of reference pictures, which supports error robust management of reference pictures using Buffer Descriptions.

HEVC WD5 [2] provides syntax for long-term reference pictures (LTRP) based on proposal 7a from JCTVC-G788 [3].

The current long-term reference picture signaling method in WD5:

-Sends a long_term_ref_pics_present_flag syntax element in PPS and

-Sends delta_poc_lsb_lt_minus1[i] value (and used_by_curr_pic_lt_flag[i] value) in slice header for each LTRP. This syntax element delta_poc_lsb_lt_minus1[i] is used to determine the value of the least significant bits of the picture order count value for the i’th LTRP included in the LTRP set of the current picture. Delta_poc_lsb_lt_minus1[i] is in the range of 0 to MaxPicOrderCntLSB-1 inclusive.

The above current technique has a limitation in that only after marking a LTRP as “unused for reference”  would the encoder be able to replace it with another LTRP with the same POC LSB value.

We propose a signaling scheme to overcome this limitation.

Our proposed signaling scheme consists of sending a delta_poc_msb_cycle_lt_minus1 value in the case a new LTRP is sent with the same POC LSB value as any existing LTRP picture. Also delta_msb_cycle_lt_minus1 value is sent in case a LTRP has different POC MSB value than the previous LTRP’s POC MSB value.

The proposed approach is bit efficient as the delta_poc_msb_cycle_lt_minus1 syntax element is only sent when required for correct POC computation.

# Proposed Change

We propose following syntax and semantics to signal a long-term reference picture.

## Syntax and Semantics:

We define the appropriate syntax and semantics based on syntax defined in WD5 [2].

### Picture Parameter set RBSP syntax

No Syntax change with respect to the HEVC WD5 [2] for PPS is required.

### Slice header RBSP syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| delta_poc_msb_present_flag[i] | u(1) |
| if(delta_poc_msb_present_flag[i] ==1) { |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| } |  |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

delta_poc_msb_present_flag[i] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present.  delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present.  delta_poc_msb_present_flag[ i ] shall be equal to 1 when DeltaPocLt[ i ] is equal to DeltaPocLt[ j ] for values of j in the range of 0 to num_long_term_pics – 1, inclusive, and being unequal to i.

delta_poc_msb_cycle_lt_minus1[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if ( i  = =  0  | |  DeltaPocLt[ i – 1 ]  ! =  DeltaPocLt[ i ] ) 
	  DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt_minus1[ i ] + 1				
	else
	  DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt_minus1[ i ] + 1 +

DeltaPocMSBCycleLt [ i – 1 ]

Complete output text is provided in an accompanying document.

# Conclusion

The document proposed syntax to signal long-term reference pictures. The proposed approach can remove limitations of existing signaling approach. We propose to adopt the proposed approach in HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] R. Sjöberg, J. Samuelsson (Ericsson), "Absolute signaling of reference pictures," JCTVC-F493/m20923 6th Meeting: Torino, IT, 14-22 July, 2011.

[2] JCTVC-G1103d06 “WD5: Working Draft 5 of High-Efficiency Video Coding, January 2012

[3] G. Sullivan, Y. Wu, J. Xu, B. Li, “Proposal on Decoded Picture Buffer Description Syntax Related to AHG21 and JCTVC-F493,” JCTVC-G788,  7th meeting Geneva, CH, 21-30 November, 2011