| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0535 |
| --- | --- |

| Title: | CE1 B3: 8-bit Linear Initialization for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo¹ Marta Karczewicz¹ Joel Sole¹ Rajan Joshi¹ Chuohao Yeo² Yih Han Tan2 Zhengguo Li2 | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com chyeo@i2r.a-star.edu.sg |
| Source: | 1: Qualcomm Incorporated 2: Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1 subset B 3, 8-bit linear initialization for CABAC. Compared to the piece-wise linear initialization in HM5, the proposed method does not need to segment the probability range and no table storage is reportedly needed. Experimental results reportedly show that there is no change in BD-rate with the linear initialization scheme compared to HM5 anchor.

# Introduction

In HM5, a piece-wise linear process is used. Specifically, the range of the probability is divided into several segments and each segment has its own slope. The initialization process is described as follows:

to calculate the initial CABAC state from the 8-bit initialization value initValue.

m = m_slopes[initValue>>4];

n = initValue & 15;

val = max( 0, min( 3839, (n<<8) + m * (qp-26) + 128 ) );

val -= 1920;

valMps = val >= 0;

val = ( val ^ (val>>11) ) + 128;

segmentIdx = val >> 8;

pStateIdx = m_accumulatedSegOffset[segmentIdx] + ( (val&255) * m_segOffset[segmentIdx] >> 8);

m_ucState = (pStateIdx<<1) + valMps;

Three tables are introduced to specify the piece-wise linear segments and calculate the slope in every segment.

# Proposed linear initialization

The initialization value initValue is an 8-bit integer with two parts: a 4-bit SlopeIdx (b0-b3) and a 4-bit IntersecIdx (b4-b7), as shown in the following figure.

A slope value m and an intersect value n are derived from SlopeIdx and InterSecIdx using the following formula (Formula1)

m  = (SlopeIdx*78-689)>>4                   (1)

n  = (IntersecIdx<<3) – 13                  (2)

Linear equations are used to calculate a probability state and a MPS value

ProbState = Clip(1,126, (m*QP>>4) + n)

valMPS = ProbState>=64

The final CABAC state is derived as follows:

m_ucState = valMps? (ProbState - 64 ):(63-ProbState);

m_ucState = (m_ucState<<1) + valMps;

Besides Formula1, we also tested another set of formula (Formula2) which is simpler.

m  =  SlopeIdx*5-45                                (3)

n  = (IntersecIdx<<3) – 16                        (4)

The following table provides a preliminary comparison between the proposed linear initialization and the piece-wise linear initialization in HM5 based on current implementation..

|  | HM5 | Formula1 | Formula2 |
| --- | --- | --- | --- |
| # of Tables | 3 | 0 | 0 |
| Total table size (bytes) | 48 | 0 | 0 |
| Number of operations | 24 | 17 | 16 |
| WD Texts | 3 tables + texts | texts | texts |

# Experimental results

The proposed linear initialization is implemented on top of HM 5.0, and evaluated under common conditions as specified in [2]. The encoding time and decoding time are obtained from cluster.

For fair comparison, no extra training is performed. The initialization values are converted from HM5 initialization values, and the codes for the conversion have been disclosed in CE1 group. Table I and II summarized the BD-rate results in different cases. Extra tests with high QP (27-42) and low QP (17-32) are also provided. It can be seen virtually there is no change in the coding efficiency compared to HM5. The detail results of normal QP are presented in Table III and IV. More detail data can be found in excel files.

Table I Summary of performance of Formula 1 under different cases and QPs

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| highQP | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| lowQP | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.1% |
| highQP | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | 0.1% |
| lowQP | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | -0.1% | -0.3% | 0.0% | -0.4% | -0.2% | 0.0% | 0.0% | 0.0% |
| highQP | 0.0% | -0.2% | -0.3% | 0.0% | -0.7% | -0.5% | 0.0% | 0.1% | 0.1% |
| lowQP | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | 0.1% | -0.2% | 0.0% | -0.3% | -0.1% | -0.1% | 0.0% | 0.1% |
| highQP | 0.0% | 0.2% | -0.4% | 0.0% | -0.5% | 0.1% | 0.0% | 0.1% | 0.2% |
| lowQP | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% |

Table II Summary of performance of Formula 2 under different cases and QPs

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| highQP | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| lowQP | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| highQP | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | 0.1% |
| lowQP | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | -0.1% |
| highQP | 0.0% | -0.2% | -0.4% | 0.0% | -0.9% | -0.2% | 0.0% | -0.2% | -0.2% |
| lowQP | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| normalQP | 0.0% | 0.2% | -0.1% | 0.1% | -0.1% | -0.3% | 0.0% | 0.1% | -0.1% |
| highQP | 0.0% | 0.2% | -0.6% | 0.0% | -0.3% | -0.2% | 0.0% | 0.0% | 0.0% |
| lowQP | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% |

Table III Detail results of Formula1 under normal QP

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  | 96% |  |  | 95% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  | 95% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.2% | 0.0% | 0.2% | -0.2% | 0.0% | 0.2% | 0.3% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.3% | -0.1% | 0.0% | 0.3% | -0.2% | 0.0% | -0.2% | 0.3% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 96% |  |  | 95% |  |  | 94% |  |  |
| Dec Time[%] | 95% |  |  | 96% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | -0.3% | 0.2% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | 0.3% | -0.2% | 0.0% | -0.2% | 0.0% |
| Class E | 0.0% | -0.2% | -1.5% | 0.1% | -0.6% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class F | -0.1% | -0.3% | -0.6% | -0.1% | -1.3% | -0.7% | -0.1% | 0.3% | 0.0% |
| Overall | 0.0% | -0.1% | -0.3% | 0.0% | -0.4% | -0.2% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | -0.4% | -0.2% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 96% |  |  | 96% |  |  |
| Dec Time[%] | 95% |  |  | 93% |  |  | 94% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | -0.3% |
| Class C | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.4% | -0.5% | -0.1% | -0.2% | 0.2% | 0.1% | -0.1% | 0.4% |
| Class E | 0.0% | 0.1% | -1.3% | -0.1% | -1.2% | -1.3% | -0.2% | -0.3% | 0.0% |
| Class F | -0.1% | 0.0% | 0.3% | 0.1% | -0.4% | 0.3% | -0.3% | 0.2% | 0.3% |
| Overall | 0.0% | 0.1% | -0.2% | 0.0% | -0.3% | -0.1% | -0.1% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | -0.2% | 0.0% | -0.3% | -0.1% | -0.1% | 0.0% | 0.1% |
| Enc Time[%] | 98% |  |  | 96% |  |  | 96% |  |  |
| Dec Time[%] | 93% |  |  | 94% |  |  | 95% |  |  |

Table IV Detail results of Formula2 under normal QP

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 97% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.1% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.3% | 0.1% | 0.0% | -0.1% | 0.3% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 95% |  |  | 96% |  |  | 94% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | -0.4% | 0.4% | 0.0% | 0.0% | -0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | -0.2% | 0.3% | -0.1% | 0.4% | 0.1% | -0.1% | 0.0% | 0.1% |
| Class E | 0.0% | 0.8% | -0.9% | 0.1% | -0.6% | 0.0% | -0.2% | -1.4% | -0.4% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | -1.0% | -0.6% | -0.1% | 0.3% | -0.2% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 95% |  |  | 97% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.3% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.4% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.4% | -0.1% | 0.1% | -0.4% | 0.1% |
| Class E | 0.0% | 1.3% | 0.0% | 0.1% | -1.2% | -1.2% | -0.1% | 0.0% | -0.7% |
| Class F | -0.1% | 0.0% | -0.3% | 0.2% | -0.3% | -0.2% | -0.2% | 0.4% | 0.0% |
| Overall | 0.0% | 0.2% | -0.1% | 0.1% | -0.1% | -0.3% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.1% | 0.1% | -0.1% | -0.2% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 96% |  |  | 96% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |

# Conclusions

This contribution reports the test results of 8-bit linear initialization. The proposed method uses a slope for the whole probability range and thus the probability range segment process is saved. Compared to piece-wise linear initialization in HM5, the proposed linear initialization have fewer operations, shorter WD texts and there is no table storage required. Two different sets of formula (Formula1 and Formula2) for the linear initialization are tested on HM5, and both of them demonstrated the same coding efficiency as HM5. It is recommended the proposed linear initialization method be adopted in the next version of HM.

# Draft Text

WD for Formula2 is provided in this section. The WD for Formula1 is the same except different formulas are used to calculate m and n.

#### Initialisation process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdxTable and ctxIdx.

Error: Reference source not found to Error: Reference source not found Error: Reference source not found to Error: Reference source not found30 contain the values of the 8 bit variable initValue used in the initialisation of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and n are derived according to the following pseudo-code process:

slopeIdx = initValue >> 4
n = initValue & 15

For every slopeIdx, a slope m is specifed in Table 9-1.

Table 9-1 – Mapping from variable m to variable slopeIdx

| slopeIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | -239 | -143 | -85 | -51 | -31 | -19 | -11 | 0 | 11 | 19 | 31 | 51 | 85 | 143 | 239 | 399 |

Table 9-2 – Mapping from variable segmentIdx to variables segOffset[segmentIdx] and accumulatedSegOffset[segmentIdx]

| segmentIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| segOffset[segmentIdx] | 6 | 7 | 5 | 7 | 10 | 14 | 16 | 1 |
| accumulatedSegOffset[segmentIdx] | −3 | 3 | 10 | 15 | 22 | 32 | 46 | 62 |

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the variable m as derived from slopeIdx according to Table 9-1 and variable n, the initialisation is specified by the following pseudo-code process using the two mappings as specified in Table 9-2.

qpMinus26 = Clip3( 0, 51, SliceQPY ) − 26
val = Clip3( 0, 3839, ( n << 8 ) + m  qpMinus26 + 128 )
val −= 1920
valMps = ( val >= 0 )	(9-1)
val = ( val ^ ( val >> 11 ) ) + 128
segmentIdx = val >> 8
pStateIdx = accumulatedSegOffset[segmentIdx] + ( (val & 255 ) * segOffset[segmentIdx] >> 8 )

In Error: Reference source not found, the ctxIdxTable and the ctxIdx for which initialisation is needed for each of the slice types are listed. The referenced context index tables ctxIdxTable include the values of initValue needed for the initialisation.

From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and intersecIdx are derived according to the following pseudo-code process:

slopeIdx = initValue>>4;

intersecIdx= initValue &15;

Slope m and Intersec n are derived from the indices as follows:

m = slopeIdx*5 - 45;

n = (intersecIdx<<3)-16;

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY which is derived in Equation Error: Reference source not found. Given the variable m and variable n, the initialisation is specified by the following process

preCtxState = Clip3( 1, 126, ( ( m  Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )

valMPS     = ( preCtxState  <=  63)? 0: 1;  
pStateIdx = valMPS? (preCtxState – 64): (63 – preCtxState);                          (9-4)

# References

- D.Marpe, “Unified PIPE-Based Entropy Coding for HEVC”, JCTVC-F268, JCT-VC Meeting, Torino, Italy, Jul. 2011.

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).