| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0536 |
| --- | --- |

| Title: | CE1 D2: Adaptive initialization for CABAC with fixed probability contexts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Joel Sole Xianglin Wang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the result of CE1 Subtest D2: an adaptive initialization method for CABAC with fixed probability contexts. At the beginning of each slice, a context can be initialized in one of two ways: 1) initialized using the pre-defined 8-bit initialization value (i.e., original HM method), or 2) initialized using a state value selected by the encoder. For each context, the encoder signals the selection of initialization method to the decoder as well as the new state value (if the second method is selected). No probability/state update will be performed in the encoding/decoding process, and thus the proposed method is friendly to possible parallel-processing applications. Experiments show that there is 0.3%-0.5% Y BD-rate increase under all intra coding conditions, and 1.7%-1.9% Y BD-rate increase under inter coding conditions. This contribution is cross-checked by RIM in JCTVC-H0652.

# Introduction

In CABAC, each context is associated to a state that is an estimation of the probability of the symbol “1”. The initial state is calculated using a piece-wise linear model with a pre-defined 8-bit initialization value. The state is continuously updated based on the coded symbol values. This context update makes parallel CABAC encoding/decoding difficult. In Torino meeting some contributions [1] were presented to disable the context update stage and use the pre-defined initial state during the whole encoding/decoding process. However, the pre-defined states are fixed values and do not adapt to different sequences or frames. There is coding efficiency drop when the pre-defined initial state derivates the true symbol statistics

This contribution describes an adaptive initialization method. For each context, a binary value is transmitted to indicate if this context is initialized using the pre-defined initialization value or using an adaptive initial state value. The initial state can be derived from a previous frame whose statistics is similar to the current one. The encoder signals this adaptive initial state to the decoder. Details are introduced in the next section.

# Proposed method

At the beginning of each slice, the encoder first sends a binary map CtxMap of size N, where N equals to the total number of valid contexts for the current slice type. In this way, each element in this map corresponds to a CABAC context as follows:

- If the i-th map entry CtxMap (i) equals to 0, the i-th context, Ctx(i), will be initialized using state_hm derived from pre-defined initialization value.

- If CtxMap(i) equals to 1, Ctx(i) is initialized using an adaptive value state_adap transmitted by the encoder.

## Transmission of CtxMap

The CtxMap is encoded using run-length coding. A symbol “run” is sent to indicate the number of consecutive 0s before it hits a ‘1’ or the end of the array. Symbol “run” is encoded using Expo-Golomb codes.

## Calculation and transmission of state_adap

The encoder first calculates state_adap based on the statistics of a previous slice which has the same slice type and encoding QP as the current one. Then the encoder compares state_adap and state_hm and calculates a state offset, state_offset = state_adap - state_hm. The absolute value of state_offset, abs_state_offset, is further quantized,

level = (abs_state_offset)/Qstep

The combo of (level-1, sign) is encoded using Expo-Golomb and sent to the decoder. In our simulation, a value of Qstep =2 is used.

# Experimental results

The proposed method is implemented on top of HM 5.0. The coding efficiency is evaluated under common conditions as specified in [2]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time and decoding time are obtained from running encoding on cluster with same CPU type.

The results are presented in Table I. As additional information, we also present results of HM5 disabling context updates in Table II, which we can see the Y-BD rate reduction from adaptively sending offset is between 2.7% to 4.0%.

# Conclusions

This contribution describes an adaptive initialization method for CABAC with fixed probability contexts. The contexts are selectively initialized using a state transmitted by the encoder. Experimental results show that there is 0.3%-0.5% Y BD-rate increase under all intra coding conditions, and 1.7%-1.9% Y BD-rate increase under inter coding conditions.  This is a 2.7% to 4.0% Y-BD rate reduction compared to HM 4 without context update.

Performance of the proposed method   Performance of the proposed method

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -1.2% | -0.9% | -0.2% | -0.9% | -0.9% |
| Class B | 0.5% | -0.3% | -0.1% | 0.9% | -0.3% | -0.4% |
| Class C | 0.4% | -0.3% | 0.0% | 0.8% | 0.0% | 0.0% |
| Class D | 0.1% | -0.3% | 0.0% | 0.3% | -0.3% | -0.1% |
| Class E | 0.2% | -1.4% | -0.5% | 0.3% | -0.9% | -0.7% |
| Overall | 0.3% | -0.6% | -0.2% | 0.5% | -0.4% | -0.4% |
|  | 0.2% | -0.5% | -0.2% | 0.5% | -0.4% | -0.3% |
| Class F | 1.4% | 0.7% | 1.1% | 1.7% | 1.3% | 1.2% |
| Enc Time[%] | 100% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.7% | -0.6% | -0.5% | 0.8% | -1.2% | -1.3% |
| Class B | 2.0% | 1.1% | 1.4% | 2.2% | 0.1% | 0.1% |
| Class C | 2.0% | 1.5% | 2.1% | 2.2% | 1.2% | 1.4% |
| Class D | 2.3% | 2.3% | 2.5% | 2.5% | 1.9% | 1.7% |
| Class E |  |  |  |  |  |  |
| Overall | 1.9% | 1.3% | 1.6% | 2.1% | 0.7% | 0.7% |
|  | 1.9% | 1.3% | 1.7% | 2.1% | 0.7% | 0.7% |
| Class F | 3.4% | 3.3% | 3.6% | 3.5% | 2.7% | 3.0% |
| Enc Time[%] | 95% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.6% | 1.0% | 1.7% | 1.6% | 0.3% | 1.5% |
| Class C | 1.4% | 1.4% | 1.8% | 1.3% | 1.3% | 1.6% |
| Class D | 2.0% | 2.7% | 2.7% | 2.1% | 3.0% | 3.0% |
| Class E | 2.7% | -1.6% | -3.1% | 3.1% | -3.1% | -2.0% |
| Overall | 1.8% | 1.1% | 1.1% | 1.9% | 0.6% | 1.2% |
|  | 1.8% | 1.1% | 1.2% | 1.9% | 0.5% | 1.2% |
| Class F | 5.9% | 6.5% | 7.0% | 6.1% | 4.5% | 5.6% |
| Enc Time[%] | 99% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 1.2% | 2.7% | 3.3% |
| Class B | 2.1% | 1.3% | 1.3% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 1.7% | 1.9% | 2.2% |
|  | 2.5% | 2.1% | 2.4% |
| Class F |  |  |  |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 99% |  |  |

Performance of the proposed method I  Performance of HM5 with context updating disabled

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 3.4% | -1.8% | -2.0% | 3.0% | 0.1% | -0.3% |
| Class B | 3.5% | -0.4% | -0.8% | 4.0% | 1.1% | 0.7% |
| Class C | 2.4% | 1.1% | 1.6% | 2.8% | 1.7% | 1.7% |
| Class D | 1.9% | 1.2% | 1.5% | 2.0% | 1.4% | 1.5% |
| Class E | 4.4% | -6.4% | -5.6% | 3.8% | -3.0% | -2.6% |
| Overall | 3.1% | -0.8% | -0.7% | 3.2% | 0.5% | 0.4% |
|  | 3.0% | -0.7% | -0.5% | 3.1% | 0.6% | 0.5% |
| Class F | 6.9% | 5.4% | 6.4% | 6.6% | 5.3% | 5.4% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 4.0% | 2.3% | 2.2% | 3.8% | 0.6% | -0.3% |
| Class B | 4.8% | 4.5% | 4.0% | 5.2% | 2.3% | 1.5% |
| Class C | 4.4% | 6.1% | 6.8% | 4.7% | 4.2% | 4.3% |
| Class D | 4.4% | 5.4% | 5.6% | 4.8% | 3.6% | 3.4% |
| Class E |  |  |  |  |  |  |
| Overall | 4.5% | 4.8% | 5.0% | 4.8% | 2.9% | 2.5% |
|  | 4.5% | 4.9% | 5.1% | 4.8% | 3.0% | 2.6% |
| Class F | 8.1% | 8.5% | 9.0% | 7.8% | 6.6% | 7.1% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 4.9% | 6.2% | 6.7% | 6.4% | 5.1% | 5.9% |
| Class C | 4.0% | 6.9% | 7.6% | 5.3% | 6.2% | 6.5% |
| Class D | 4.4% | 5.9% | 6.0% | 6.0% | 6.0% | 4.8% |
| Class E | 4.2% | -1.0% | -3.1% | 5.6% | -4.3% | -2.9% |
| Overall | 4.4% | 5.0% | 4.9% | 5.9% | 3.8% | 4.1% |
|  | 4.4% | 5.0% | 5.0% | 5.9% | 4.0% | 4.4% |
| Class F | 8.4% | 10.2% | 10.8% | 8.6% | 7.6% | 8.8% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 5.5% | 5.6% | 6.3% |
| Class B | 4.9% | 4.4% | 3.5% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 5.2% | 5.0% | 4.7% |
|  | 5.9% | 5.3% | 5.0% |
| Class F |  |  |  |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | #NUM! |  |  |

# Draft Text

### 7.3.3  Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| if( entropy_coding_mode_flag) |  |
| adaptive_initialization_flag | u(v) |
| if (adaptive_initialization_flag == 1) |  |
| ctx_update_info() |  |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

| ctx_update_info ( ) { | Descriptor |
| --- | --- |
| ctx_map_size | u(v) |
| ctx_idx = 0 |  |
| while (ctx_idx < ctx_map_size) { |  |
| run | ue(v) |
| ctx_idx = ctx_idx + run |  |
| If (ctx_idx > ctx_map_size) { |  |
| break |  |
| } |  |
| ctxUpdateMap[ctx_idx] =1 |  |
| ctx_idx ++ |  |
| } |  |
| ctx_idx = 0 |  |
| while (ctx_idx < total_idx_num) { |  |
| if (ctxUpdateMap[ctx_idx] == 1){ |  |
| level_minus1_and_sign | ue(v) |
| ctxUpdateLevel [ctx_idx] = level_sign |  |
| } |  |
| ctx_idx++ |  |
| } |  |
| } |  |

### 4. 3. Slice header semantics

adaptive_initialization_flag: equal to 1 specifies that the adaptive initialization process is invoked.

ctx_map_size specifies the size of ctxUpdateMap

run specifies the number of consecutive elements in ctxUpdateMap with zero value before a non-zero valued element

level_minus1_and_sign specifies the codewords of a different initial state for CABAC context.

#### 2. 1. 1 Initialisation process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdxTable and ctxIdx.

Error: Reference source not found toError: Reference source not found contain the values of the 8 bit variable initValue used in the initialisation of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and n are derived according to the following pseudo-code process:

slopeIdx = initValue >> 4
n = initValue & 15

For every slopeIdx, a slope m is specifed in Table 9-1.

Table 9-1 – Mapping from variable m to variable slopeIdx

| slopeIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | -239 | -143 | -85 | -51 | -31 | -19 | -11 | 0 | 11 | 19 | 31 | 51 | 85 | 143 | 239 | 399 |

Table 9-2 – Mapping from variable segmentIdx to variables segOffset[segmentIdx] and accumulatedSegOffset[segmentIdx]

| segmentIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| segOffset[segmentIdx] | 6 | 7 | 5 | 7 | 10 | 14 | 16 | 1 |
| accumulatedSegOffset[segmentIdx] | −3 | 3 | 10 | 15 | 22 | 32 | 46 | 62 |

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the variable m as derived from slopeIdx according to Table 9-1 and variable n, the initialisation is specified by the following pseudo-code process using the two mappings as specified in Table 9-2.

qpMinus26 = Clip3( 0, 51, SliceQPY ) − 26
val = Clip3( 0, 3839, ( n << 8 ) + m  qpMinus26 + 128 )
val −= 1920
valMps = ( val >= 0 )	(9-1)
val = ( val ^ ( val >> 11 ) ) + 128
segmentIdx = val >> 8
pStateIdx = accumulatedSegOffset[segmentIdx] + ( (val & 255 ) * segOffset[segmentIdx] >> 8 )

- If ctxUpdateMap[ctx_idx] == 1

InitState = valMps? (64+pStateIdx);  63-pStateIdx);

sign = (ctxUpdateLevel [ctx_idx]%2)

level = ctxUpdateLevel [ctx_idx]>>1+1;

CtxStateOffset = sign? –level<<1: level<<1;

preCtxState  = InitState + CtxStateOffset;

else if ctxUpdateMap[ctx_idx] == 0

preCtxState = InitState

- pStateIdx  =  valMps? preCtxState – 64: 63 − preCtxState

if( preCtxState  <=  63 )  {
	pStateIdx = 63 − preCtxState
	valMPS = 0	
} else {
	pStateIdx = preCtxState − 64
	valMPS = 1
}

In Error: Reference source not found, the ctxIdxTable and the ctxIdx for which initialisation is needed for each of the slice types are listed. The referenced context index tables ctxIdxTable include the values of initValue needed for the initialisation.

# References

- D.Marpe, “Unified PIPE-Based Entropy Coding for HEVC”, JCTVC-F268, JCT-VC Meeting, Torino, Italy, Jul. 2011.

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).