| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0537 |
| --- | --- |

| Title: | CABAC Contexts Reduction for Last Position Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Wei-Jung Chien Xianglin Wang Marta Karczewicz | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, a method to reduce the contexts for last position coding is proposed. First, a single context is used for the last bins (with context) from different block sizes; Second, chroma 8x8 and 16x16 blocks share the same set of contexts for last position coding. It is reported a total context saving of 16 (from 60 to 44) is achieved. Experimental results reportedly show there is 0.0% Y-BD rate change under the common test conditions.

# Introduction

In the last meeting, contribution G704 [1] was adopted into HM5.0 for last position coding. The last position is encoded using a binary string which consists of two parts. For bins in the first part, which are labeled as ai in Fig.1, they are encoded with contexts. For bins in the second part, which are labeled as bj, they are encoded using bypass mode (and no context is needed).

Fig1. An example binary string for Last position coding

Table I and II lists the context assignments for luma and chroma components in different block sizes. There are 18 contexts for luma component, and 12 contexts for chroma components. Separate contexts are used for last position coding in X dimension and in Y dimension. So in total there are (18+12)x2 = 60 contexts.

Table I Context assignment for Last (Luma) in HM5

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 6 |  |  |  |  |
| TU 16x16 | 7 | 8 | 9 | 9 | 10 | 10 | 11 |  |  |
| TU 32x32 | 12 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 |

Table II Context assignment for Last (Chroma) in HM5

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 6 |  |  |  |  |
| TU 16x16 | 7 | 8 | 9 | 9 | 10 | 10 | 11 |  |  |

# Proposed solution

We observed that in last position coding some bins have very similar statistics and propose to merge contexts for these bins. First, we let bin an-1 (the last one with context coding) from different block size share the same context. Second, bins from chroma block 8x8 and 16x16 share the same set of contexts.

Table III and IV show the proposed context assignment for Luma and Chroma respectively.

Table III Proposed context assignment for Last (Luma)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 2 |  |  |  |  |
| TU 16x16 | 6 | 7 | 8 | 8 | 9 | 9 | 2 |  |  |
| TU 32x32 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 |

Table IV Proposed context assignment for Last (Chroma)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 2 |  |  |  |  |
| TU 16x16 | 3 | 4 | 5 | 5 | 6 | 6 | 2 |  |  |

Table V summaries the number of contexts in HM5 and the proposed scheme. The total saving is 16 contexts, which is a saving of 27% compared to HM5.

Table V Number of contexts for last position coding in HM5 and proposed method

|  | HM5 |  | Proposed |  |
| --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma |
| LastX | 18 | 12 | 15 | 7 |
| LastY | 18 | 12 | 15 | 7 |
| Total | 60 |  | 44 |  |

# Experimental results

The proposed method is implemented on top of HM 5.0. The coding efficiency is evaluated under common conditions as specified in [2]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time and decoding time are obtained from running encoding on cluster with same CPU type.

Table VI shows the results of the proposed method.

Table VI Performance of the proposed contexts reduction

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 96.5% |  |  | 98.5% |  |  |  |  |  |
| Dec Time[%] | 94.6% |  |  | 93.1% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.3% | 0.1% | 0.0% | 0.4% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.2% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class F | -0.1% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% |  |  |  |
| Enc Time[%] | 96.5% |  |  | 96.3% |  |  | 90.6% |  |  |
| Dec Time[%] | 94.7% |  |  | 95.3% |  |  | 93.9% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.4% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class C | 0.0% | 0.1% | 0.4% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | 0.7% | -0.5% |  |  |  |
| Class E | 0.0% | 0.0% | -0.9% | 0.0% | -0.4% | 0.2% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | -0.1% | -0.1% | 0.0% | -0.2% | -0.8% | -0.5% |  |  |  |
| Enc Time[%] | 96.0% |  |  | 98.4% |  |  |  |  |  |
| Dec Time[%] | 93.1% |  |  | 92.4% |  |  |  |  |  |

# Conclusions

In this proposal, a method to reduce CABAC contexts for last position coding is described. Compared to HM5, the number of contexts has been reduced from 60 to 44. It is reported that the proposed context reduction has nearly no impact on BD-rate. Based on the results, we suggest including this context reduction into HM.

# Draft Text

##### 9.2.3.1.1.3. Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the color component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

Table 9-1 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 10 | 10 | 11 | 11 | 12 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 | 17 |

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| I | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 | 2 |

The vector lastCtx[ i ] is specified in Table 9-1 and ctxIdxInc is derived as follows.

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth – 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-1)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight – 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-2)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

if  log2TrafoDimension equals to16 and binIdx is smaller than 6

ctxIdxInc =  ctxIdxInc  +  15

where

log2TrafoDimension  = log2TrafoWidth for last_significant_coeff_x_prefix,

log2TrafoDimension  = log2TrafoHeight for last_significant_coeff_y_prefix

else

ctxIdxInc =  ctxIdxInc  +  18

# References

- W.-J. Chien , J. Sole, and M. Karczewicz, “Last position coding for CABAC”, JCTVC-G704, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).