| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0538 |
| --- | --- |

| Title: | CE2 4.3: Report on Limiting Chroma Transform Depth in RQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Xianglin Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the result of CE2 Subtest 4.3: Limiting Chroma Transform Depth in RQT. The maximum chroma transform depth is limited relative to luma component, so that chroma components do not have to share the same transform depth as luma.

# Introduction

RQT scheme is applied to select and perform transform in HEVC. With the current structure, the luma and chroma transform depths are synchronized. However, since chroma components tend to be more smooth than luma component, transform at a lower depth (compared to luma) may be more appreciate. CE2 has been established at the 7-th JCT-VC meeting to study this issue, and this contribution reports the result of CE2 subtest 4.3.

# Proposed method

The following two parameters are introduced to define the maximum chroma depth for inter and intra respectively:

- max_transform_hierarchy_depth_inter_chroma

- max_transform_hierarchy_depth_intra_chroma.

The depth of chroma transform is derived as follows:

Inter case:

DepthChroma = min( DepthLuma, max_transform_hierarchy_depth_inter_chroma)

Intra case:

DepthChroma = min( DepthLuma, max_transform_hierarchy_depth_intra_chroma)

Furthermore, 32x32 transform is also enabled for chroma transform in 64x64 CU.

# Experimental results

The proposed method is implemented on top of HM 5.0. The coding efficiency is evaluated under common conditions as specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time and decoding time are obtained from running encoding on cluster.

The results are presented in Table I. 2.1%-2.5% chroma BD-rate reduction is observed in all intra coding cases.

Test results of CE2 4.3   Test results of CE2 4.3

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -2.4% | -3.1% | 0.0% | -1.5% | -2.4% |  |  |  |
| Class B | -0.1% | -3.0% | -3.1% | 0.0% | -2.5% | -3.2% |  |  |  |
| Class C | -0.1% | -1.3% | -1.2% | 0.0% | -1.2% | -1.4% |  |  |  |
| Class D | 0.0% | -0.7% | -0.6% | 0.0% | -0.7% | -0.7% |  |  |  |
| Class E | 0.0% | -5.7% | -4.8% | 0.0% | -5.1% | -4.3% |  |  |  |
| Overall | -0.1% | -2.5% | -2.4% | 0.0% | -2.1% | -2.4% |  |  |  |
|  | -0.1% | -2.5% | -2.4% | 0.0% | -2.2% | -2.4% |  |  |  |
| Class F | 0.0% | -1.6% | -1.6% | 0.0% | -1.0% | -1.0% |  |  |  |
| Enc Time[%] | 95% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.5% | -1.4% | 0.0% | -1.3% | -2.7% | -0.1% | -5.3% | -4.6% |
| Class B | -0.3% | -0.4% | -0.7% | -0.1% | -2.5% | -3.4% | -0.2% | -0.8% | -0.9% |
| Class C | -0.2% | 0.7% | 0.7% | 0.0% | -0.7% | -1.1% |  |  |  |
| Class D | -0.2% | 1.1% | 1.2% | -0.1% | -0.3% | -0.7% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | 0.3% | 0.1% | 0.0% | -1.3% | -2.0% | -0.2% | -2.8% | -2.5% |
|  | -0.2% | 0.3% | 0.1% | -0.1% | -1.3% | -2.0% | -0.2% | -2.8% | -2.5% |
| Class F | -0.2% | -0.9% | -0.9% | 0.1% | -1.2% | -0.9% |  |  |  |
| Enc Time[%] | 93% |  |  | 96% |  |  | 88% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.3% | 4.5% | 4.9% | -0.2% | -1.9% | -3.2% |  |  |  |
| Class C | -0.3% | 3.0% | 3.2% | -0.1% | -0.5% | -1.1% |  |  |  |
| Class D | -0.3% | 4.6% | 3.9% | -0.2% | 0.8% | -1.0% |  |  |  |
| Class E | -0.3% | -0.1% | 0.9% | 0.0% | -6.1% | -5.3% |  |  |  |
| Overall | -0.3% | 3.3% | 3.5% | -0.1% | -1.6% | -2.5% |  |  |  |
|  | -0.3% | 3.3% | 3.4% | -0.1% | -1.6% | -2.5% |  |  |  |
| Class F | -0.4% | 0.4% | 0.2% | 0.0% | -2.2% | -2.0% |  |  |  |
| Enc Time[%] | 93% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |  |  |  |

# Conclusions

This contribution reports the result of CE2 Subtest 4.3. The results show there is 2.1%-2.5% chroma BD-rate reduction in all intra coding cases.

# References

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).