| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0540 |
| --- | --- |

| Title: | CE1 subtest B4 – On CABAC Init IDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1 subtest B4. The test considers a modification of the cabac_init_idc indicator that is associated with CABAC initialization based on the proposal JCTVC-G716.  The proposed approach allows an inter-predicted slice to choose between a P/B slice table for CABAC initialization.  It is also asserted that it clarifies the HEVC design and simplifies the design relative to H.264/AVC.

# Introduction

JCTVC-G716 proposed three changes and/or clarifications on the cabac_init_idc syntax element used forCABAC context initialization:

- Maintain the number of tables for CABAC initialization to three.  This is consistent with the HM software and reduces the number of tables from 7 to 3 compared to H.264/AVC

- Define cabac_init_idc to be a one-bit flag to allow for P slice types to be initialized with either the “P slice type” table or “B slice type” table.  Similarly, to allow for B slice types to be initialized with either the “P slice type” table or “B slice type” table.  (Optionally, rename the syntax element to cabac_init_flag.)

- Signal the presence of cabac_init_idc in the PPS

A primary benefit of the proposed method is simplification and clarification of the cabac_init_idc indicator, including reducing the number of tables stored at the decoder.  However, in addition to this primary benefit, we also observe coding efficiency improvements relative to the HM.  These improvements appear at lower rates or smaller slice sizes, where the entropy coder initialization is most critical.

# Results

The performance of the proposed approach was studied under experimental conditions specified in JCTVC-G1201[5]. The proposed method was integrated in HM 5.0 [1] and the source code was distributed to CE1 participants. All sequences were coded using the configuration described in JCTVC-G1201 for QP=17, 22, 27, 32, 37, 42 with high efficiency and low complexity settings. Full results are available in the attached XLS sheets.

Compared to JCTVC-G716, two changes were made.  First, a process to initialize a forward predicted B-slice with the P-slice table was removed.  This was proposed in G716 but retracted after integration with HM5.0.  Second, the encoder control was modified to use a 2-pass approach.  Again, this change was made after integration with HM5.0.  Thus, the performance reported here provides a best case performance measurement.

The rate-distortion summary is as follows:

## Low QP Settings (QP = 17, 22, 27, 32)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class D | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class F | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |  |  |  |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class D | -0.2% | 0.0% | -0.2% | -0.2% | 0.2% | 0.0% |  |  |  |
| Class E | -0.1% | 0.1% | -0.5% | -0.3% | -0.2% | -0.1% |  |  |  |
| Overall | -0.1% | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% |  |  |  |
|  | -0.1% | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% |  |  |  |
| Class F | -0.3% | -0.4% | -0.4% | -0.1% | -0.2% | -0.4% |  |  |  |

## Common conditions QP Settings (QP = 22, 27, 32, 37)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class C | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| Class D | -0.2% | -0.3% | -0.4% | -0.2% | -0.2% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% |
|  | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class F | -0.3% | -0.3% | -0.3% | -0.1% | -0.2% | -0.1% |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | 0.0% |  |  |  |
| Class C | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | -0.3% |  |  |  |
| Class D | -0.2% | 0.1% | -0.1% | -0.2% | 0.3% | 0.1% |  |  |  |
| Class E | -0.2% | 0.6% | -0.5% | -0.3% | -0.6% | 0.0% |  |  |  |
| Overall | -0.2% | 0.1% | -0.2% | -0.2% | -0.1% | -0.1% |  |  |  |
|  | -0.2% | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% |  |  |  |
| Class F | -0.3% | -0.5% | -0.7% | -0.3% | -1.1% | -0.8% |  |  |  |

## High QP Settings (QP = 27, 32, 37, 42)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class B | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% |
| Class C | -0.1% | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% |  |  |  |
| Class D | -0.1% | -0.5% | -0.6% | -0.2% | -0.4% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |
| Class F | -0.3% | -0.3% | -0.3% | -0.2% | -0.3% | -0.2% |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.3% | -0.2% | -0.4% | -0.5% |  |  |  |
| Class C | -0.2% | -0.5% | -0.3% | -0.2% | 0.2% | -0.6% |  |  |  |
| Class D | -0.2% | -0.1% | -0.1% | -0.2% | 0.2% | 0.4% |  |  |  |
| Class E | -0.2% | 0.6% | -0.4% | -0.2% | -1.2% | 0.0% |  |  |  |
| Overall | -0.2% | -0.1% | -0.3% | -0.2% | -0.3% | -0.2% |  |  |  |
|  | -0.2% | -0.1% | -0.2% | -0.2% | -0.3% | -0.2% |  |  |  |
| Class F | -0.2% | -0.4% | -0.8% | -0.4% | -2.1% | -1.4% |  |  |  |

Since there is no change in context initialization for an intra-predicted slice the average BD bitrate for the all-intra case is 0.0%. For the remaining test configurations the average Y BD bitrate is between -0.1% and -0.2%.

Further experiments were conducted with the 1-pass encoder control implementation of JCTVC-G716 [3] integrated into HM-5.0 and distributed to CE1 participants. The BD bitrate for this 1-pass encoder control for low delay high efficiency common conditions [4] are:

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | 0.1% |
| Class E | 0.0% | 0.0% | -0.1% |
| Class F | -0.2% | -0.3% | -0.2% |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% |

The BD bitrate improvement for Class F sequences indicate that sequences outside of the test environment may stand to benefit from the proposed approach.

The 1-pass encoder control was further tested on sequences outside of the test set with 1500 byte slices (implemented in HM 5.1 since slices is broken in HM5.0) showing an average Y BD bitrate improvement of -0.2%

| Lowdelay B HE (QP=22,27,32,37) |  |  |  |  |  | Lowdelay B LC (QP=22,27,32,37) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QCIF | Y | U | V |  | QCIF | Y | U | V |
| akiyo | -0.7% | -1.9% | -1.7% |  | akiyo | -0.2% | -0.4% | -1.3% |
| bowing | -0.4% | 0.3% | 0.3% |  | bowing | 0.0% | -0.4% | 1.1% |
| carphone | 0.4% | 0.4% | 3.7% |  | carphone | -0.6% | -3.0% | -1.5% |
| coastguard | 0.3% | 0.4% | 6.1% |  | coastguard | 0.2% | -3.0% | 2.6% |
| container | -0.7% | 0.6% | 0.8% |  | container | 0.0% | 1.3% | 0.5% |
| football | 0.0% | 0.2% | -0.7% |  | football | 0.0% | -0.2% | -0.4% |
| foreman | -0.5% | -0.4% | -0.2% |  | foreman | -0.3% | -0.2% | -0.5% |
| mobile | -0.5% | -0.4% | -0.2% |  | mobile | -0.3% | -0.2% | -0.5% |
| motherdaughter | -0.5% | -1.1% | 2.1% |  | motherdaughter | 0.1% | -1.6% | -1.0% |
| news | -0.1% | -1.9% | -0.8% |  | news | -0.2% | 0.8% | 0.0% |
| silent | 0.4% | 0.6% | -0.5% |  | silent | -0.4% | -1.5% | -0.4% |
| stefan | 0.0% | -0.3% | 0.4% |  | stefan | -0.2% | -0.7% | -0.5% |
| table | -0.1% | -1.1% | -0.3% |  | table | -0.4% | 3.3% | -0.2% |
| AVERAGE | -0.2% | -0.4% | 0.7% |  | AVERAGE | -0.2% | -0.4% | -0.1% |

- Conclusion

We propose simplification of the CABAC initialization process and cabac_init_idc flag.  The cabac_init_idc is redefined to be a binary flag that allows selection of either the “P” or “B” table for all inter-predicted slices.  Furthermore, the proposal clarifies the HEVC design and reduce the number of context tables compared to H.264/AVC.  We propose adoption of the method into the HM.  Syntax, semantics and draft text are provided below.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] High efficiency test model (HM) version 5.0 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0/

[2] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[3] Kiran Misra, Andrew Segall, "On CABAC Init IDC” JCTVC-G716 7th Meeting: Geneva, CH, 21-30 November, 2011.

[4] Frank Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200 7th Meeting: Geneva, CH, 21-30 November, 2011.

[5] R. Joshi, E. Alshina, H. Sasai, H. Kirchhofer, J. Lainema (CE coordinators), “Description of Core Experiment 1 (CE1): Entropy coding investigation,” JCTVC-G1201 7th Meeting: Geneva, CH, 21-30 November, 2011.

#### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| cabac_init_idc_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| long_term_ref_pics_present_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| tile_info_present_flag | u(1) |
| if( tile_info_present_flag = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( cabac_init_idc_enabled_flag && slice_type != I) |  |
| cabac_init_idc | u(1) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| inherit_dbl_params_from_APS_flag | u(1) |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| byte_align( ) |  |
| alf_cu_control_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| 5_minus_max_num_merge_cand | ue(v) |
| for( i = 0; i < num_substreams_minus1 + 1; i++ ){ |  |
| substream_length_mode | u(2) |
| substream_length[i] | u(v) |
| } |  |
| } |  |

# Semantics

cabac_init_idc_enabled_flag equal to 0 specifies that cabac_init_idc is not present in the slice header.  When cabac_init_idc_enabled_flag is not present, it shall be inferred to be 0.

cabac_init_idc specifies the method for determining the initialisation table used in the initialisation process for context variables. When cabac_init_idc is not present, it shall be inferred to be 0.

# Text

In Table 9-7, the ctxIdxTable and the ctxIdx for which initialisation is needed for each of the slice types are listed. The referenced context index tables ctxIdxTable include the values of initValue needed for the initialisation.

The variable InitializationType in Table 9-7 is derived as follows.

- InitializatonType = slice_type

- If cabac_init_idc equal to 1 and slice_type equal to 1,

InitializationType = 1-InitializationType

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type InitializationType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I 2 | P 0 | B 1 |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Error: Reference source not found | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..31 | 32..63 | 64..95 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..23 | 24..47 | 48..71 |