| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0541 |
| --- | --- |

| Title: | Limiting dynamic range when using a quantization weighing matrix |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Louie Kerofsky, Andrew Segall 5750 Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 9451 lkerofsky@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

This contribution examines the dynamic range issues related to quantization weighting matrices identified in JVTVC-G1044.  This contribution reports that under extremely low weight and QP values, levels exceeding 16-bits may occur and that under extremely high weight and QP values intermediate values exceeding 32-bits may occur.  This contribution proposes limiting level values prior to dequantization to 15-bits for 4x4 block size and limiting levels to 16-bits for all other block sizes.  The proposed method reportedly allows a dequantization implementation with weighting matrix requiring 16-bit multiplies and intermediate values within 32-bits.

# Introduction

Support for quantization weighting matrices in the HEVC design was introduced in [1] and has been under development since that time.  Dynamic range analysis indicated at low values of weighting matrix, the range of coefficient values represented by 16-bit levels is less than the entire range of 16-bit coefficients [2]. As a result, an encoder encountering a large level value at low QP and weight values may generate a level value greater than 16-bits.  Dynamic range analysis also indicated the need for intermediate values exceeding 32-bits under worst case conditions of large level, low QP, and small block size [2].  The addition of a weighting matrix effectively increases the range of quantization values and hence increasing dynamic range in extreme cases.  The purpose of this proposal is to mitigate these extreme cases of level values exceeding 16-bits and intermediate values exceeding 32-bits during dequantization when using a weighting matrix. The proposal limits input to the dequantization process to 16-bits for transform block sizes other than 4x4, and limits input to the dequantization process to 15-bits for transform block size of 4x4.  This limit could be defined as a normative constraint on the bitstream or by normative clipping defined in the reconstruction process.

# Dequantization formula

We describe dequantization formula incorporating a weighting matrix.  We follow the notation of dequantization given in [3] modified to include a weighting matrix.

Definitions

B	= internal bit depth (as specified by InternalBitDepth in the common config files)

N 	= transform size

M	= log2(N)

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

W[i][j]	= 8-bit unsigned weighting matrix

Equations

coeffQ[i][j]       =  ((level[i][j]*W[i][j]*IQ[QP%6] << (QP/6)) + offset)>>(M-1+(B-8)+4),          offset = 1<<(M-2+(B-8)+4), with i=0…nW-1, j = 0..nH-1      (2)

coeffQ = min(32767,max(-32768,coeffQ))

# Analysis of extreme level values

For the design without quantization weighting matrix, level values in excess of 16-bits are not useful since they result in values following dequatzation which are clipped to 16-bits.  Thus no additional information is communcated despite use of higher magntude level values.  The level values which produce maximal coefficeints following dequantization depending on block size and QP were determined in [4].  The largest such level value of 13106 is less than 16-bits.  When a weigthing matrix is used, the level value giving largest 16-bit coefficient can be calculated by setting the output of dequantization to 32768, set the weight value W=1 and solving for the level.

MaxLevel = (32768<<(M+3))>>(QP/6)/(IQ[QP%6])

The value of MaxLevel versus QP with weighting value 1 is plotted for various block size parameters M below.  Observe that for small QP the maximum level exceeds the magnitude of a 16-signed bit limit i.e. 32768.  For these small QP values and weight value of 1, quantization of an extreme 16-bit coefficient gives a level exceeding 16-bits.  The nonweighting matrix case corresponds to W=16 which corresponds to W=1 and QP increased by 4*6 = 24. In this range, the Max Level does not exceed 16-bits as shown in prior analysis.

Figure 1 Max Level vs. QP (W=1)

In the case of small weighting matrix values, extreme levels could occur in acutal codec operation when large coefficients are quantized with low QP and low weighting matrix values.

Observation 1:  The range of level values may be exceed 16-bits in practice for low QP, low weighting values and large coefficient values.  Note without a weighting matrix such extreme levels carry no additional information following the clip after dequantization.

# Intermediate bit-width analysis

We assume the levels are limited to 16-bits and analyse the intermediate bit-width needed during dequantization.  In calculating the dequantization process, the product of three terms level, weight W, and IQ should be computed by first multiplying W and IQ to give a 15-bit unsigned or 16-bit signed value.  The level can then be multiplied by this value.  Since the level values are limited to 16-bits, a 16-bit multiplier is sufficient for dequantization.

The necessary internal bit-depth for dequantization is determined noting that the maximum left shift of 8 due to QP/6 occurs when the IQ values are only 6-bit i.e. QP=48, 49, 50, 51 where QP%6 = 0, 1, 2, 3 so IQ is at most 57.

At extreme bit-width, we have: level 16-bits, W 8-bits, IQ 6-bits, and QP/6=8; so the extreme bit-width is 16+8+6+8 = 38. Thus a straightforward implementation of dequantization uses a 16-bit multiplier and has 38-bit intermediates.

To avoid the need for 38-bits intermediates, a conditional test can be performed to avoid the extreme left shift followed by a right shift.

iShift = M-1+B-8+4;

offset = 1<<(iShift-QP/6-1);

If (iShift > QP/6)

coeffQ[i][j]       =  (level[i][j]*W[i][j]*IQ[QP%6] + offset)>>(iShift-QP/6);

else

coeffQ[i][j]       =  (level[i][j]*W[i][j]*IQ[QP%6] + offset) << (QP/6-iShift);

coeffQ = min(32767,max(-32768,coeffQ))

Denote the bit-width of the levels by L.  From the previous constraint L is at most 16.  Worst case bit-width analysis in the case of 8-bit source (B=8) notes that iShift = M+3.  At worst case, we have a level value with L-bits, W is 8-bits, IQ is 6-bits, and QP/6=8.  Calculation shows QP/6-iShift=5-M so the extreme bit-width is L+8+6+5-M = L+19-M.  When L is 16, the extreme bit-width ranges from 33 to 30 as M ranges from 2 to 5.  To avoid 32-bit overflow the level must be limited to 15 signed bits when M=2..  A plot of bit-width versus QP for maximum weight W=255 is shown for various blocks sizes below.

Figure 2 Bit-Width vs. QP for M=255

Observation 2:  With large weighting matrix values and level values, dequantization intermediates may exceed 32-bits for the 4x4 blocksize.

# Allowable coefficients

When quantization weighting matrices are used, extreme cases can result in level values exceeding 16-bits or dequantization intermediates exceeding 32-bits.  To provide normative operation with a 16-bit multiply and intermediates not exceeding 32-bits, the level values should be limited to 16-bits in all cases and additionally to 15-bits in the case of 4x4 blocks.  This ensures the input to the dequantization multiplier is 16-bits and that the internal bit-width does not exceed 32-bits.  A formula which encompasses this switching between 16-bits and 15-bits based on block size is given using the minimum function.  Let L=min(15,12+M) and clipping is done to the range (-2L,2L-1).  Alternately, the clipping limit for 4x4 blocks can be raised to 16-bits , L=15,  at low QP since 32-bit overflow occurs for 4x4 blocks only at QP/6=8.  The derivation of the limit L can be given by L = 1<<min(15,21+ log2(nS)-QP/6) in this case. This has the advantage of not clipping extreme levels for 4x4 blocks at low QP but the disadvantage of additional complexity in that the clipping level depends on QP in addition to the block size.

There are two ways to use this limit: normative clipping in the decoding process prior to dequantization or imposing a limit on allowed levels in a bitstream or defining. We propose two alternatives: normative clipping prior to the dequantization calculation and a normative limit on bitstream contents.  When using a quantization weightin matrix, these extreme conditions can result from extreme but possible operating points so the normative clipping solution would block these without having to rely on an encoder to detect and avoid this possible cases.  Although these limits are motivated by use with a quantization weighting matrix, these limits can be used without a quantization weighting matrix;  the limits are not achieved by typical forward encoding.

# Conclusions

A limit on the level values is proposed to enable dequantization with 16-bit multiplier and 32-bit intermediates when using a weighting matrix.  A 16-bit limit on levels is needed to ensure dequantization with a 16-bit multiplier even with flat quantization.  In the case of a weighting matrix, dequantization corresponding to M=2 needs to be restricted further to 15-bits to avoid 32-bit overflow of intermediates.

# References

- J. Tanaka, Y. Morigami, T. Suzuki, “Quantization Matrix for HEVC”, JCTVC-E073, Geneva, Switzerland, March 2011.

- M. Budagavi, L. Kerofsky “BoG report on Quantization”, JCTVC-G1044, Geneva, Switzerland, November 2011.

- A. Fuldseth, G. Bjøntegaard, M. Budagavi, V. Sze, JCTVC-G495, “CE10: Core transform design for HEVC”, JCTVC-G495, Geneva, Switzerland, November 2011.

- L. Kerofsky, A. Segall, K. Misra, E. Alshina, A. Alshina, JCTVC-G719, “CE10: Core transform design for HEVC”, JCTVC-G495, Geneva, Switzerland, November 2011.

# Patent rights declaration

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A

This annex provides proposed text changes to the working draft for the normative limitation of levels. Only paragraphs that need to be changed are included in this appendix.

### 8.5.3 Scaling process for transform coefficients

L = 1<<min(15,12+ log2(nS))

cij= Clip3(-L,L-1, cij)

Appendix B

This annex provides proposed text changes to the working draft for the normative limitation of levels. Only paragraphs that need to be changed are included in this appendix.

### 8.5.3 Scaling process for transform coefficients

A bitstream conforming to this Recommendation | International Standard shall not contain data that results in any element level cij that exceeds the range of integer values from (-2L, 2L-1) inclusive where L=min(15,12+ log2(nS)).