| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0542 |
| --- | --- |

| Title: | Non-CE2: Cross-verification of Separate RQT for Y,U and V from MediaTek (JCTVC-H0315) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the implementation and simulation results of JCTVC-H0315 which is proposed by MediaTek.

# Introduction

This contribution is a cross-verification report of MediaTek’s contribution JCTVC-H0315 Non-CE2: Separate RQT structure for Y, U and V components.

In JCTVC-H0315, separate RQT trees are employed for Y, U and V components, i.e., the transform split flags are independently transmitted for Y, U, V components.  The software provided by MTK has been reviewed and the implementation matches the description in H0315.

# Experimental results

The codebase is HM 5.0. The coding efficiency is evaluated under common conditions as specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time and decoding time are obtained from cluster.

Test results of RQT Depth=1  Test results of RQT Depth=1

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.02% | -2.43% | -3.41% | 0.01% | -1.51% | -2.42% |  |  |  |
| Class B | -0.11% | -3.09% | -3.34% | 0.02% | -2.53% | -3.24% |  |  |  |
| Class C | -0.05% | -1.27% | -1.47% | 0.02% | -1.20% | -1.46% |  |  |  |
| Class D | -0.02% | -0.75% | -0.81% | 0.01% | -0.72% | -0.70% |  |  |  |
| Class E | 0.02% | -5.67% | -4.94% | 0.05% | -5.00% | -4.26% |  |  |  |
| Overall | -0.04% | -2.52% | -2.64% | 0.02% | -2.13% | -2.36% |  |  |  |
|  | -0.05% | -2.54% | -2.64% | 0.02% | -2.16% | -2.38% |  |  |  |
| Class F | 0.03% | -1.70% | -1.88% | 0.02% | -1.02% | -1.04% |  |  |  |
| Enc Time[%] | 98.94% |  |  | 101.37% |  |  |  |  |  |
| Dec Time[%] | 101.13% |  |  | 98.89% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.10% | -1.98% | -3.25% | -0.01% | -1.41% | -2.59% | 0.01% | -7.25% | -7.54% |
| Class B | -0.08% | -2.67% | -2.93% | -0.02% | -2.40% | -3.12% | -0.03% | -2.97% | -3.23% |
| Class C | -0.03% | -0.87% | -0.97% | 0.01% | -0.66% | -0.77% |  |  |  |
| Class D | -0.03% | -0.09% | -0.03% | -0.01% | -0.03% | -0.42% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% | -1.41% | -1.68% | -0.01% | -1.17% | -1.70% | -0.01% | -4.87% | -5.14% |
|  | -0.05% | -1.43% | -1.68% | -0.01% | -1.19% | -1.74% | -0.01% | -4.86% | -5.08% |
| Class F | -0.06% | -1.51% | -1.72% | 0.06% | -1.24% | -0.89% |  |  |  |
| Enc Time[%] | 94.89% |  |  | 95.13% |  |  | 90.02% |  |  |
| Dec Time[%] | 102.94% |  |  | 105.32% |  |  | 100.30% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.09% | -1.68% | -1.59% | -0.10% | -1.90% | -2.25% |  |  |  |
| Class C | -0.06% | -0.18% | -0.42% | -0.04% | -0.05% | -0.50% |  |  |  |
| Class D | -0.10% | 1.20% | 0.48% | -0.07% | 1.43% | -0.12% |  |  |  |
| Class E | -0.09% | -5.63% | -4.76% | 0.06% | -5.67% | -5.11% |  |  |  |
| Overall | -0.08% | -1.32% | -1.37% | -0.05% | -1.31% | -1.82% |  |  |  |
|  | -0.08% | -1.26% | -1.37% | -0.05% | -1.36% | -1.83% |  |  |  |
| Class F | -0.20% | -1.51% | -1.65% | 0.13% | -1.76% | -1.62% |  |  |  |
| Enc Time[%] | 94.41% |  |  | 97.64% |  |  |  |  |  |
| Dec Time[%] | 102.40% |  |  | 100.33% |  |  |  |  |  |

Test results of RQT Depth=1I  Test results of RQT Depth=2

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.16% | -3.42% | -4.58% | 0.00% | -1.62% | -2.52% |  |  |  |
| Class B | 0.06% | -4.23% | -4.53% | 0.00% | -2.62% | -3.44% |  |  |  |
| Class C | 0.04% | -1.87% | -2.18% | 0.01% | -1.21% | -1.55% |  |  |  |
| Class D | 0.04% | -1.05% | -1.29% | 0.01% | -0.70% | -0.78% |  |  |  |
| Class E | 0.16% | -6.11% | -5.39% | 0.10% | -4.83% | -4.17% |  |  |  |
| Overall | 0.08% | -3.22% | -3.44% | 0.02% | -2.14% | -2.45% |  |  |  |
|  | 0.07% | -3.22% | -3.43% | 0.02% | -2.16% | -2.48% |  |  |  |
| Class F | 0.09% | -1.84% | -2.11% | 0.03% | -0.97% | -1.00% |  |  |  |
| Enc Time[%] | 101.27% |  |  | 105.01% |  |  |  |  |  |
| Dec Time[%] | 98.28% |  |  | 96.33% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.11% | -2.52% | -3.84% | -0.01% | -1.96% | -3.09% | 0.03% | -10.40% | -9.93% |
| Class B | -0.11% | -2.93% | -3.16% | -0.01% | -3.21% | -3.92% | -0.07% | -3.35% | -3.60% |
| Class C | -0.10% | -0.92% | -1.12% | -0.03% | -1.26% | -1.52% |  |  |  |
| Class D | -0.08% | -0.01% | -0.24% | -0.04% | -0.45% | -0.84% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.10% | -1.56% | -1.93% | -0.03% | -1.79% | -2.34% | -0.03% | -6.49% | -6.41% |
|  | -0.10% | -1.60% | -1.92% | -0.03% | -1.80% | -2.42% | -0.02% | -6.46% | -6.34% |
| Class F | 0.00% | -1.69% | -1.73% | 0.04% | -1.41% | -1.05% |  |  |  |
| Enc Time[%] | 95.93% |  |  | 96.03% |  |  | 89.94% |  |  |
| Dec Time[%] | 100.13% |  |  | 101.11% |  |  | 98.92% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.22% | -0.08% | 0.28% | -0.09% | -2.63% | -3.42% |  |  |  |
| Class C | -0.20% | 0.54% | 0.77% | -0.05% | -1.01% | -1.34% |  |  |  |
| Class D | -0.18% | 2.76% | 2.33% | -0.10% | 0.23% | -1.12% |  |  |  |
| Class E | -0.06% | -2.71% | -3.44% | 0.02% | -7.45% | -6.06% |  |  |  |
| Overall | -0.18% | 0.29% | 0.22% | -0.06% | -2.42% | -2.82% |  |  |  |
|  | -0.18% | 0.33% | 0.15% | -0.06% | -2.43% | -2.83% |  |  |  |
| Class F | -0.31% | -1.47% | -1.95% | 0.09% | -1.86% | -2.15% |  |  |  |
| Enc Time[%] | 96.46% |  |  | 100.23% |  |  |  |  |  |
| Dec Time[%] | 99.05% |  |  | 98.58% |  |  |  |  |  |

Test results of RQT Depth=1II  Test results of RQT Depth=3

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.19% | -3.55% | -4.71% | 0.00% | -1.66% | -2.56% |  |  |  |
| Class B | 0.08% | -4.44% | -4.72% | 0.00% | -2.66% | -3.49% |  |  |  |
| Class C | 0.05% | -1.90% | -2.25% | 0.01% | -1.21% | -1.56% |  |  |  |
| Class D | 0.05% | -1.12% | -1.30% | 0.02% | -0.71% | -0.77% |  |  |  |
| Class E | 0.15% | -6.15% | -5.39% | 0.10% | -4.83% | -4.18% |  |  |  |
| Overall | 0.09% | -3.33% | -3.52% | 0.02% | -2.15% | -2.47% |  |  |  |
|  | 0.09% | -3.32% | -3.51% | 0.02% | -2.18% | -2.50% |  |  |  |
| Class F | 0.10% | -1.91% | -2.16% | 0.03% | -0.88% | -1.01% |  |  |  |
| Enc Time[%] | 101.08% |  |  | 107.38% |  |  |  |  |  |
| Dec Time[%] | 101.63% |  |  | 97.38% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.09% | -2.80% | -3.78% | 0.01% | -2.04% | -3.24% | 0.09% | -12.42% | -12.40% |
| Class B | -0.10% | -3.31% | -3.44% | 0.00% | -3.34% | -4.06% | -0.06% | -3.56% | -3.77% |
| Class C | -0.12% | -1.00% | -1.32% | 0.03% | -1.31% | -1.63% |  |  |  |
| Class D | -0.08% | -0.10% | -0.28% | -0.03% | -0.51% | -0.95% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.10% | -1.77% | -2.08% | 0.00% | -1.87% | -2.47% | 0.01% | -7.50% | -7.60% |
|  | -0.10% | -1.79% | -2.07% | 0.00% | -1.89% | -2.53% | 0.01% | -7.45% | -7.51% |
| Class F | 0.00% | -1.73% | -1.84% | 0.02% | -1.44% | -1.16% |  |  |  |
| Enc Time[%] | 98.43% |  |  | 99.81% |  |  | 90.15% |  |  |
| Dec Time[%] | 101.58% |  |  | 102.91% |  |  | 97.25% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.18% | -0.25% | -0.09% | -0.07% | -2.89% | -3.18% |  |  |  |
| Class C | -0.17% | 0.24% | 0.58% | -0.02% | -0.97% | -1.42% |  |  |  |
| Class D | -0.19% | 2.44% | 1.53% | -0.08% | 0.54% | -1.12% |  |  |  |
| Class E | -0.13% | -1.79% | -2.84% | 0.02% | -8.41% | -4.61% |  |  |  |
| Overall | -0.17% | 0.26% | -0.03% | -0.04% | -2.59% | -2.50% |  |  |  |
|  | -0.17% | 0.31% | -0.07% | -0.04% | -2.59% | -2.48% |  |  |  |
| Class F | -0.17% | -1.50% | -1.45% | 0.13% | -2.25% | -1.69% |  |  |  |
| Enc Time[%] | 96.03% |  |  | 98.50% |  |  |  |  |  |
| Dec Time[%] | 99.85% |  |  | 100.95% |  |  |  |  |  |

# Conclusions

This contribution reports the cross-verification result of JCTVC-H0315. The BD-rate reported in JCTVC-H0315 is confirmed. Encoding/decoding time are comparable to that reported by proponents.

# References

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).