| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0543 |
| --- | --- |

| Title: | CE2: Cross verification of subtest 4.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo M. Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the implementation and simulation results of JCTVC-H0317 which is proposed by MediaTek.

# Introduction

RQT scheme is applied to select and perform transform in HEVC. With the current structure, the luma and chroma transform depths are synchronized. However, since chroma components tend to be more smooth than luma component, transform at a lower depth (compared to luma) may have some benefits on coding efficiency. CE2 has been established at the 7-th JCT-VC meeting to study this issue, and this contribution reports the cross-check result of CE2 subtest 4.1.

# Method in CE2 subtest 4.1

The following two parameters are introduced to define the maximum chroma depth for inter and intra respectively:

- max_transform_hierarchy_depth_inter_chroma

- max_transform_hierarchy_depth_intra_chroma.

The depth of chroma transform is derived as follows:

Inter case:

DepthChroma = min( DepthLuma, max_transform_hierarchy_depth_inter_chroma)

Intra case:

DepthChroma = min( DepthLuma, max_transform_hierarchy_depth_intra_chroma)

For intra case, there is a further adjustment of Depth Chroma as follows:

if DepthLuma = 0, and PU is not 2Nx2N, DepthChroma = 1;

The largest chroma transform size is still 16x16 as in HM5.0.

# Experimental results

The codebase is HM 5.0. According to CE2 description, the following settings are used:

- max_transform_hierarchy_depth_inter_chroma = 1

- max_transform_hierarchy_depth_intra_chroma = 1

The coding efficiency is evaluated under common conditions as specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time and decoding time are obtained from cluster.

Test results of CE2 4.1   Test results of CE2 4.1

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | -3.0% | -3.9% | 0.0% | -1.4% | -2.3% |  |  |  |
| Class B | 0.0% | -3.0% | -3.2% | 0.0% | -2.1% | -2.7% |  |  |  |
| Class C | 0.0% | -1.3% | -1.4% | 0.0% | -1.1% | -1.4% |  |  |  |
| Class D | 0.0% | -0.7% | -0.7% | 0.0% | -0.7% | -0.7% |  |  |  |
| Class E | 0.1% | -5.0% | -4.3% | 0.0% | -4.1% | -3.5% |  |  |  |
| Overall | 0.0% | -2.4% | -2.5% | 0.0% | -1.8% | -2.0% |  |  |  |
|  | 0.0% | -2.4% | -2.5% | 0.0% | -1.8% | -2.1% |  |  |  |
| Class F | 0.1% | -1.7% | -1.6% | 0.0% | -0.8% | -0.8% |  |  |  |
| Enc Time[%] | 100% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -2.1% | -3.1% | 0.0% | -1.4% | -2.5% | 0.0% | -3.3% | -2.6% |
| Class B | 0.0% | -2.1% | -2.2% | 0.0% | -1.9% | -2.5% | 0.0% | -2.2% | -2.4% |
| Class C | -0.1% | -0.7% | -0.8% | 0.0% | -0.7% | -1.0% |  |  |  |
| Class D | -0.1% | -0.4% | -0.1% | 0.0% | -0.4% | -0.9% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -1.2% | -1.4% | 0.0% | -1.1% | -1.7% | 0.0% | -2.7% | -2.5% |
|  | -0.1% | -1.2% | -1.4% | 0.0% | -1.1% | -1.7% | 0.0% | -2.7% | -2.4% |
| Class F | 0.0% | -1.2% | -1.2% | 0.0% | -0.8% | -0.7% |  |  |  |
| Enc Time[%] | 96% |  |  | 97% |  |  | 92% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | -0.7% | -1.3% |  |  |  |
| Class C | -0.1% | 0.2% | 0.1% | 0.0% | 0.0% | -0.4% |  |  |  |
| Class D | -0.1% | 0.9% | 0.2% | -0.1% | 0.3% | -0.5% |  |  |  |
| Class E | 0.0% | -1.7% | -0.5% | 0.0% | -3.4% | -2.4% |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | -0.8% | -1.1% |  |  |  |
|  | -0.1% | 0.0% | -0.1% | 0.0% | -0.8% | -1.1% |  |  |  |
| Class F | -0.2% | -0.6% | -0.8% | 0.1% | -0.6% | -0.6% |  |  |  |
| Enc Time[%] | 96% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 96% |  |  |  |  |  |

# Conclusions

This contribution reports the cross-verification result of CE2 Subtest 4.1. Compared to the proponents results, the BD-rate is exactly the same and the encoding/decoding time are similar.

# References

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).