| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 |  |  | Document: JCTVC-H0544 WG11 Number: m23425 |
| --- | --- | --- | --- |
| Title: | CE6.a: New modes (LML and LMA) for chroma intra prediction |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Oscar C. AU Jingjing Dai Xing Wen Feng Zou Chao Pang | Email: | eexyzhang@ust.hk eeau@ust.hk jjdai@ust.hk wxxab@ust.hk fengzou@ust.hk pcece@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution reports the test results of two new luma-based chroma intra prediction modes, i.e. LML and LMA. LML and LMA are quite similar to existing LM mode, except that the neighbors used to derive parameters alpha and beta are from different locations. LML uses the left and down left 2N neighboring pixels for an NxN chroma block, while LMA uses the above and above right 2N neighboring pixels. Compared to HM5.0, the average BD-bitrate gain is 0.1%, 2.2%, 2.2% and 0.5 for all intra HE configuration for Y, U, V components and YUV respectively. This contribution is included in the CE6.a “Intra Chroma Prediction”.

# Introduction

## LM mode

The LM mode predicts chroma samples of the current block based on corresponding reconstructed luma samples in a linear relationship.

(1)

The parametersand in equation (1) are derived from its neighbors, as Fig. 1 shows, so that the decoder can drive the same value of and without explicit indication.

Fig. 1 neighboring samples used for parameters derivation in LM mode

## LML and LMA mode

When LML mode is used, the chroma samples are predicted from the downsampled reconstructed luma samples in the same block according to equation (1). The parametersand in equation (1) are derived from its neighborhoods, as Fig 2 shows. For an NxN chroma block, only 2N samples from left and down left neighboring locations are used as the training set. The calculation ofandare the same as LM mode.

Fig. 2 neighboring samples used for parameters derivation in LML mode (Left template)

Similarly, when LMA mode is used, only above and above right neighboring samples are used to derive parameter and, as Fig 3 shows. For an NxN chroma block, 2N samples are used as the training set. The calculation procedures of andare the same as LM mode.

Fig. 3 neighboring samples used for parameters derivation in LMA mode (Above template)

# Implementation and Experimental Results

LML and LMA are added as two independent modes in HM5.0. It is placed after DM mode and LM mode. Tables 1-2 show the coding efficiency results under the JCT-VC common test conditions for Intra HE and Intra HE10bit configurations. The BD bitrate of YUV is generated as the formula specified in JCTVC-F386.

Table 1. Performance on HM5.0 reference software for mandatory test

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | YUV |
| Class A (8bit) | -0.1% | -2.0% | -1.6% | -0.4% |
| Class B | -0.1% | -2.6% | -2.1% | -0.5% |
| Class C | -0.2% | -2.3% | -2.9% | -0.6% |
| Class D | -0.1% | -2.1% | -2.3% | -0.5% |
| Class E | 0.0% | -1.7% | -1.8% | -0.3% |
| Overall | -0.11% | -2.22% | -2.21% | -0.47% |
|  | -0.1% | -2.2% | -2.2% | -0.5% |
| Class F | -0.4% | -2.5% | -3.0% | -0.8% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 2. Performance on HM5.0 reference software including optional test

|  | All Intra HE |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.2% | -9.0% | -9.3% | -1.4% | -0.2% | -9.0% | -9.7% | -1.4% |
| Class B | -0.1% | -2.6% | -2.1% | -0.5% | -0.1% | -2.8% | -2.2% | -0.5% |
| Class C | -0.2% | -2.3% | -2.9% | -0.6% | -0.2% | -2.4% | -3.0% | -0.6% |
| Class D | -0.1% | -2.1% | -2.3% | -0.5% | -0.1% | -2.2% | -2.4% | -0.5% |
| Class E | 0.0% | -1.7% | -1.8% | -0.3% | 0.0% | -1.8% | -1.9% | -0.3% |
| Class F | -0.4% | -2.5% | -3.0% | -0.8% | -0.4% | -2.5% | -3.3% | -0.8% |
| Overall | -0.2% | -3.4% | -3.6% | -0.7% | -0.2% | -3.5% | -3.7% | -0.7% |
|  | -0.2% | -3.4% | -3.5% | -0.7% | -0.2% | -3.5% | -3.7% | -0.7% |
| Enc Time[%] | 104% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

# Conclusion

This contribution reports the test results of two new luma-based chroma intra prediction modes, i.e. LML and LMA, based on HM5.0. Compared to HM5.0, the average BD-bitrate gain is 0.1%, 2.2%, 2.2% and 0.5 for all intra HE configuration for Y, U, V components and YUV respectively. Note that adding the proposed modes does not add computation complexity on the decoder side. No new syntax is introduced.  It’s recommended to adopt LML and LMA to HM6.0.

# References

- X. Zhang, O. C. Au, J. Dai, F. Zou, C. Pang, X. Wen, “New modes for chroma intra prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G358, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- J. Chen, V. Seregin, S. Lee, W. Han, J. Kim, B.M. Jeon, “CE6.a: Chroma intra prediction by reconstructed luma samples”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11,  JCTVC-E266, Geneva, March, 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E900, 6th Meeting: Torino, IT, July 2011.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)

# WD changes (based on JCTVC-G1103-d6)

### 8.3.1 Derivation process for luma intra prediction mode

……

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Vertical |
| 1 | Intra_Horizontal |
| 2 | Intra_DC |
| Otherwise (3..33) | Intra_Angular |
| 34 | Intra_Planar |
| 35 | Intra_FromLuma (used only for chroma) |
| 37 | Intra_FromLumaLeftTemplate (used only for chroma) |
| 38 | Intra_FromLumaAboveTemplate (used only for chroma) |

……

### Derivation process for chroma intra prediction mode

Table 8-2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 0 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | 1 | 7 | 0 | 0 | 1 |
| 2 | 2 | 2 | 7 | 1 | 2 |
| 3 | 3 | 3 | 3 | 7 | 3 |
| 4 | LM | LM | LM | LM | LM |
| 5 | LML | LML | LML | LML | LML |
| 6 | LMA | LMA | LMA | LMA | LMA |
| 7 | 0 | 1 | 2 | 3 | X |

### ………..

##### Specification of Intra_FromLumaLeftTemplate prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples p[ x, y ], with x= -1, y = 0..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 37.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-1)

pY’[ x, y ] = ( PLM [ 2x, 2y ] + PLM [ 2x, 2y+1 ] ) >> 1, with x= -1, y = 0..2*nS-1

pY’[ x, y ] = ( PLM [ 2x, 2y ] + PLM [ 2x, 2y+1 ] ) >> 1, with x= 0..nS-1, y = 0..nS-1	(8-2)

- Variables L, C, LL, LC and k2 are derived as follows:

L =			(8-3)

C =			(8-4)

LL =			(8-5)

LC =			(8-6)

k2 = log2( (2*nS) >> k3 )		(8-7)

……

##### Specification of Intra_FromLumaAboveTemplate prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples p[ x, y ], with x= 0..2*nS-1, y = -1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 38.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-8)

pY’[ x, y ] = ( PLM [ 2x-1, 2y+1 ] + 2* PLM [ 2x, 2y+1 ] + PLM [ 2x+1, 2y+1 ] + 2 ) >> 2, with x=0..2*nS-1, y = -1		(8-9)

pY’[ x, y ] = ( PLM [ 2x, 2y ] + PLM [ 2x, 2y+1 ] ) >> 1, with x= 0..nS-1, y = 0..nS-1	(8-10)

- Variables L, C, LL, LC and k2 are derived as follows:

L = 		(8-11)

C = 		(8-12)

LL = 		(8-13)

LC =			(8-14)

k2 = log2( (2*nS) >> k3 )		(8-15)

……