| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0546 |
| --- | --- |

| Title: | On weighted prediction precision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a reduced precision weighted prediction scheme with negligible impact on coding efficiency. In the current design, input to the weighting module of weighted prediction scheme has 16-bit precision. When input precision to the weighting module is decreased by 4 bits there is no drop in performance.  Simulation results show 0.0%, 0.0%, 0.0% and 0.0% BD rate change for RA_HE, RA_LC, LB_HE and LB_LC cases, respectively.

# Introduction

In current HM 5.0, inputs to the weighted prediction module, predSamplesLX[x,y], are 16-bit values. If the precision of the input values to the weighted prediction module are decreased by 4 bits there is negligible drop in performance. The expressions for bi-pred and uni-pred weighted prediction operations would effectively become:

Bi-prediction:

predSamples[ x, y ] = Clip1H( ( (( ((predSamplesL0 [ x, y ]+8)>>4) * w0C +

( ((predSamplesL1 [ x, y ]+8)>>4) *  w1C + 2logWDC - 4 ) >> ( logWDC + 1 - 4 ) ) + ( ( o0C + o1C + 1 ) >> 1 ) )

Uni-prediction:

predSamples[ x, y ] = Clip1H( ( ( ((predSamplesLX [ x, y ]+8)>>4) * wXC + 2logWDC – 1- 4 ) >> (logWDC - 4)) + oXC )

All the derivations for the original predSamplesLX[x,y] are not modified.

# Results

The proposed weighted prediction precision reduction is implemented in HM5.0. The simulations are conducted based on the JCT-VC common test condition defined in [1] using fade to black and fade to white sequences. Proposed changes affect RA_HE, RA_LC, LB_HE, LB_LC, and RA_HE10 configurations. Table 1 shows summary of simulations for fade to black sequences and Table 2 shows summary of simulations for fade-to-white sequences with explicit weighted prediction. As can be from the results on average there is 0.0% BD rate loss.

Table 1: Summary of simulation results for proposed algorithm on  fade-to-black sequences with explicit weighted prediction.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.3% | 0.1% | 0.0% | -0.3% | -0.1% | 0.0% | 0.3% | 0.2% |
| Class B | 0.1% | -0.2% | -0.4% | 0.0% | -0.1% | -1.2% | 0.0% | 0.3% | 0.5% |
| Class C | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | 0.3% |  |  |  |
| Class D | 0.0% | -0.9% | 0.0% | 0.0% | -0.1% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | 0.3% | 0.4% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.3% | 0.0% | 0.3% | 0.5% |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.8% | 0.1% | 0.1% | -2.9% |  |  |  |
| Class C | 0.0% | 0.0% | -0.3% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class D | -0.1% | -0.7% | -0.8% | -0.1% | -1.0% | -0.1% |  |  |  |
| Class E | -0.1% | 1.0% | -1.0% | 0.2% | -1.3% | -1.0% |  |  |  |
| Overall | 0.0% | -0.1% | -0.7% | 0.0% | -0.5% | -1.2% |  |  |  |
|  | 0.0% | 0.0% | -0.7% | 0.0% | -0.4% | -1.2% |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 102% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |  |  |  |

Table 2: Summary of simulation results for proposed algorithm on fade-to-white sequences with explicit weighted prediction.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.3% | 0.0% | 0.1% | -0.2% | 0.0% | 0.1% | 0.3% |
| Class B | 0.0% | -0.1% | 0.1% | 0.1% | -0.1% | -1.0% | 0.0% | 0.3% | 0.3% |
| Class C | 0.0% | -0.1% | -0.4% | 0.1% | -0.2% | -0.1% |  |  |  |
| Class D | 0.2% | -0.5% | 0.0% | 0.0% | -0.1% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.3% | 0.0% | 0.2% | 0.3% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% | 0.2% | 0.3% |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -1.0% | 0.0% | 0.7% | -3.2% |  |  |  |
| Class C | 0.1% | -0.1% | 0.3% | 0.1% | -0.3% | -0.2% |  |  |  |
| Class D | 0.1% | -0.1% | -0.3% | 0.0% | -0.7% | -0.8% |  |  |  |
| Class E | 0.2% | 0.7% | 0.2% | 0.1% | 0.0% | -2.0% |  |  |  |
| Overall | 0.1% | 0.0% | -0.3% | 0.1% | 0.0% | -1.6% |  |  |  |
|  | 0.1% | 0.0% | -0.1% | 0.1% | -0.1% | -1.6% |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 101% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |  |  |  |

# Conclusions

This contribution has presented reduced precision weighted prediction. When the input data precision is reduced by 4 bits, there is negligible impact on coding efficiency.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-G1200, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).