| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0547-r2 |
| --- | --- |

| Title: | Significance Map Contexts for Small Block Units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document describes implementation details and reports the results of Motorola Mobility’s significance map context reduction for small TU. Cross-check will be provided by Huawei.

# Introduction

In current HEVC WD [1], the significant bits of small block units are using fixed contexts according to its physical locations.  For 4x4 blocks, luma significant map utilizes 9 contexts while the chroma significant map uses 6 contexts, as Fig. 1 shows.  For 8x8 blocks, luma and chroma are using 10 contexts for AC components and 1 additional context for DC component, as shown in Fig. 2. There are total 20 (9 for 4x4 and 11 for 8x8) contexts are required to code significance map of luma small block units and 17 (6 for 4x4 and 11 for 8x8) contexts are required to code significance map of chroma block units.

4x4-Luma:

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 4 | 5 | 2 | 3 |
| 6 | 6 | 7 | 7 |
| 8 | 8 | 7 | x |

4x4-Chroma:

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 1 | 1 | 2 | 3 |
| 4 | 4 | 5 | 5 |
| 3 | 3 | 5 | x |

Fig. 1. HM5.0 significance map context index assignment of 4x4 blocks

8x8-Luma/Chroma:

| 10 | 0 | 1 | 1 | 2 | 2 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 2 | 2 | 3 | 3 |
| 4 | 4 | 5 | 5 | 6 | 6 | 3 | 3 |
| 4 | 4 | 5 | 5 | 6 | 6 | 3 | 3 |
| 8 | 8 | 6 | 6 | 6 | 6 | 7 | 7 |
| 8 | 8 | 6 | 6 | 6 | 6 | 7 | 7 |
| 9 | 9 | 9 | 9 | 7 | 7 | 7 | 7 |
| 9 | 9 | 9 | 9 | 7 | 7 | 7 | x |

Fig. 2. HM5.0 significance map context index assignment of 8x8 blocks

# Proposal

In this proposal, we aim on further reducing the contexts by exploring symmetric properties and sharing contexts between luma and chroma.

Specifically, we propose to include symmetric property among some components within significance map. The symmetric property impacts components in the symmetric positions with respect to the diagonal line of a block. That is, the components at (a,b) and (b,a) in (x,y) coordinate domain are in symmetric locations.

4x4-Luma:

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 4 | 5 | 2 | 3 |
| 2 | 2 | 6 | 6 |
| 3 | 3 | 6 | x |

4x4-Chroma:

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 1 | 1 | 2 | 3 |
| 2 | 2 | 4 | 4 |
| 3 | 3 | 4 | x |

Fig. 3. Proposed significance map context assignment of 4x4 blocks

8x8-Luma/Chroma:

| 10 | 0 | 1 | 1 | 2 | 2 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 2 | 2 | 3 | 3 |
| 4 | 4 | 5 | 5 | 6 | 6 | 3 | 3 |
| 4 | 4 | 5 | 5 | 6 | 6 | 3 | 3 |
| 2 | 2 | 6 | 6 | 6 | 6 | 7 | 7 |
| 2 | 2 | 6 | 6 | 6 | 6 | 7 | 7 |
| 3 | 3 | 3 | 3 | 7 | 7 | 7 | 7 |
| 3 | 3 | 3 | 3 | 7 | 7 | 7 | x |

Fig. 4. HM5.0 significance map context index assignment of 8x8 blocks

In order to further reduce the number of contexts for significance map coding, the proposed context map may be used for both luma and chroma components.

Since the intra pictures generally produce more residual coefficients and result in more sensitive context statistics of its significance map, we may further limit the context sharing between luma and chroma for P and B slices only to keep the small impact on performance.

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-H547.xls).

Table 1 Simulation results

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class C | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class E | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
|  | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| Class C | 0.1% | -0.1% | 0.0% | 0.1% | 0.2% | -0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.5% | 0.1% | 0.6% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.2% | 0.1% | 0.2% | -0.1% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.2% | 0.1% | 0.2% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 55% |  |  | 81% |  |  | 80% |  |  |
| Dec Time[%] | 84% |  |  | 84% |  |  | 80% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.3% | 0.3% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.1% |  |  |  |
| Class D | 0.1% | -0.2% | 0.1% | 0.0% | 0.6% | -0.4% |  |  |  |
| Class E | 0.0% | -0.4% | -0.7% | 0.1% | -1.0% | -0.5% |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | -0.1% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | 0.2% | -0.4% | 0.5% |  |  |  |
| Enc Time[%] | 80% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 89% |  |  | 100% |  |  |  |  |  |

# Conclusions

This contribution document proposes to reduce contexts for coding of significance map for small block units. The experimental results show that there is a minor loss in both intra and inter conditions.

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” JCTVC-G1103.

# Patent Rights Declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).