| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0548-r2 |
| --- | --- |

| Title: | Implicit signaling of collocated picture for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Limin Wang | Tel: Email: | 858-404-2251 yyu@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

In the current working draft of HEVC, a flag, collocated_from_l0_flag, is used to explicitly indicate the collocated picture is a reference picture specified in either list0 or list1. In this proposal, we propose to remove collocated_from_l0_flag for low delay case because this flag is redundant. Furthermore, we propose to add a flag at sequence level to indicate that only one picture motion information is kept for collocated picture purpose instead of keeping multiple pictures in the current working draft. This way may save memory for low delay application without any performance impact if multiple reference pictures are used. Simulation results show that there is no any PSNR change for every slice while we save N-1 bits where N is number of pictures.

# Introduction

A flag, collocated_from_l0_flag, is used to explicitly indicate the collocated picture is a reference picture specified in either list0 or list1 in the current working draft [1]. It may help to provide better coding perfomance for random access. However, this flag is redundant for low delay case because both List0 and List 1 may be same for every slice.

If multiple reference pictures are used, all motion information for all reference pictures must be stored in decoder according to the current working draft. In the hardware implementation, all of motion information may be stored in DRAM and hence very costly. Therefore, it is better to compress motion information as much as possible.

# Proposed Solution

For low delay case, we propose to remove collocated_from_l0_flag. The collocated picture is defined as the first entry of reference list 0. Furthermore, we propose to add a flag, update_collocated_picture_idx_SPS, at sequence level to indicate that only one picture motion information is kept for collocated picture purpose even multiple reference pictures are used.  The proposed syntax changes are as follows.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| max_num_ref_frames | ue(v) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| update_collocated_picture_idx_SPS | u(1) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if ( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if ( !uniform_spacing_flag ) { |  |
| for ( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for ( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| inherit_dbl_params_from_APS_flag | u(1) |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( slice_type = = B && !update_collocated_picture_idx_SPS) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| byte_align( ) |  |
| alf_cu_control_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| 5_minus_max_num_merge_cand | ue(v) |
| for( i = 0; i < num_substreams_minus1 + 1; i++ ){ |  |
| substream_length_mode | u(2) |
| substream_length[i] | u(v) |
| } |  |
| } |  |

If update_collcated_picture_idx_sps is 1, there is only one picture buffer size to store motion information for collocated picture purpose, all the coded picture, except I picture, will have its MV to replace the previously stored MVs, and  collocated_from_l0_flag is no longer used. If update_collcated_picture_idx_sps is 0, the picture buffer size and collocated_from_l0_flag are kept unchanged.

# Simulation results

The proposed method is implemented on HM 5.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM5.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate.

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 85% |  |  | 85% |  |  |
| Dec Time[%] | 84% |  |  | 87% |  |  |

Table 3.1 BD-rate saving of the proposed algorithm relative to HM5.0.

From the attached spreadsheet, we can find that there is no any PSNR change for every slice. At the same time, we save N-1 bits where N is number of pictures.

# Conclusions

This contribution proposes to remove collocated_from_l0_flag for low delay case to reduce the syntax redunancy. Furthermore, this contribution proposes to add a flag at sequence level to indicate that only one picture motion information is kept for collocated picture purpose instead of keeping multiple pictures as in the current working draft. This way may save memory for low delay application without any performance impact if multiple reference pictures are used.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1103_d4, Jan. 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, Dec, 2011.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).