| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0549 |
| --- | --- |

| Title: | Enabling Sub-LCU level parallel decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sanjeev Kumar, Hsiao-Chiang Chuang, Shu Xiao, Xianglin Wang, Wei-jung Chien, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Marta Karczewicz +1-858-658-5673 {sanjeevk,hchuang,sxiao,xianglin,wchien,martak}@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In HEVC (HM5.0), parallel decoding options include parallel decoding of different LCU rows. A more fine grained parallelism is not possible due to dependencies among blocks inside an LCU. E.g. it is not possible to split an LCU row into a top half and a bottom half for parallel decoding. This contribution identifies the minimal set of dependency (in intra prediction) preventing the feasibility of aforementioned granularity of parallelism (assuming parsing and MV prediction can be done ahead of time).  Simulation results show that breaking this minimal set of dependency does not have obvious impact on coding performance. Practical advantages include better performance of any cache subsystem, arising from reduced distance between different in-flight processing units and the consequent improved locality of reference.

# Introduction

In HEVC (HM5.0), a decoder can do either frame based decoding or LCU based decoding.  For software implementation, a frame based decoding is preferred. For hardware implementation, an LCU based decoding is preferred. For 64x64 LCU and 32x32 maximum transform sizes, 32x32 Sub-LCU based decoding is also possible. Smaller processing unit sizes are more desirable in hardware.

HEVC has several tools to facilitate parallel encoding and decoding, such as Tiles and Wavefront. These tools can parallelize entropy coding as well as rest of the processing.  From decoder’s perspective, different rows of LCU can be decoded in parallel by different decode engines. These different decode engines can be different CPU cores in a multi-core system or different hardware units in a hardware implementation. In order to meet dependencies, there is a delay of two LCUs between decoding processes for different LCU rows. This large granularity (different LCU row) of parallelism, along with large delay between decoding of vertically adjacent LCUs (2 LCU delay in wavefront) can have an adverse impact on the performance of any cache (even more so for 4x and above parallelism), due to reduced locality of references.  In practice, this can significantly nullify the advantages of parallelization from wavefront etc.

In this contribution, we propose to enable a more fine grained parallel decoding, in which for an LCU size of 64x64, an LCU row can be split into two halves (upper and lower) and for much of the decoding process, those two halves can be processed in parallel. In particular, after parsing and motion vector prediction, rest of the decoding is parallelized for top half and bottom half.  In current standard, such a parallelization is not possible due to intra-prediction dependencies among blocks inside an LCU. This contribution identifies the minimal set of dependencies which are preventing aforementioned granularity of parallelism, and proposes to break those dependencies by selectively marking corresponding pixels unavailable during intra prediction of the relevant TU.

As opposed to wavefront decoding of two LCU rows in parallel, this results in reduced distance between different in-flight processing units being decoded simultaneously.  This is very beneficial for performance of the cache from improved locality of reference, especially when fetching parts of reference frame for motion compensation. It also has other advantages from hardware implementation perspective. However, cache performance related advantage, by itself, is very significant and is applicable to software as well as hardware implementations of an HEVC decoder.

# Analysis of processing unit size and parallel decode delay

In this section we consider two choices of processing unit size for an LCU size of 64x64. First, we consider the case when an entire 64x64 LCU is processed as a unit. In this case, delay between consecutive parallel decoders is 2 LCU. This is similar to the wavefront decoding. In second case, we consider the case of 32x32 processing units. Although processing unit size is 32x32, different parallel decoders are still forced to work on different LCU rows (similar to the first case). However, delay between different parallel decoders is slightly improved to 1.5 LCU as opposed to 2 LCU delay in the first case.

These two or any other alternative schemes of parallel decoding suffer from the disadvantage of larger horizontal as well as vertical distances between in-flight processing units and attendant disadvantage of inefficient cache usage resulting from reduced locality of reference, especially when fetching parts of a reference frame for motion compensation.

## 64x64 LCU based decoding

Figure 1: A part of picture showing 2 LCUs ( First LCU: T1,2,3,4, Second LCU: T5,6,7,8) in one LCU row and 1 LCU (B1,2,3,4) in the next LCU row.

Figure 2: LCU basis decoding possibility in current HEVC standard (HM 5.0). It has a delay of 2 LCU between consecutive parallel decoders, in order to meet dependencies. Some dependencies have been shown by yellow arrows. This situation is similar to wavefront decoding in the current HEVC standard.

## 32x32 Sub-LCU based decoding with a LCU size of 64x64

Figure 3: Sub-LCU (32x32 processing unit) based decoding possibility in current HEVC standard (HM 5.0). It has a delay of 1.5 LCU between consecutive parallel decoders, in order to meet dependencies. Some dependencies have been shown by yellow arrows. Different parallel decoders still have to work on different LCU rows.

# Proposed Sub-LCU level decode parallelization

We propose to enable a finer grained parallelism (Figure 4) by breaking intra-prediction dependency based on the LCU coding order. In the figure, yellow arrows indicate dependencies from current HEVC standard which are not affected by proposed contribution. Red arrow indicates dependency from current HEVC standard, which proposed contribution intends to get rid of.

Figure 4: Sub-LCU (32x32 processing unit) based parallel decoding

Based on the proposal, intra-prediction dependency from T4 to T5 (red arrow in figure 4) is removed. In particular,  while doing intra prediction for T5 (actually only bottom left corner TU in T5, which could span whole of T5 in case of 32x32 TU), pixels belonging to T4 are marked “unavailable for intra prediction”, and corresponding pixels are filled using padding. In every 64x64 LCU, this affects only one TU, which satisfies the condition (tuOffsetX == 0 && ((tuOffsetY + tuHeight)==32) ). Here, offsets are with respect to LCU. Furthermore, only in the case of planar and horizontal positive slope angular intra-prediction modes for that one TU per LCU, does the proposed technique differ in its behavior from HM 5.0.

With this scheme, it has a delay of 1 LCU between consecutive decoders. Furthermore, two decoders can work in parallel on top and bottom halves of an LCU row. In every LCU, only one TU (bottom left TU in T5 and its counterparts in other LCUs) is affected by this dependency breaking.

# Proposed text change to working draft

(Red color indicates newly added text corresponding to this contribution.)

In section 8.3.3.1 of the working draft,

Each sample p[ x, y ] with x = -1, y= -1..nS*2-1  and x = 0, y = -1 is derived as follows

- If any of the following condition is true, the sample p[ x, y ] is marked as “not available for intra prediction”

  - the coding unit covering ( xBN, yBN ) is not available

  - the coding unit covering ( xBN, yBN ) is not coded as intra mode and constrained_intra_pred_flag is equal to 1

  - all of the following conditions are true

    - TbSize == 64

    - xB % 64 == 0

    - (yB + nS)% 64 == 32

    - x == -1

    - yBN % 64 >= 32

- Otherwise, the sample p[ x, y ] is marked as “available for intra prediction” and the sample at the location ( xBN, yBN ) inside the treeblock tbAddrN is assigned to p[ x, y ].

# Simulation Results

Table 1 below shows simulation results. It can be seen that selective dependency breaking proposed in this contribution has very small impact on BD rate and encoding/decoding time.

Table 1: BD rate comparison with HM 5.0 anchor as per common test conditions [2].

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.06% | -0.01% | 0.07% | 0.06% | 0.07% | 0.06% | 0.07% | -0.01% | 0.13% |
| Class B | 0.05% | 0.04% | 0.06% | 0.05% | 0.05% | 0.10% | 0.05% | 0.04% | 0.09% |
| Class C | 0.04% | 0.05% | 0.06% | 0.04% | 0.04% | 0.04% | 0.03% | 0.08% | 0.07% |
| Class D | 0.00% | 0.04% | 0.05% | 0.01% | 0.04% | -0.01% | 0.01% | -0.01% | 0.01% |
| Class E | 0.10% | 0.01% | 0.00% | 0.09% | 0.05% | 0.05% | 0.10% | 0.02% | -0.09% |
| Class F | 0.01% | 0.04% | 0.10% | 0.04% | 0.03% | 0.02% | 0.02% | -0.06% | -0.04% |
| Overall | 0.04% | 0.03% | 0.06% | 0.05% | 0.05% | 0.05% | 0.05% | 0.01% | 0.04% |
|  | 0.04% | 0.04% | 0.06% | 0.05% | 0.05% | 0.05% | 0.05% | 0.01% | 0.03% |
| Enc Time[%] | 98% |  |  | 101% |  |  | 102% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  | 95% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.10% | -0.06% | 0.03% | 0.09% | 0.26% | 0.03% | 0.22% | 0.69% |
| Class B | 0.00% | 0.06% | 0.03% | 0.06% | 0.09% | 0.02% | 0.03% | 0.01% | -0.05% |
| Class C | -0.04% | 0.01% | -0.01% | 0.00% | 0.01% | 0.07% | 0.03% | 0.13% | -0.02% |
| Class D | -0.02% | -0.12% | 0.13% | -0.07% | 0.28% | -0.22% | 0.02% | -0.01% | 0.03% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.08% | -0.05% | -0.12% | 0.03% | -0.05% | -0.04% | 0.10% | 0.11% | 0.13% |
| Overall | -0.01% | -0.03% | 0.02% | 0.01% | 0.12% | 0.03% | 0.03% | 0.08% | 0.15% |
|  | 0.00% | -0.05% | 0.05% | 0.01% | 0.10% | 0.06% | 0.03% | 0.07% | 0.11% |
| Enc Time[%] | 99% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  | 89% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.08% | -0.03% | 0.01% | -0.35% | 0.07% | 0.02% | -0.12% | 0.12% |
| Class C | -0.02% | -0.12% | 0.07% | -0.01% | 0.15% | -0.03% | 0.03% | -0.11% | -0.05% |
| Class D | 0.04% | -0.65% | 0.18% | -0.04% | 0.66% | 0.02% | -0.01% | 0.21% | 0.61% |
| Class E | 0.07% | 0.50% | -0.72% | 0.06% | 0.25% | -0.59% | -0.04% | -0.33% | -0.53% |
| Class F | -0.05% | 0.03% | -0.68% | -0.29% | -0.24% | -0.52% | -0.06% | 0.15% | 0.15% |
| Overall | 0.01% | -0.05% | -0.20% | -0.06% | 0.06% | -0.18% | -0.01% | -0.03% | 0.09% |
|  | 0.01% | -0.10% | -0.23% | -0.06% | 0.04% | -0.18% | -0.01% | -0.01% | 0.10% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 90% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.03% | -0.21% | 0.05% | 0.08% | -0.30% | 0.04% | -0.04% | -0.11% |
| Class C | 0.01% | -0.06% | -0.25% | 0.02% | 0.22% | 0.00% | -0.01% | -0.04% | 0.11% |
| Class D | 0.01% | -0.22% | 0.24% | -0.02% | -0.10% | 0.18% | 0.04% | -0.13% | 0.75% |
| Class E | -0.01% | 0.54% | -0.90% | 0.17% | -0.15% | -1.30% | -0.14% | -0.34% | -0.37% |
| Class F | 0.01% | -0.05% | 0.26% | -0.18% | 0.53% | -0.30% | -0.17% | 0.20% | 0.19% |
| Overall | 0.01% | 0.01% | -0.14% | 0.00% | 0.13% | -0.29% | -0.04% | -0.06% | 0.13% |
|  | 0.00% | 0.02% | -0.19% | 0.00% | 0.02% | -0.31% | -0.04% | 0.00% | 0.14% |
| Enc Time[%] | 98% |  |  | 101% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  | 89% |  |  |

# Conclusions

This contribution reports that finer granularity of parallelization (compared to Wavefront etc) during decoding can be achieved without incurring any significant BD rate penalty. Therefore, we recommend that the proposal be adopted into HM.

# Patent right declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G1103, Geneva, November 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011