| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0550 |
| --- | --- |

| Title: | Non-CE11: context assignment for parallel coefficient level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jianle Chen, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In current HM, the context modeling of the syntax element coeff_abs_level_greater1_flag is dependent on the value of previous coded coeff_abs_level_greater1_flag. Up to 16 coeff_abs_level_greater1_flag might be coded consecutively. This dependency hinders the throughput of the entropy coding and allegedly, it is a main bottleneck in current CABAC design. This contribution removes the dependency of previous coded coeff_abs_level_greater1_flag for context modeling. The coding performance impact of the proposed scheme is an average 0.1% BD-rate increase for all common test configurations.

# Introduction

In current HM5.0, there are 6 context sets for the luma components and 2 context sets of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. The index of the context set, ctxSet, is selected based on the location of the current subset and the number of coeff_abs_level_greater1_flag being one in previous coded coefficient subsets (numGreater1). Each coefficient subset consists of 16 consecutive coefficients in the inverse scan order, and there is no overlapped coefficients between the subsets. For coeff_abs_level_greater1_flag, the index of the context within a context set, greater1Ctx, is determined based on the trailing ones (coeff_abs_level_greater1_flag being 0) in the same coefficient subset to a maximum of 3. The context index is derived as:

ctxIdx_level_greater1  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )           (1)

For coeff_abs_level_greater2_flag, the index of the context within a context set, greater2Ctx, is based on the number of coeff_abs_level_greater1_flag being 1 in the same coefficient subset to a maximum of 2. The context index can be represented as:

ctxIdx_level_greater2  =  ( ctxSet  *  3 )  +  Min( 2, greater2Ctx )           (2)

For the context modeling of the syntax element coeff_abs_level_greater1_flag, the dependency between the consecutive coeff_abs_level_greater1_flags hinders the maximum throughput of the entropy coder. By removing the dependency, multiple bin per cycle decoding could be achieved for the coefficient level coding.

# Proposed solution

# In this contribution, the context modeling dependency between current coeff_abs_level_greater1_flag and the number of trailing ones in the same subset is removed. Two modifications have been made in Eq. 1.

# Maximum value of the ctxSet for luma is reduced from 6 to 4 by removing the condition on whether ( numGreater1  >>  1 ) being greater than 3.

# greater1Ctx is equal to the half of the accumulated number of coded coeff_abs_level_greater1_flag in the same subset and is capped at 6 for luma component and 4 for chroma component.

# With the modification, the total number of contexts remains unchanged. The coding efficiency impact due to these changes is shown in Section 3.1 as Test A.

# Further, we unify the derivation of luma ctxSet for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag by using the derivation method in Test A. The greater2Ctx is capped at 3 to keep the similar number of contexts (reduced from 18 to 16).  Results for this method are provided in Section 3.2 as Test B.

# Simulation results

The proposed method is implemented on HM 5.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM5.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table 3.1 and 3.2 summarize the experimental results for Test A and Test B, respectively.

## Test A

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |  |  |  |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class E | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | -0.1% | 0.2% | 0.1% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class D | 0.1% | 0.1% | 0.3% | 0.1% | 0.6% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.0% | 0.1% | 0.1% | 0.2% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.0% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.2% | 0.1% | -0.1% | 0.2% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.1% | 0.3% | 0.1% |  |  |  |
| Class D | 0.1% | 0.4% | 0.2% | 0.1% | 0.9% | 0.1% |  |  |  |
| Class E | 0.1% | 1.2% | -0.5% | 0.0% | -0.5% | -0.3% |  |  |  |
| Overall | 0.1% | 0.4% | 0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
|  | 0.1% | 0.4% | 0.0% | 0.1% | 0.2% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

  - Test B

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class E | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | -0.2% | 0.2% | 0.2% | -0.1% | 0.2% | 0.0% | 0.6% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% |  |  |  |
| Class D | 0.1% | -0.1% | 0.2% | 0.1% | 0.6% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.3% | 0.0% | 0.1% | 0.1% | 0.2% |
|  | 0.1% | 0.0% | 0.2% | 0.1% | 0.3% | -0.1% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | -0.2% | 0.5% |  |  |  |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.3% |  |  |  |
| Class D | 0.1% | 0.0% | 0.2% | 0.1% | 1.1% | -0.3% |  |  |  |
| Class E | 0.1% | -0.3% | -0.5% | 0.0% | -1.4% | 0.3% |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% |  |  |  |
|  | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusions

In this contribution, we propose to remove the dependency on the value of coeff_abs_level_greater1_flag in the same coefficient subset for the context selection of coeff_abs_level_greater1_flag. With the proposed modification, CABAC throughput is improved. The coding performance impact from the proposed scheme is small based on the simulation results.

# Reference

[1] F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, Dec, 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).