| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0552 |
| --- | --- |

| Title: | Non-CE8: Modified EO offset operation and offset quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo and David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

In the HM5.0 process, an offset is added to each pixel in attempt to reduce the distortion of the reconstructed pixel relative to the original pixel.  In this contribution, the operation in which the offset is applied to the pixel is modified, and the offsets are quantized.

# Introduction

In HM5.0, the four possible EO types indicate the direction along which pixel classification will take place.  The possible EO classes per type are defined based on whether neighboring pixels are greater than, less than, or equal to the current pixel.  The offset for a given class is then applied to all pixels in the class by adding the respective offset to the pixel.  In this contribution, the operation in which the offset is applied to the pixel is modified, and the offsets are quantized.

# Modified EO offset operation

It is proposed that the offset is applied as in HM5.0 but with the following modification.  Consider an interpolated current pixel value that is generated using a weighted combination of its two neighbors.  For example, let the current pixel value be C.  If the neighbors are L and R, then 2*C > (L + R), then the interpolated value I can be computed as (L + R) >> 1 (I is rounded away from C).  If 2*C < (L + R), then the interpolated value I can be computed as (L + R + 1) >> 1 (I is rounded away from C).  For the case of 2*C = (L + R), C is effectively already on the interpolated line so that C = I.

In this proposal we apply the offset O depending on the relative position of C and I:

- When C is not equal to I: In this case:

  - If the offset O applied to C brings the pixel value away from I, or away from the segmented line formed by (L, I, R), then the offset is applied (as in HM5.0) and V = C + O.

  - If the offset O applied to C brings the pixel value towards I (towards the line) but does not pass I, then the offset is applied (as in HM5.0) and V = C + O.

  - If the offset O applied to C brings the pixel value towards I and it passes I, then the output pixel value is set to I, that is, V = I.

- When C is equal to I: In this case the sign of offset determines the direction of offset as in HM5.0.

# Quantization and coding of offsets

In HM5.0, SAO offsets are determined based upon an RD-optimization per class and type, where the optimization and coding is performed over all possible values within a range.  In this contribution, it is proposed that the offsets are restricted to a certain subset of possible values within the entire range, and these values are assigned codewords that are entropy coded.  The selection of the offset is then done as in HM5.0, and the offset codewords are transmitted instead of the offset values.  Although the precision of the offset levels is reduced, the codewords can be coded more efficiently.

# Simulation Results

Table 1. 2 edge offset for classes combination of (1,4) and (2, 3) (Decimated sequence).

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% |
| Class E | -0.1% | 0.3% | 0.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.02% | 0.14% | 0.07% |
|  | -0.02% | 0.13% | 0.06% |
| Enc Time[%] | 87% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | -0.1% | 0.1% | 0.2% |
| Class B | 0.1% | 0.3% | 0.5% |
| Class C | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.05% | 0.17% | 0.27% |
|  | 0.05% | 0.15% | 0.25% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 1.0% | 0.9% |
| Class C | 0.1% | 0.0% | 0.1% |
| Class D | 0.1% | 0.2% | 0.1% |
| Class E | 0.9% | 2.0% | 1.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.26% | 0.76% | 0.65% |
|  | 0.25% | 0.76% | 0.60% |
| Enc Time[%] | 66% |  |  |
| Dec Time[%] | 107% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 0% |  |  |
| Dec Time[%] | 0% |  |  |

Table 2. 1 edge offset for classes combination of (1,2,3,4) (Decimated sequence)

|  |  | All Intra HE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |
| Class A (8bit) |  | 0.1% |  | 0.2% |  | 0.2% |
| Class B |  | 0.1% |  | 0.4% |  | 0.4% |
| Class C |  | 0.1% |  | 0.2% |  | 0.2% |
| Class D |  | 0.0% |  | 0.2% |  | 0.2% |
| Class E |  | 0.1% |  | 0.2% |  | -0.2% |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Overall |  | 0.08% |  | 0.24% |  | 0.21% |
|  |  | 0.08% |  | 0.26% |  | 0.21% |
| Enc Time[%] |  | 84% |  |  |  |  |
| Dec Time[%] |  | 81% |  |  |  |  |
|  |  | Random Access HE |  |  |  |  |
|  |  | Y |  | U |  | V |
| Class A (8bit) |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Class B |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Class C |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Class D |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Class E |  |  |  |  |  |  |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |
|  |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Enc Time[%] |  | #NUM! |  |  |  |  |
| Dec Time[%] |  | #NUM! |  |  |  |  |
|  |  | Low delay B HE |  |  |  |  |
|  |  | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |
| Class B |  | 0.3% |  | 1.7% |  | 1.5% |
| Class C |  | 0.3% |  | 1.3% |  | 1.0% |
| Class D |  | 0.2% |  | 0.9% |  | 1.1% |
| Class E |  | 1.2% |  | 2.8% |  | 4.1% |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Overall |  | 0.46% |  | 1.58% |  | 1.75% |
|  |  | 0.45% |  | 1.63% |  | 1.68% |
| Enc Time[%] |  | 64% |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |
|  |  | Random Access HE-10 |  |  |  |  |
|  | Y |  | U |  | V |  |
| Class A | -0.1% |  | 0.2% |  | 0.4% |  |
| Class B | 0.0% |  | -0.4% |  | -0.4% |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.04% |  | -0.14% |  | -0.07% |  |
|  | -0.04% |  | -0.14% |  | -0.09% |  |
| Enc Time[%] | 124% |  |  |  |  |  |
| Dec Time[%] | 129% |  |  |  |  |  |

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).