| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0553r1 |
| --- | --- |

| Title: | Non-CE8: Harmonization of LCU-based SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, Yue Yu, David Baylon, and Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com yyu@motorola.com david.baylon@motorola.com limin.wang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

In HM5.0, SAO uses two fixed band types B0 and B1 covering the entire intensity range, and each band further divides the respective intensity range into sixteen equal sub-partitions or classes.  When applied at the LCU-level, the fixed band types may not match the pixel values well, and the overhead associated with the BO classes can be quite large.  This contribution proposes to adaptively specify the range of values for the BO types without requiring any overhead and to reduce the number of BO classes.

# Introduction

SAO in HM5.0 uses two fixed band types B0 and B1 covering the entire intensity range, and each band further divides the respective intensity range into sixteen equal sub-partitions or classes.  Because the statistics of a given picture may not fall nicely into one of the two existing band types B0 and B1, it may be preferable to define bands that better match the data.  This is especially true at the LCU-level, where the full range of intensity values is typically not observed.

# LCU-based BO types and classes

The specification of a BO type and associated classes is proposed as follows.  Since the range of reconstructed values per LCU is known at both the encoder and decoder, it does not need to be transmitted.  In this case, the range of reconstructed values can be determined after the unit is encoded or decoded, and the BO type is derived from this range.  Note that because natural images tend to exhibit high spatial correlation, a smaller unit may tend to have a smaller dynamic range of intensity values, thereby requiring fewer classes or offsets.  In this proposal, the encoder and decoder  reconstruct an LCU and determine the range of values [min, max] in the LCU.  Each BO type divides the [min, max] span into segments for which the offset would be calculated, transmitted and applied. Different BO types and classes can be defined for both luma and chroma channels.  In addition to LCU-based BO types and classes, EO operations per LCU will be used, such as described in [1].

# Simulation Results

In this simulatin we used 4 bandoffset per BO_0 SAO type and 8 bandoffsets per BO_1 SAO typ.

Note that in comparison to CE8.a.3, H0553 uses the LCU based optimization for all the frames including non-reference frames, while CE8.a.3 takes advantage of picture based optimization. So to get a picture how much improvement H0553 can provide, one can concentrate on the Low-delay coding test conditions.

Table 1. Simulation results: Anchor H0553 vs. Anchor HM5.0

|  | All Intra HE |  |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |
| Class A (8bit) | 0.8% | -1.6% | -2.6% |  |  |  |  |
| Class B | 0.7% | -1.8% | -2.5% |  |  |  |  |
| Class C | 0.5% | -1.5% | -1.8% |  |  |  |  |
| Class D | 0.5% | -1.1% | -1.3% |  |  |  |  |
| Class E | 1.4% | -5.4% | -4.6% |  |  |  |  |
| Overall | 0.76% | -2.15% | -2.45% |  |  |  |  |
|  | 0.76% | -2.08% | -2.38% |  |  |  |  |
| Class F | 0.92% | -1.46% | -0.67% |  |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access HE-10 |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A (8bit) | 0.8% | -2.1% | -3.2% |  | 1.0% | 0.5% | 0.4% |
| Class B | 0.9% | -2.5% | -3.3% |  | 0.9% | -0.3% | -1.1% |
| Class C | 0.7% | -1.4% | -1.5% |  |  |  |  |
| Class D | 0.9% | -1.8% | -1.8% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.85% | -1.98% | -2.41% |  | 0.92% | 0.04% | -0.41% |
|  | 0.85% | -1.96% | -2.38% |  | 0.97% | 0.09% | -0.34% |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  |  | 97% |  |  |
|  | Low delay B HE |  |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | 1.1% | -2.2% | -3.2% |  |  |  |  |
| Class C | 0.8% | -1.3% | -1.5% |  |  |  |  |
| Class D | 0.8% | -1.7% | -1.7% |  |  |  |  |
| Class E | 1.4% | -5.9% | -3.3% |  |  |  |  |
| Overall | 1.01% | -2.56% | -2.43% |  |  |  |  |
|  | 1.01% | -2.45% | -2.38% |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |  |
| Dec Time[%] | 97% |  |  |  |  |  |  |
|  | Low delay P HE |  |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | 1.2% | -1.5% | -2.7% |  |  |  |  |
| Class C | 0.8% | -0.7% | -1.4% |  |  |  |  |
| Class D | 0.9% | -1.4% | -2.4% |  |  |  |  |
| Class E | 1.3% | -6.4% | -2.9% |  |  |  |  |
| Overall | 1.03% | -2.19% | -2.31% |  |  |  |  |
|  | 1.03% | -2.09% | -2.31% |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |  |

Table 2. Simulation results: Anchor H0553 vs. CE8.a.3

|  | All Intra HE |  |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |
| Class A (8bit) | 0.8% | -1.7% | -2.7% |  |  |  |  |
| Class B | 0.7% | -2.1% | -3.0% |  |  |  |  |
| Class C | 0.5% | -1.6% | -1.9% |  |  |  |  |
| Class D | 0.5% | -1.2% | -1.4% |  |  |  |  |
| Class E | 1.3% | -5.6% | -4.7% |  |  |  |  |
| Overall | 0.71% | -2.31% | -2.65% |  |  |  |  |
|  | 0.71% | -2.24% | -2.59% |  |  |  |  |
| Class F | 0.83% | -1.66% | -0.88% |  |  |  |  |
| Enc Time[%] | 74% |  |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access HE-10 |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A (8bit) | 0.4% | -2.2% | -3.4% |  | 0.5% | -2.5% | -3.0% |
| Class B | 0.5% | -3.6% | -4.4% |  | 0.4% | -2.4% | -3.3% |
| Class C | 0.3% | -2.1% | -2.4% |  |  |  |  |
| Class D | 0.6% | -2.0% | -2.3% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.47% | -2.57% | -3.19% |  | 0.42% | -2.42% | -3.18% |
|  | 0.47% | -2.57% | -3.17% |  | 0.44% | -2.38% | -3.15% |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |
| Enc Time[%] | 71% |  |  |  | 68% |  |  |
| Dec Time[%] | 82% |  |  |  | 81% |  |  |
|  | Low delay B HE |  |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | 0.2% | -5.8% | -7.5% |  |  |  |  |
| Class C | 0.2% | -3.5% | -4.2% |  |  |  |  |
| Class D | 0.3% | -3.0% | -4.2% |  |  |  |  |
| Class E | 0.5% | -11.0% | -10.4% |  |  |  |  |
| Overall | 0.30% | -5.50% | -6.39% |  |  |  |  |
|  | 0.30% | -5.38% | -6.28% |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |
| Enc Time[%] | 70% |  |  |  |  |  |  |
| Dec Time[%] | 83% |  |  |  |  |  |  |
|  | Low delay P HE |  |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | 0.2% | -6.3% | -7.5% |  |  |  |  |
| Class C | 0.1% | -3.5% | -4.3% |  |  |  |  |
| Class D | 0.3% | -3.0% | -4.4% |  |  |  |  |
| Class E | 0.5% | -11.6% | -10.9% |  |  |  |  |
| Overall | 0.28% | -5.79% | -6.57% |  |  |  |  |
|  | 0.29% | -5.63% | -6.47% |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |
| Enc Time[%] | 70% |  |  |  |  |  |  |
| Dec Time[%] | 89% |  |  |  |  |  |  |

Table 3. BDR based on (6PSNR_Y + PSNR_U + PSNR_V)/8 for H0553 vs HM5.0

| PSNR_YUV |  |  |  |  |
| --- | --- | --- | --- | --- |
| AI_HE | 0.24% |  |  |  |
| RA_HE | 0.34% |  | RAHE_10b | 0.74% |
| LB_HE | 0.52% |  |  |  |
| LP_HE | 0.44% |  |  |  |

Table 4. BDR based on (6PSNR_Y + PSNR_U + PSNR_V)/8 for H0553 vs CE8.a.3

| PSNR_YUV |  |  |  |  |
| --- | --- | --- | --- | --- |
| AI_HE | 0.17% |  |  |  |
| RA_HE | -0.11% |  | RAHE_10b | -0.08% |
| LB_HE | -0.82% |  |  |  |
| LP_HE | -0.77% |  |  |  |

# Reference

[1]  K. Minoo and D. Baylon, “Non-CE8: Modified EO offset operation and offset quantization,” JCTVC-H0552, Feb. 2011.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).