| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0554 |
| --- | --- |

| Title: | Non-CE1: throughput improvement on CABAC coefficients level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen, Wei-Jung Chien, Joshi Rajan, Joel Sole, Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8028 cjianle@qualcomm.com wchien@qualcomm.com rajanj@qualcomm.com joels@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal targets on improving CABAC throughput by reducing context coded bins for coefficients level coding. In current HM, three flags, significant_coeff_flag, coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag are coded using contexts before switching to the Golomb-Rice by-pass mode coding for the remaining level. This contribution proposes coding coeff_abs_level_greater1_flag only for a few starting non-zero coefficients and only one coeff_abs_level_greater2_flag in a subset consisting of16 coefficients, and an early switch to Golomb-Rice code.

Coding efficiency impact of the first proposed scheme is -0.04% BD-Rate gain on average in common test condition, and -0.10% at low QPs (1, 5, 9 and 13). It is further reported that this scheme reduces the maximum number of context adaptively coded bins by averagely 41% among all configurations, up to 44% in intra configuration at QP being 1, and, by averagely 10% among all configurations, up to 18% in intra configuration at QP being 22.

Coding efficiency impact of the second proposed scheme is -0.00% BD-Rate gain on average in common test condition, and -0.05% at low QPs. It is further reported that this scheme reduces the maximum number of context adaptively coded bins by averagely 32% among all configurations, up to 36% in intra configuration at QP being 1, and, by averagely 8% among all configurations, up to 15% in intra configuration at QP being 22.

It is further reported that all the modules and processes of current CABAC are kept unchanged and no additional module is needed in the proposed approaches.

# Introduction

In the current HEVC test model, significance and level information is coded for groups of 16 coefficients, which is referred to as a subset. A subset consists of 16 coefficients in the inverse scan order. For 16×16 and 32×32 TUs, which use a 4×4 sub-block based scans, each subset corresponds to a 4×4 sub-block. The following five syntax elements are signaled to represent the coefficients level information within a subset.

significant_coeff_flag (significantFlag): indicates the significance of each coefficient.

coeff_abs_level_greater1_flag (abbr. gr1Flag): indicates whether the coefficient amplitude is larger than one for each non-zero coefficient (i.e. with significantFlag as 1).

coeff_abs_level_greater2_flag (abbr. gr2Flag): indicates whether the coefficient amplitude is larger than two for each coefficient with amplitude larger than one (i.e. with gr1Flag as 1).

coeff_sign_flag (abbr. signFlag): indicates sign information of the non-zero coefficients.

coeff_abs_level_minus3 (abbr. levelRem): indicates remaining absolute level value; i.e. a value of (abs(level)-3) is signaled for each coefficient with amplitude larger than two (i.e. with gr2Flag as 1).

Among the five syntax element, the bins of significantFlag, gr1Flag and gr2Flag are encoded with adaptive context models (regular mode); signFlag and binarized bins of levelRem are encoded through by-pass mode with fixed equal probability model. With the current design, three bins are encoded with adaptive context model for a coefficient larger than one. Context-based bin coding is one of the main bottlenecks of HEVC throughput.

Fig.1 shows an example of quantized coefficients in a 4×4 block. The encoded symbols for these coefficients in inverse scan order are summarized in Tab.1. In case of the 4×4 TU or the last 4×4 sub-block in large TU, the first 2 significantFlags (shown in gray) are not necessary since the position of the last non-zero coefficient is known.

Fig.1 Quantized coefficients in a 4×4 TU or sub-block

Tab1. Coded symbols for the coefficients of a chunk in HM5.0

| Scan_pos | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coefficients | 0 | 1 | -1 | 0 | 2 | 4 | -1 | -4 | 4 | 2 | -6 | 4 | 7 | 6 | -12 | 18 |
| significantFlag | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| gr1Flag |  | 0 | 0 |  | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| gr2Flag |  |  |  |  | 0 | 1 |  | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| signFlag |  | 0 | 1 |  | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| levelRem |  |  |  |  |  | 1 |  | 1 | 1 |  | 3 | 1 | 4 | 3 | 9 | 15 |

The coefficients level information is coded in inverse scan order, which means the high frequency components of transformed coefficients are scanned first. In such design, the amplitude of the first few coefficients in the reverse scan order tends to be small. Furthermore if they are significant, with high probability, the absolute level is 1 or 2. For these coefficients, it is beneficial to use symbols gr1Flag and gr2Flag to reduce the length of binarized bins and the flags can be efficiently encoded with well adapted context model. However, after encountering a certain number of greater than 1 coefficients, due to reverse scanning, it is much likely that the remaining significant coefficients also have high magnitudes. In such a case, using gr1Flag and gr2Flag is not likely to improve the compression performance while increasing the number of context-coded bins.

This proposal targets on improving the CABAC throughput by reducing the number of context-coded gr1Flag and gr2Flags.

# Proposed method

In this contribution, we propose to encode gr1Flag and gr2Flag only for a few starting non-zero coefficients in inverse scan order. We propose two schemes offering different trade-offs of throughput and compression efficiency.

Scheme I

- Encode gr1Flag until two coefficients with absolute level greater than 1 are encountered in a subset.

- Encode gr2Flag only for the first coefficient with gr1Flag being 1 in a subset.

Scheme II

- Encode gr1Flag only for the first 8 non-zero coefficients in a subset.

- Encodegr2Flag only for the first coefficient with gr1Flag being 1 in a subset.

For scheme II, we will test different N (2, 4 and 8) value to find the best trade-off of performance and throughput improvement.

Tab. 2 shows the symbols to be coded for the example subset shown in Fig.1 with scheme I. Compared with the coded symbols in Tab.1 (HM5.0 method), 10 gr1Flags and 10 gr2Flags at the positions marked as magenta are removed. The value of levelRem is adjusted appropriately to account for the missing gr1Flag and/or gr2Flag.

Tab2. Coded symbols for the coefficients in an intra-chunk with proposed method

| Scan_pos | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coefficients | 0 | 1 | -1 | 0 | 2 | 4 | -1 | -4 | 4 | 2 | -6 | 4 | 7 | 6 | -12 | 18 |
| significantFlag | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| gr1Flag |  | 0 | 0 |  | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| gr2Flag |  |  |  |  | 0 | 1 |  | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| signFlag |  | 0 | 1 |  | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| levelRem |  |  |  |  |  | 2 | 0 | 3 | 3 | 1 | 5 | 3 | 6 | 5 | 11 | 17 |

In the proposed method, when gr1Flag and gr2Flag are not coded, the statistics of levelRem value will be changed comparing to the case of HM5.0. Hence, we also propose to modify the Golomb-Rice-parameter-update-table to accommodate changed statistics of levelRem.

# Simulation Results

Simulations are performed using common test conditions [1]. Additionally, simulations for low QP conditions (QP 1, 5, 9, 13) are performed as specified in the CE1 document [2].

Tab.3 shows the results of the proposed schemes I in common test condition and low QP test condition.

Tab.4 shows the results of the proposed schemes II  with N being 8 in common test condition and low QP test condition.

Tab.3 BD-Rate of proposed schemes I

| Test configuration | Scheme I (Normal QP) |  |  | Scheme I (Low QP) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |
|  | Y | Cb | Cr | Y | Cb | Cr |
| HE_Intra | -0.1% | -0.1% | -0.1% | -0.5% | -0.3% | -0.3% |
| HE_RA | -0.1% | -0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| HE_LD | 0.0% | 0.1% | -0.1% | 0.1% | 0.3% | 0.3% |
| LC_Intra | -0.1% | -0.1% | -0.1% | -0.5% | -0.3% | -0.3% |
| LC_RA | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| LC_LD | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| HE_RA_10bits | 0.1% | 0.1% | 0.1% | 0.0% | 0.4% | 0.4% |
| Average | -0.04% | 0.01% | -0.03% | -0.10% | 0.08% | 0.07% |

Tab.4 BD-Rate of proposed schemes II (N = 8)

| Test configuration | Scheme II (Normal QP) |  |  | Scheme II (Low QP) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |
|  | Y | Cb | Cr | Y | Cb | Cr |
| HE_Intra | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| HE_RA | 0.0% | -0.1% | -0.2% | 0.1% | 0.2% | 0.1% |
| HE_LD | 0.0% | 0.1% | -0.1% | 0.1% | 0.2% | 0.2% |
| LC_Intra | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| LC_RA | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| LC_LD | 0.0% | 0.1% | -0.3% | 0.0% | 0.1% | 0.1% |
| HE_RA_10bits | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.3% |
| Average | 0.00% | 0.02% | -0.06% | -0.05% | 0.06% | 0.03% |

# Complexity analysis

## Context coded bin statistics

With the proposed method, at most one gr2Flag is context coded instead of 16 in the worst case for HM5.0. Coding of gr1Flag is also quickly switched off when there are many non-zero coefficients in a subset.

Tab.5 summarize the average saving of context coded bin number of proposed scheme I by comparing HM5.0 at each QP.

Tab.6 summarize the worst-case saving of context coded bin number of proposed scheme I by comparing HM5.0 at each QP. Here the worst-case means maximum number of context coded bins per pixel in one picture within the sequence. For random access and low delay configuration, only P and B pictures are considered.

Tab.5 Saving of average context coded bin number of proposed scheme I

| QP | AI_HE | RA_HE | LB_HE | RA_10 | AI_LC | RA_LC | LB_LC | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 37% | 29% | 29% | 32% | 36% | 26% | 26% | 31% |
| 5 | 30% | 21% | 21% | 23% | 30% | 18% | 18% | 23% |
| 9 | 25% | 14% | 14% | 17% | 23% | 12% | 11% | 17% |
| 13 | 20% | 10% | 8% | 12% | 19% | 8% | 6% | 12% |
| 22 | 14% | 6% | 4% | 12% | 11% | 5% | 2% | 8% |
| 27 | 10% | 5% | 3% | 8% | 8% | 4% | 2% | 6% |
| 32 | 6% | 3% | 2% | 3% | 5% | 2% | 1% | 3% |
| 37 | 3% | 2% | 1% | 2% | 2% | 1% | 1% | 2% |

Tab.6 Saving of maximum context coded bin number of proposed scheme I

| QP | AI_HE | RA_HE | LB_HE | RA_10 | AI_LC | RA_LC | LB_LC | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 44% | 41% | 37% | 40% | 44% | 40% | 38% | 41% |
| 5 | 40% | 36% | 33% | 34% | 40% | 34% | 30% | 35% |
| 9 | 35% | 28% | 22% | 23% | 35% | 26% | 21% | 27% |
| 13 | 30% | 22% | 16% | 12% | 30% | 21% | 17% | 21% |
| 22 | 18% | 13% | 7% | 3% | 17% | 10% | 5% | 10% |
| 27 | 13% | 5% | 2% | 18% | 10% | 4% | 2% | 8% |
| 32 | 4% | 2% | 4% | 5% | 3% | -1% | 1% | 3% |
| 37 | 5% | 3% | 1% | 0% | 4% | 2% | -2% | 2% |

Tab.7 summarize the average saving of context coded bin number of proposed scheme II (N = 8) by comparing HM5.0 at each QP.

Tab.8 summarize the worst-case saving of context coded bin number of proposed scheme II (N = 8) by comparing HM5.0 at each QP. Here the worst-case means maximum number of context coded bins per pixel in one picture within the sequence. For random access and low delay configuration, only P and B pictures are considered.

Tab.7 Saving of average context coded bin number of proposed scheme II (N = 8)

| QP | AI_HE | RA_HE | LB_HE | RA_10 | AI_LC | RA_LC | LB_LC | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29% | 22% | 22% | 24% | 28% | 18% | 18% | 23% |
| 5 | 23% | 16% | 16% | 18% | 22% | 12% | 12% | 17% |
| 9 | 19% | 11% | 11% | 13% | 17% | 9% | 8% | 13% |
| 13 | 16% | 7% | 6% | 9% | 14% | 5% | 4% | 9% |
| 22 | 11% | 5% | 3% | 9% | 8% | 3% | 2% | 6% |
| 27 | 8% | 4% | 2% | 6% | 6% | 2% | 1% | 4% |
| 32 | 5% | 2% | 1% | 3% | 3% | 1% | 1% | 2% |
| 37 | 2% | 1% | 1% | 2% | 2% | 1% | 0% | 1% |

Tab.8 Saving of maximum context coded bin number of proposed scheme II (N = 8)

| QP | AI_HE | RA_HE | LB_HE | RA_10 | AI_LC | RA_LC | LB_LC | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 36% | 33% | 31% | 32% | 35% | 31% | 28% | 32% |
| 5 | 32% | 28% | 25% | 26% | 32% | 26% | 21% | 27% |
| 9 | 28% | 23% | 17% | 18% | 27% | 20% | 15% | 21% |
| 13 | 24% | 17% | 13% | 10% | 23% | 15% | 14% | 16% |
| 22 | 15% | 10% | 6% | 3% | 13% | 6% | 3% | 8% |
| 27 | 9% | 4% | 1% | 14% | 8% | 2% | 2% | 6% |
| 32 | 4% | 0% | 0% | 5% | 2% | -1% | 2% | 2% |
| 37 | 3% | 3% | 3% | 0% | 3% | 1% | -7% | 1% |

## Context models reduction

In the proposed methods, only one gr2Flag is coded in each subset, therefore only the first context model in each context set is used, which means 2 context models are saved in each context set. There are 8 context sets (6 for luma and 2 for chroma) for gr2Flag in HM5.0. Hence the number context models for gr2Flag syntax can be reduced by 16 using the proposed methods.

# Conclusions

In this contribution, we propose to code gr1Flag and gr2Flag only for a few starting non-zero coefficients in inverse scan order and early switching to Golomb-Rice by-pass mode coding for the remaining coefficients. The proposed method and its advantages can be summarized as

- Context coded bins are dramatically reduced, especially at the high bit where CABAC throughput is more problematic.

  - In both scheme I and scheme II, at most one gr2Flag is context coded in one subset instead of 16 in the worst case in HM5.0.

  - In scheme I, context-based gr1Flag coding is quickly switched off once 2 coefficients with level greater than 1 are encountered in a subset.

  - In scheme II, at most 8 gr1Flags are context coded in one subset instead of 16 in the worst case in HM5.0.

- The performance impact of proposed scheme I is averagely -0.04% (normal QP) and -0.10% (low QP) BD-rate gain comparing to HM5.0.

- The performance impact of proposed scheme II is averagely -0.00% (normal QP) and -0.05% (low QP) BD-rate gain comparing to HM5.0.

- The number of context models for gr2Flag syntax is reduced by 16 for both schemes.

- No additional calculation complexity is added, i.e. all the modules and process of current CABAC are kept unchanged and no additional module is needed, i.e. very small change of software and specification text.

We recommend adoption of the proposed scheme I or scheme II in the next version of HM.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, Nov. 2011.

- R. Joshi, E. Alshina, et al, “Description of Core Experiment 1 (CE1): Entropy coding investigation”, Document of Joint Collaborative Team on Video Coding, JCTVC-G1201, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text Changes (changes marked in yellow)

Changes in table 7.3.10 residual coding syntax with scheme I

| residual_coding ( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| Else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| Else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| numGreater1Coeff = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && numGreater1Coeff < 2 ){ |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ n ]) |  |
| numGreater1Coeff ++ |  |
| } |  |
| } |  |
| isFirstGreater1Coeff = 1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater2Flag) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| isFirstGreater1Coeff = 0 |  |
| } |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numGreater1Coeff = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
|  | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if(baseLevel == numGreater1Coeff < 2 ? 2 + !numGreater1Coeff: 1 ){ |  |
| coeff_abs_level_remaining[ n ] |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( coeff_abs_level_greater1_flag[ n ]) |  |
| numGreater1Coeff ++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

Changes in table 7.3.10 residual coding syntax with scheme II

| residual_coding ( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| Else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| Else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| numSigCoeff = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && numSigCoeff < 8 ){ |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| } |  |
| } |  |
| isFirstGreater1Coeff = 1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater2Flag) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| isFirstGreater1Coeff = 0 |  |
| } |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| isFirstGreater1Coeff = 1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
|  | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if(baseLevel == numSigCoeff < 8 ? 2 + isFirstGreater1Coeff : 1 ){ |  |
| coeff_abs_level_remaining[ n ] |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ]) |  |
| isFirstGreater1Coeff = 0 |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

Changes in 7.4.10 Residual coding semantics

coeff_abs_level_remaining[ n ] is the remained value of a transform coefficient level that is coded with golomb-Rice code at the scanning position n. The value of coeff_abs_level_ remaining is constrained by the limits in subclause XX.When coeff_abs_level_ remaining [ n ] is not present, it is inferred as 0.

Change in section 9.2.1.1 Initialization process for context variables

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| … | … | … | … | … | … |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Table 9-28 | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..31 | 32..63 | 64..95 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..7 | 8..15 | 16..23 |

Change in Table 9-2 – Specifcation of cRiceParam depending on cLastSE and cLastRiceParam

| cLastRiceParam | cLastSE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | >11 |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

Change in Table 9-3 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  | 1 |  |  | 2 |  |  | 3 |  |  |
| initValue | 86 |  |  | 102 |  |  | 103 |  |  | 115 |  |  |
|  | 4 |  |  | 5 |  |  | 6 |  |  | 7 |  |  |
| initValue | 117 |  |  | 102 |  |  | 101 |  |  | 101 |  |  |
|  | 8 |  |  | 9 |  |  | 10 |  |  | 11 |  |  |
| initValue | 84 |  |  | 117 |  |  | 103 |  |  | 83 |  |  |
|  | 12 |  |  | 13 |  |  | 14 |  |  | 15 |  |  |
| initValue | 100 |  |  | 85 |  |  | 84 |  |  | 117 |  |  |
|  | 16 |  |  | 17 |  |  | 18 |  |  | 57 |  |  |
| initValue | 84 |  |  | 117 |  |  | 87 |  |  | 83 |  |  |
|  | 60 |  |  | 63 |  |  | 66 |  |  | 69 |  |  |
| initValue | 84 |  |  | 69 |  |  | 84 |  |  | 117 |  |  |

###### Change in 9.3.1.1.7 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

Inputs to this process are the color component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxIdxInc specifies the current context index and for its derivation the following applies.

-

-

- ctxIdxInc = 0;

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of this subclause for the subset i + 1 and is modifed as follows.

numGreater1  =  numGreater1  >>  1		(9-3)

- When numGreater1 is greater than 0, ctxSet is incremented by one as follows.

ctxIdxInc =  ctxIdxInc +  1		(9-4)

- When numGreater1 is greater than 3 and cIdx is equal to 0, ctxSet is incremented by one as follows.

ctxIdxInc =  ctxIdxInc +  1		(9-5)

(9-6)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  3