| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0555 |
| --- | --- |

| Title: | Range restriction of motion vector predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Muhammed Coban Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8028 cjianle@qualcomm.com mcoban@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Problem and suggestion

In previous video standards, there is a limitation on vertical/horizontal motion vector (MV) component range according to profile/level. The range of vertical MV component is usually smaller than the range of horizontal MV component. It’s likely that the similar restriction is necessary and will be included in HEVC standard.

In HEVC standard, when a MVP is derived from a MV pointing to a different reference picture; the MV is scaled to the target reference picture as the final MVP. In the MV scaling process, the scaling factor is defined according to the POC distances. The scaled MV based on POC distances can exceed the MV range specified for each profile/level. When Merge mode is used, the scaled MV will be used directly as the MV of current block which results in uncomforted bit stream. Otherwise, this would prevent the encoder to use the merge mode for cases where the merge mode MV exceeds the motion vector range. In order to allow this case, it is suggested to limit the scaled MV to stay within the MV range limits.

The problem can be solved by either clipping the scaled MV to stay within the bounds of the MV range or clipping the MVP of current block to stay within the bounds of the MV range.

The suggested method could also be applied to AMVP motion vector in normal inter mode cases. The extra-ordinary overhead bits are needed to encode MVD syntax if the selected MVP exceeds the motion vector range.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).