| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0557r1 |
| --- | --- |

| Title: | CE8.e.4 Selection and Coding of SAO Parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo and David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document is a Core Experiment report for CE8.e.4 corresponding to [1].  Compared to [1], this experiment allows to retain the existing four EO classes per type where an offset is applied, but changes how the offsets are applied.  In addition, as being studied in CE8.c.1, this experiment also studies the restriction of the EO offset to be of the same sign (In contrast to CE8.c.1 the sign is inferred from the relative position of the current pixel and a bilinear interpolation from the two neighboring pixels, according to the edge direction, which may or may not be the same for each class type). A method similar to [1] is used for coding of SAO types and furthermore, in this experiment we modified the entropy coding of the offsets.

Simulations with 4 EO classes and signed offsets for HE reportedly show savings of up to 0.1% for luma and 0.9% for chroma.

# Introduction

In HM5.0, the four possible EO types indicate the direction along which pixel classification will take place.  The five possible EO classes per type are determined based upon whether neighboring pixels are greater than, less than, or equal to the current pixel.  The offset for a given class is then applied to all pixels in the class.  In HM5.0, the offset can be positive or negative.

# Modification in the application of the EO offset

In this experiment, the EO offset is restricted to be of the same sign (where the sign is inferred from the relative position of the current pixel and a bilinear interpolation from the two neighboring pixels according to the edge direction).  It is proposed that this offset be applied per class but with the following modification.  Consider an interpolated current pixel value that is generated using a weighted combination of its two neighbors.  For example, let the current pixel value be C.  If the neighbors are L and R, then if 2*C > (L + R), then the interpolated value I can be computed as (L + R + 1) >> 1 (round to +infinity).  If 2*C < (L + R), then the interpolated value I can be computed as (L + R)  >> 1 (round to –infinity). In both of these cases I will be the average of L and R, rounded towards the C. It is also possible to derive I by averaging L and R and rounding away from C.

Let a given offset with value V be applied to current pixel with value C to change its value to O.

- If C = I, then no offset is applied, and O = C.

- As shown below, if the offset V applied to C brings the pixel value towards I but does not pass I, then the offset is applied (as in HM5.0) and O = C + V.

- As shown below, if the offset V applied to C brings the pixel value towards I and passes I, then the output pixel value is set to I, that is, O = I.  With this approach, the offset can serve both as a noise threshold to reduce variations caused by noise and as a signal enhancer to restore signal values.

In another experiment, the EO offset is allowed to have both signs.  In this case, the above applies but when the offset is negative, the offset magnitude is applied in the direction away from I.

# Modification in the entropy coding of the EO offset

When the EO offset is defined to be positive when going towards I, the distribution of the EO offsets is expected to be different than that of the same positive range in the traditional (e.g.g HM5.0) case because the direction (sign) is now taken into account according to class type.  It has been observed (see below) that the +1 offset tends to be more frequent than the zero offset over EO types (especially at low QPs).  Taking this into account, the codewords for the +1 and 0 offsets were switched for EO types.

# Modification in the number of edge offset classes

As proposed in [1], the number of potential classes has expanded to 8. With a proper mapping, a certain number of classes per edge type can be achieved. In this experiment we study when the total number of edge offsets is either 4 or 8.

# Modification in the entropy coding of the SAO types

As proposed in [1], the codeword for the representation of SAO type is changed to make the bit cost for signaling the SAO types more even, as shown below.

SAO type		code

SAO off		1

SAO E0		0000

SAO E1		0001

SAO E2		0010

SAO E3		0011

SAO B0		010

SAO B1		011

# Simulation Results

In this CE8.e.4 experiment, three tests are studied.  In all tests, the modified EO offset is applied (Section 2), as with the modified entropy coding of offsets (Section 3) and SAO type (Section 5).  In  Test 1, the EO offset is unsigned (non-negative) and 4 EO classes are used.  In Test 2, the EO offset is signed and 4 EO classes are used.  In Test 3, the EO offset is unsigned (non-negative) and 8 EO classes (Section 4) are used.  The compiler directives are set as follows:

Test 1:

#define SAO_UNSIGNED_EDGE_OFFSET          1

#define SAO_ENH_EDGE_CLASS                        0

Test 2:

#define SAO_UNSIGNED_EDGE_OFFSET          0

#define SAO_ENH_EDGE_CLASS                        0

Test 3:

#define SAO_UNSIGNED_EDGE_OFFSET          1

#define SAO_ENH_EDGE_CLASS                        1

For all the tests the followings settings are required:

#define SAO_EDGE_OFFSET_DIRECTION               1

#define SAO_OFFSET_SWAP_ZERO_ONE               1

#define SAO_TYPE_CODING                                     1

Simulation Results for Test 1 (4 Classes per EO-Type+UnSignedOffset):

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.3% | -0.3% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class B | -0.1% | -0.2% | -0.4% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class C | -0.2% | -0.3% | -0.5% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | -0.1% | -0.2% | -0.4% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E | -0.2% | 0.5% | 0.3% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | -0.13% | -0.13% | -0.30% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
|  | -0.13% | -0.13% | -0.30% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class F | -0.49% | -0.32% | -0.31% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 107% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | #NUM! |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.07% | 0.04% | -0.03% | #VALUE! | #VALUE! | #VALUE! | 0.0% | 1.1% | 1.2% |
| Class B | -0.07% | 0.28% | 0.16% | #VALUE! | #VALUE! | #VALUE! | -0.1% | -0.2% | -0.3% |
| Class C | -0.11% | -0.44% | -0.25% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | -0.08% | -0.13% | -0.52% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.05% | -0.16% | #VALUE! | #VALUE! | #VALUE! | -0.06% | 0.38% | 0.40% |
|  | -0.08% | -0.06% | -0.14% | #VALUE! | #VALUE! | #VALUE! | -0.05% | 0.36% | 0.40% |
| Class F | -0.5% | -0.4% | -0.3% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 102% |  |  | #NUM! |  |  | 105% |  |  |
| Dec Time[%] | 106% |  |  | #NUM! |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 1.7% | 3.0% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class C | 0.0% | 0.5% | 0.5% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | 0.0% | 0.0% | 0.8% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E | 0.0% | 3.4% | 0.5% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | 0.00% | 1.33% | 1.34% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
|  | -0.01% | 1.32% | 1.34% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class F | -0.4% | 0.1% | 0.3% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 101% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | #NUM! |  |  |  |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 2.0% | 2.7% |  |  |  |  |  |  |
| Class C | 0.0% | 0.5% | 0.4% |  |  |  |  |  |  |
| Class D | -0.1% | 0.8% | 0.6% |  |  |  |  |  |  |
| Class E | 0.0% | 3.7% | 0.5% |  |  |  |  |  |  |
| Overall | -0.01% | 1.64% | 1.20% |  |  |  |  |  |  |
|  | 0.00% | 1.59% | 1.11% |  |  |  |  |  |  |
| Class F | -0.4% | 0.8% | 0.3% |  |  |  |  |  |  |
| Enc Time[%] | 102% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 106% |  |  |  |  |  |  |  |  |

Simulation Results for Test 2 (4 Classes per EO-Type+SignedOffset):

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.3% | -0.4% | -1.1% | -2.5% | -2.8% |  |  |  |
| Class B | 0.0% | -0.3% | -0.5% | -0.6% | -2.7% | -4.1% |  |  |  |
| Class C | -0.1% | -0.4% | -0.4% | -0.8% | -2.9% | -3.9% |  |  |  |
| Class D | 0.0% | -0.2% | -0.3% | -0.5% | -2.0% | -2.7% |  |  |  |
| Class E | 0.0% | -0.5% | -0.5% | -1.0% | -5.0% | -4.5% |  |  |  |
| Overall | -0.03% | -0.33% | -0.42% | -0.7% | -3.0% | -3.7% |  |  |  |
|  | -0.03% | -0.31% | -0.40% | -0.7% | -2.9% | -3.6% |  |  |  |
| Class F | -0.32% | -0.36% | -0.37% | -3.1% | -4.0% | -5.8% |  |  |  |
| Enc Time[%] | 103% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | #NUM! |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.2% | -0.4% | -1.4% | -2.7% | -3.7% | 0.0% | -0.1% | 0.2% |
| Class B | 0.0% | -0.4% | -0.6% | -2.0% | -6.1% | -6.4% | -0.1% | -0.5% | -0.6% |
| Class C | -0.1% | -0.5% | -0.6% | -1.0% | -4.6% | -6.0% |  |  |  |
| Class D | 0.0% | -0.4% | -0.4% | -0.4% | -2.6% | -3.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.42% | -0.50% | -1.2% | -4.3% | -5.0% | -0.03% | -0.34% | -0.25% |
|  | -0.05% | -0.43% | -0.48% | -1.2% | -4.3% | -5.0% | -0.02% | -0.37% | -0.27% |
| Class F | -0.4% | -0.5% | -0.6% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 101% |  |  | #NUM! |  |  | 102% |  |  |
| Dec Time[%] | 105% |  |  | #NUM! |  |  | 106% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.4% | -3.0% | -9.5% | -10.9% |  |  |  |
| Class C | 0.0% | -0.6% | -0.5% | -1.9% | -8.1% | -11.8% |  |  |  |
| Class D | -0.1% | -0.8% | -0.8% | -0.8% | -4.3% | -5.9% |  |  |  |
| Class E | 0.0% | 0.0% | -1.6% | -3.7% | -16.4% | -15.1% |  |  |  |
| Overall | -0.05% | -0.48% | -0.74% | -2.3% | -9.1% | -10.7% |  |  |  |
|  | -0.05% | -0.45% | -0.73% | -2.3% | -9.0% | -10.5% |  |  |  |
| Class F | -0.4% | -0.7% | -0.8% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 101% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | #NUM! |  |  |  |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.6% | -1.1% | -11.0% | -12.8% | -14.3% |  |  |  |
| Class C | -0.1% | -0.5% | -0.9% | -6.4% | -10.3% | -13.5% |  |  |  |
| Class D | -0.1% | -0.5% | -0.6% | -3.6% | -6.6% | -7.5% |  |  |  |
| Class E | -0.1% | -1.1% | -0.9% | -11.6% | -20.8% | -19.4% |  |  |  |
| Overall | -0.09% | -0.64% | -0.87% | -8.12% | -12.13% | -13.36% |  |  |  |
|  | -0.09% | -0.63% | -0.92% | -8.15% | -12.09% | -13.29% |  |  |  |
| Class F | -0.4% | -0.4% | -0.7% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 101% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | #NUM! |  |  |  |  |  |

Simulation Results for Test 3 (8 Classes per EO-Type+UnSignedOffset):

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.4% | -0.5% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class B | 0.1% | 0.0% | -0.4% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class C | 0.0% | -0.1% | -0.1% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | 0.0% | -0.1% | -0.1% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E | 0.1% | -0.5% | 0.0% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | 0.03% | -0.15% | -0.22% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
|  | 0.03% | -0.14% | -0.20% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class F | -0.25% | -0.11% | -0.21% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 103% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | #NUM! |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.3% | -0.8% | #VALUE! | #VALUE! | #VALUE! | 0.1% | 1.0% | 1.1% |
| Class B | 0.2% | 0.3% | -0.3% | #VALUE! | #VALUE! | #VALUE! | 0.0% | -0.1% | -0.6% |
| Class C | 0.1% | 0.3% | 0.3% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.10% | 0.13% | -0.10% | #VALUE! | #VALUE! | #VALUE! | 0.08% | 0.37% | 0.15% |
|  | 0.10% | 0.12% | -0.08% | #VALUE! | #VALUE! | #VALUE! | 0.09% | 0.35% | 0.14% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 101% |  |  | #NUM! |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | #NUM! |  |  | 105% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.2% | 0.5% | -0.1% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class C | 0.1% | 0.5% | 0.7% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | 0.1% | 0.2% | 0.3% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E | 0.2% | 0.0% | -0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | 0.17% | 0.33% | 0.17% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
|  | 0.16% | 0.37% | 0.18% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class F | -0.3% | 0.3% | 0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 101% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | #NUM! |  |  |  |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.2% | 0.4% | -0.2% |  |  |  |  |  |  |
| Class C | 0.2% | 0.5% | 0.3% |  |  |  |  |  |  |
| Class D | 0.1% | 0.6% | 0.4% |  |  |  |  |  |  |
| Class E | 0.3% | -0.2% | 0.3% |  |  |  |  |  |  |
| Overall | 0.19% | 0.38% | 0.18% |  |  |  |  |  |  |
|  | 0.19% | 0.41% | 0.17% |  |  |  |  |  |  |
| Class F | -0.2% | 0.2% | 0.6% |  |  |  |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 104% |  |  |  |  |  |  |  |  |

- Reference

[1]  D. Baylon and K. Minoo, “Coding and selection of SAO parameters,” JCTVC-G915, Nov. 2011.

- Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).