| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0559 WG 11Number : m23440 |
| --- | --- |

| Title: | Non-CE 7: Recent results for secondary transforms for intra/inter prediction residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Youji Shibahara Elena Alshina Felix C. Fernandes Takahiro Nishi | Tel: Email: | 1-9727617761 asaxena@sta.samsung.com, +81-50-3587-2044 shibahara.youji@jp.panasonic.com, elena_a.alshina@samsung.com felix.f@sta.samsung.com, nishi.takahiro@jp.panasonic.com |
| Source: | Samsung Electronics Co., Ltd & Panasonic |  |  |

_____________________________

# Abstract

Secondary transforms for intra and inter residue have been previously proposed in JCTVC-G108 and JCTVC-G632 respectively, and are being tested in Core Experiment 7. This contribution provides some new results related to the proposals, JCTVC-H0125 and JCTVC-H0126 on secondary transforms. Specifically, it is shown that the transform schemes in JCTVC-H0125 and JCTVC-H0126 provide similar or additional gains when integrated with various other tools such as SDIP in AhG 16, Transform Skip Mode in CE 5, and Data Hiding in CE 11. Also, we show that the complexity of the 4x4 secondary transform is negligible as compared to the 32x32 transform in HM, while it can provide more than half (roughly 60%-70 %) of the gain for Intra TU’s that a 32x32 transform is providing in HM.

# Introduction

JCTVC-H0125 [1] and JCTVC-H-0126 [2] present results as part of Core Experiment 7 for secondary transforms for intra and inter prediction residual respectively. Here we provide some additional results related to these proposals. A Working Draft description for the 4x4 secondary transform scheme for JCTVC-H0125 and JCTVC-H-0126 is also provided as an attachment.

# Tests on Secondary Transforms

Test 1: In this test, we test the compression efficiency of the 32x32 transform in HM 5.0. The anchor for this test is HM 5.0, while the tested scenario is HM 5.0 with 32x32 Transform Unit Off. Table 1 shows the summary of simulation results (average BD-Rates, and encoding/decoding times relative to HM 5.0 anchor) for Intra and Random Access settings. Detailed results are available in the attached Excel sheets.

Table 1. BD Rate gains when the 32x32 Transform Unit is switched Off in HM 5.0

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | 0.6% | 4.1% | 4.6% | 0.5% | 2.0% | 2.9% |
|  | Class B | 1.7% | 6.9% | 6.8% | 1.5% | 5.0% | 6.1% |
|  | Class C | 0.5% | 2.5% | 2.2% | 0.4% | 1.5% | 1.5% |
|  | Class D | 0.4% | 2.3% | 1.9% | 0.4% | 1.2% | 1.1% |
|  | Class E | 1.6% | 12.8% | 10.1% | 1.4% | 9.9% | 8.9% |
|  | Overall | 1.0% | 5.6% | 5.0% | 0.9% | 3.9% | 4.1% |
|  |  | 1.0% | 5.6% | 5.0% | 0.9% | 3.9% | 4.2% |
|  | Class F | 0.5% | 2.6% | 2.6% | 1.0% | 2.1% | 2.0% |
|  | Enc Time[%] | 95% |  |  | 93% |  |  |
|  | Dec Time[%] | 101% |  |  | 101% |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | 0.6% | 4.9% | 5.3% | 0.5% | 2.5% | 3.5% |
|  | Class B | 2.4% | 8.7% | 8.1% | 2.2% | 6.3% | 7.7% |
|  | Class C | 1.1% | 3.5% | 3.1% | 0.9% | 2.2% | 2.3% |
|  | Class D | 0.5% | 2.0% | 1.9% | 0.4% | 1.9% | 1.2% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 1.3% | 5.0% | 4.7% | 1.2% | 3.5% | 4.0% |
|  |  | 1.3% | 5.0% | 4.8% | 1.2% | 3.5% | 4.0% |
|  | Class F | 1.2% | 3.4% | 3.3% | 1.4% | 3.7% | 3.3% |
|  | Enc Time[%] | 91% |  |  | 94% |  |  |
|  | Dec Time[%] | 102% |  |  | 104% |  |  |

Test 2: In this test, we keep the anchor as HM 5.0. In the tested configuration, we set the 32x32 Transform Unit Off, and apply the 4x4 and 8x8 Mode-Dependent Secondary Transforms (for only intra residue) respectively from JCTVC-H0125. The 4x4 (or 8x8) secondary transform is used at sizes 8 and larger. Tables 2.1 and 2.2 show the summary of simulation results (average BD-Rates, and encoding/decoding times relative to HM 5.0 anchor) for Intra settings. Detailed results are available in the attached Excel sheet Test2_4.xls and Test2_8.xls.

Table 2.1 BD Rate gains when the 32 x 32 Transform Unit is switched off in HM 5.0, and 4x4 secondary transform is applied to intra residues. Anchor is HM 5.0.

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.3% | 3.3% | 3.7% | -0.6% | 1.3% | 2.4% |
|  | Class B | 0.9% | 6.0% | 6.0% | 0.5% | 4.4% | 5.5% |
|  | Class C | 0.1% | 2.0% | 1.8% | 0.0% | 1.1% | 1.2% |
|  | Class D | 0.1% | 2.0% | 1.6% | 0.1% | 0.9% | 0.9% |
|  | Class E | 0.7% | 11.5% | 8.8% | 0.4% | 8.8% | 8.0% |
|  | Overall | 0.4% | 4.8% | 4.3% | 0.2% | 3.3% | 3.6% |
|  |  | 0.4% | 4.8% | 4.3% | 0.2% | 3.4% | 3.7% |
|  | Class F | 0.4% | 2.4% | 2.2% | 0.4% | 1.8% | 1.7% |
|  | Enc Time[%] | 96% |  |  | 96% |  |  |
|  | Dec Time[%] | 101% |  |  | 104% |  |  |

Table 2.2: BD Rate gains when the 32 x 32 Transform Unit is switched off in HM 5.0, and 8x8 secondary transform is applied to intra residues. Anchor is HM 5.0.

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.7% | 2.6% | 3.0% | -1.0% | 0.8% | 1.7% |
|  | Class B | 0.7% | 5.3% | 5.2% | 0.3% | 3.8% | 4.9% |
|  | Class C | 0.0% | 1.7% | 1.5% | -0.1% | 0.9% | 0.9% |
|  | Class D | 0.0% | 1.8% | 1.4% | 0.0% | 0.8% | 0.7% |
|  | Class E | 0.5% | 10.8% | 8.2% | 0.2% | 8.2% | 7.4% |
|  | Overall | 0.2% | 4.3% | 3.8% | 0.0% | 2.9% | 3.1% |
|  |  | 0.2% | 4.4% | 3.8% | 0.0% | 3.0% | 3.2% |
|  | Class F | 0.4% | 2.3% | 2.2% | 0.9% | 1.8% | 1.8% |
|  | Enc Time[%] | 98% |  |  | 101% |  |  |
|  | Dec Time[%] | 103% |  |  | 108% |  |  |

From the above  tests, we see switching the 32x32 transform results in around 1.0 % and 0.9 % loss for Intra HE and LC setting. However, from test 2, we can retrieve back roughly 0.6 % and 0.7 % of this loss by simply applying a secondary transform after 8x8 and 16x16 TU’s. Further note that the 4x4 secondary transform has negligible complexity in comparison to the 32x32 transform. In addition, by applying a 8x8 secondary transform, we can obtain all the gains for Intra LC setting as well.

Tests 3 and 4: Integration of Mode-Dependent Secondary Transform (Intra tool) and SDIP

In these tests, we show that the mode-dependent secondary transform (which is applied on Intra TU’s only), when integrated with SDIP provides additional gains by applying it on rectangular SDIP blocks such as 2x8, 4x32 etc. as well. Here we set the anchor as HM 5.0+SDIP, and the tested configuration is HM 5.0 + SDIP + 4x4 secondary transform for Test 4, and HM5.0 + SDIP + 8x8 secondary transform for Test 5 respectively. Since, SDIP is primarily an Intra tool, we present the results for All Intra settings only. Table 3 shows the summary of simulation results for 4x4 secondary transform case, while Table 4 shows the summary for 8x8 secondary transform case (average BD-Rates, and encoding/decoding times relative to HM 5.0+SDIP anchor) for All Intra settings. Detailed results are available in the attached Excel sheets.

Table 3. BD Rate gains when the 4x4 secondary transform is applied to SDIP Case 2. Anchor is HM5.0+SDIP Case 2_2NxhN, and Tested scenario is HM 5.0 + SDIP + 4x4 secondary transform

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.8% | -0.5% | -0.5% | -1.1% | -0.4% | -0.3% |
|  | Class B | -0.6% | -0.4% | -0.5% | -0.8% | -0.3% | -0.3% |
|  | Class C | -0.4% | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% |
|  | Class D | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |
|  | Class E | -1.0% | -0.8% | -0.9% | -1.2% | -0.5% | -0.6% |
|  | Overall | -0.5% | -0.4% | -0.5% | -0.7% | -0.3% | -0.3% |
|  |  | -0.6% | -0.4% | -0.5% | -0.7% | -0.3% | -0.3% |
|  | Class F | 0.0% | -0.2% | 0.0% | 0.0% | -0.4% | -0.3% |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |
|  | Dec Time[%] | 101% |  |  | 101% |  |  |

Table 4. BD Rate gains when the 8x8 secondary transform is applied to SDIP Case 2. Anchor is HM5.0+SDIP Case 2_2NxhN, and Tested scenario is HM 5.0 + SDIP + 8x8 secondary transform

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -1.2% | -0.9% | -0.9% | -1.5% | -0.8% | -0.6% |
|  | Class B | -0.9% | -0.8% | -0.9% | -1.1% | -0.7% | -0.7% |
|  | Class C | -0.6% | -0.5% | -0.5% | -0.7% | -0.4% | -0.4% |
|  | Class D | -0.4% | -0.3% | -0.3% | -0.5% | -0.3% | -0.3% |
|  | Class E | -1.3% | -1.2% | -1.2% | -1.5% | -0.9% | -0.9% |
|  | Overall | -0.8% | -0.7% | -0.7% | -1.0% | -0.6% | -0.6% |
|  |  | -0.8% | -0.7% | -0.7% | -1.0% | -0.6% | -0.6% |
|  | Class F | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% |
|  | Enc Time[%] | 103% |  |  | 106% |  |  |
|  | Dec Time[%] | 103% |  |  | 104% |  |  |

From the above tests, we note that the gains for mode-dependent secondary transform in H-0125, which were 0.5 % and 0.6 % for Intra HE and LC case for the 4x4 case, are 0.5 % and 0.7 %, in the above case when applied to SDIP. Similarly for the 8x8 secondary transform case, the gains have increased from 0.7 % and 0.8% for Intra HE and LC settings to 0.8 % and 1.0 % respectively when applied to SDIP.

Test 5: Integration of Boundary-Dependent Transform Scheme (Inter tool) and Transform Skipping (TSM) (Inter tool)

In this test, we show that the compression gains of boundary dependent transform scheme, with 4x4 secondary transform (which is applied on Inter TU’s only), when integrated with Transform Skip Mode, are consistent as provided to HM 5.0. Table 5 shows the BD-Rate and PSNR performance of the integrated TSM software presented in CE 5g with HM 5.0 for anchor. For simplicity, we disable the scans and scan order macros in the integrated software.

Table 5. BD Rate gains for the TSM integrated software in CE 5g. Anchor is HM5.0.

Table 6 shows the BD-Rate and PSNR performance of the integrated TSM+ boundary dependent transform scheme from H-0126 with 4x4 secondary transform, and HM 5.0 for anchor. As in the previous case, for simplicity, we disabled the scans and scan order macros in the integrated software.

Table 6. BD Rate gains for the TSM integrated software in CE 5g + boundary-dependent secondary transform scheme in H-0126. Anchor is HM5.0.

From the above tests, we note that the boundary-dependent transform scheme of H-0126 provides almost the same compression gain when combined with TSM, as compared to when it is used on HM 5.0.

Tests 6 and 7: Integration of Mode-Dependent Secondary Transform (Intra tool) and Boundary-Dependent Transform Scheme (Inter tool) and Data Hiding (JCTVC-G271)

In these tests, we show that the mode-dependent secondary transform (which is applied on Intra TU’s only) and boundary-dependent transform scheme, when integrated with signed Data hiding (JCTVC-G271) being tested in CE 11, provides almost the same gain, as when they are applied on HM 5.0 alone. Here we set the anchor as HM 5.0+Data Hiding, and the tested configuration is HM 5.0 + Data Hiding + 4x4 secondary transform for Test 6, and HM5.0 + Data Hiding + 8x8 secondary transform for Test 7 respectively. Table 7 shows the summary of simulation results for the 4x4 secondary transform case, while Table 8 shows the summary for 8x8 secondary transform case (average BD-Rates, and encoding/decoding times relative to HM 5.0+Data Hiding anchor). Detailed results are available in the attached Excel sheets. We show the results for HE settings only, since these were being tested in CE 11 only.

Table 7. BD Rate gains for the HM5.0 + Data Hiding + Proposed Scheme with 4x4 secondary transform. Anchor is HM5.0 + Data Hiding

|  |  | All Intra HE |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.8% | -0.6% | -0.5% |
|  | Class B | -0.5% | -0.3% | -0.3% |
|  | Class C | -0.3% | -0.4% | -0.4% |
|  | Class D | -0.2% | -0.3% | -0.2% |
|  | Class E | -0.8% | -0.6% | -0.7% |
|  | Overall | -0.5% | -0.4% | -0.4% |
|  |  | -0.5% | -0.4% | -0.4% |
|  | Class F | -0.1% | -0.2% | -0.1% |
|  | Enc Time[%] | 100% |  |  |
|  | Dec Time[%] | 102% |  |  |
|  |  | Random Access HE |  |  |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.8% | -0.3% | -0.1% |
|  | Class B | -0.5% | -0.1% | -0.1% |
|  | Class C | -0.4% | -0.2% | -0.3% |
|  | Class D | -0.3% | 0.0% | -0.5% |
|  | Class E |  |  |  |
|  | Overall | -0.5% | -0.1% | -0.3% |
|  |  | -0.5% | -0.1% | -0.3% |
|  | Class F | -0.1% | 0.0% | 0.0% |
|  | Enc Time[%] | 100% |  |  |
|  | Dec Time[%] | 101% |  |  |
|  |  | Low delay B HE |  |  |
|  |  | Y | U | V |
|  | Class A |  |  |  |
|  | Class B | -0.5% | -0.3% | -0.7% |
|  | Class C | -0.4% | -0.3% | -0.6% |
|  | Class D | -0.4% | -0.1% | -0.4% |
|  | Class E | 0.0% | 1.2% | 0.4% |
|  | Overall | -0.4% | 0.0% | -0.4% |
|  |  | -0.4% | 0.0% | -0.4% |
|  | Class F | -0.4% | -0.4% | -0.2% |
|  | Enc Time[%] | 100% |  |  |
|  | Dec Time[%] | 100% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.5% | 0.2% | 0.5% |
|  | Class B | -0.5% | 0.0% | -0.3% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.5% | 0.1% | 0.0% |
|  |  | -0.5% | 0.0% | -0.1% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 100% |  |  |
|  | Dec Time[%] | 99% |  |  |

Table 8. BD Rate gains for the HM5.0 + Data Hiding + Proposed Scheme with 4x4 secondary transform. Anchor is HM5.0 + Data Hiding

|  |  | All Intra HE |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -1.2% | -1.0% | -1.0% |
|  | Class B | -0.7% | -0.7% | -0.8% |
|  | Class C | -0.4% | -0.5% | -0.6% |
|  | Class D | -0.3% | -0.3% | -0.3% |
|  | Class E | -1.1% | -1.1% | -1.1% |
|  | Overall | -0.7% | -0.7% | -0.7% |
|  |  | -0.7% | -0.7% | -0.7% |
|  | Class F | -0.1% | -0.1% | -0.2% |
|  | Enc Time[%] | 101% |  |  |
|  | Dec Time[%] | 104% |  |  |
|  |  | Random Access HE |  |  |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.9% | -0.2% | -0.1% |
|  | Class B | -0.7% | -0.3% | -0.2% |
|  | Class C | -0.5% | -0.4% | -0.3% |
|  | Class D | -0.4% | 0.0% | -0.5% |
|  | Class E |  |  |  |
|  | Overall | -0.6% | -0.2% | -0.3% |
|  |  | -0.6% | -0.2% | -0.3% |
|  | Class F | -0.2% | -0.2% | -0.2% |
|  | Enc Time[%] | 101% |  |  |
|  | Dec Time[%] | 102% |  |  |
|  |  | Low delay B HE |  |  |
|  |  | Y | U | V |
|  | Class A |  |  |  |
|  | Class B | -0.6% | -0.4% | -0.4% |
|  | Class C | -0.5% | -0.4% | -0.6% |
|  | Class D | -0.4% | 0.3% | -0.4% |
|  | Class E | -0.2% | 0.8% | 0.6% |
|  | Overall | -0.5% | 0.0% | -0.3% |
|  |  | -0.5% | 0.0% | -0.2% |
|  | Class F | -0.3% | -0.6% | -0.2% |
|  | Enc Time[%] | 101% |  |  |
|  | Dec Time[%] | 101% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.6% | 0.1% | 0.3% |
|  | Class B | -0.7% | -0.4% | -0.5% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.7% | -0.2% | -0.1% |
|  |  | -0.7% | -0.2% | -0.2% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 102% |  |  |
|  | Dec Time[%] | 100% |  |  |

From the above tests, we note that the gains for the integrated mode-dependent + boundary dependent secondary transform scheme in H-0126 had similar gains when tested on HM 5.0 + Data hiding, as compared to when it was tested on HM 5.0 alone in H-0126.

Test 8: This test simply checks the performance of the boundary-dependent scheme with 4x4 secondary transform in H-0126. The only difference is in implementation. Specifically, in H-0126, for a TU (for example 8x8) which shared only left boundary with a PU, at the encoder the order of flip, DCT and secondary transform was:

- Flip input

- Apply DCT

- Apply secondary transform.

In the implementation, we first simply determine a flag whether the flip is required. Then we choose the following logic:

- If (flipFlag){

In the DCT function, use the inputs in a flipped fashion

}

else{

Apply DCT

}

- Apply secondary transform

As expected, the BD-Rate/PSNR’s are identical as before and the encoding/decoding times are virtually unchanged in the following set of results in Table 10.

Table 9. BD Rate gains for the HM5.0 + boundary-dependent transform scheme in H-0126 with 4x4 secondary transform. Anchor is HM5.0.

# Conclusions

In this contribution, we have presented some additional results related to secondary transforms for the proposals JCTVC-H0125 and JCTVC-H0126 which are being tested as part of Core Experiment 7 on additional transforms. Specifically, we show that the transform schemes in JCTVC-H0125 and JCTVC-H0126 provide similar or additional gains when integrated with various other tools such as SDIP in AhG 16, Transform Skip Mode in CE 5, and Data Hiding in CE 11.

# References

1. A. Saxena, Y. Shibahara, F. Fernandes and T. Nishi, “On secondary transforms for intra prediction residual”, JCTVC-H0125, San Jose, USA, Feb 2012.

2. A. Saxena, Y. Shibahara, F. Fernandes and T. Nishi, “On secondary transforms for inter prediction residual”, JCTVC-H0126, San Jose, USA, Feb 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).