| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0561 |
| --- | --- |

| Title: | On CABAC Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current HEVC software/WD, each slice type (I/P/B) has its own initialization value set. This contribution proposes to share initialization value for contexts from different slice types to reduce the storage. Two methods are described. In the first method, P slice and B slice share the same initialization set. In the second method, for a subset of contexts, same initialization values are shared for all the three slice types. Experimental results under common test condition and common test condition with 1500 bytes slices are provided.

# Introduction

In current HEVC software/WD, each slice type (I/P/B) has its own initialization value set, and each set consists of around 200 values. In this contribution, it is proposed contexts in different slice types share the same initialization value to save the storage. Two different methods are described to achieve this goal.

# Proposed solutions

Method 1:

Based on the observation that syntax elements in inter slices tend to have similar statistics, it is proposed that I slice has its own initialization set while P slice and B slice share the same set of initialization values, as shown in Table 1. In the table, Init_Set_Intra and Init_Set_Inter represent respectively the I slice initialization value set and the B slice initialization set in HM5.

Table 1: Context sharing between different slice types

| Slice Type | I | P | B |
| --- | --- | --- | --- |
| Initialization Set | Init_Set_Intra | Init_Set_Inter | Init_Set_Inter |

Method 2:

In this method, a subset of contexts is defined. If belonging to this subset, the corresponding contexts in all the three slice types share the same initialization value. In our implementation, the common initialization values are from Init_Set_Inter. For other contexts outside the subset, the initialization values are still slice-type dependent (i.e., the same as HM5).  The defined subset includes the following syntax: skip flag, merge flag, merge index, AMPX position, AMPY position, inter prediction direction, mvd, reference frame index, deltaQP, root cbf flag, mvp index, level_greater_than_1, level_greater_than_2, ALF flag, ALF UVLC, ALF SVLC, SAO FLAG, SAO UVLC, SAO SVLC.

In additional to the two methods above, an alternative way of sharing initialization value set could be realized through cabac_init_idc. For each slice, cabac_init_idc is transmitted to signal the selection  (from the 3 initialization sets in HM). However, this may not reduce the storage of initialization value sets. In addition the syntax “cabac_init_idc” has been missing in HM software.

Table 1: Number of Initialization values

|  | HM5.0 | Method1 | Method2 |
| --- | --- | --- | --- |
| # of Initialization values (estimated) | 607 | 399 | 462 |

# Experimental results

The proposed methods are evaluated under common test conditions [1] and common test conditions with 1500 bytes slice. For the common test conditions, the platform is HM5.0. For slice coding, HM5.1-rc2 is used as the platform since slice mode in HM5.0 is broken.

## Results for method 1:

Table 3. Results of method 1 under common test condition

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.3% | 0.0% | 0.2% | 0.0% |
| Class D | 0.0% | 0.4% | 0.3% | 0.0% | -0.4% | 0.6% |
| Class E | 0.0% | -0.3% | -0.1% | 0.0% | -0.8% | -0.5% |
| Class F | 0.0% | 0.1% | 0.4% | 0.1% | -0.8% | 0.1% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | 0.1% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 4. Results of method 1 under common test condition with 1500bytes slice

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.6% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | -0.1% |
| Class E | 0.0% | -0.4% | -0.4% | 0.1% | -0.3% | 0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Results for method 2:

Table 5. Results of method 2 under common test condition

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | 0.4% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  | 97% |  |  | 88% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.1% | 0.0% | -0.3% | 0.1% |  |  |  |
| Class C | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class D | 0.0% | -0.3% | 0.0% | 0.0% | 0.5% | 0.0% |  |  |  |
| Class E | -0.1% | 0.5% | -0.1% | 0.0% | -0.6% | -0.1% |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class F | -0.2% | -0.2% | -0.3% | -0.1% | -0.9% | -0.2% |  |  |  |
| Enc Time[%] | 95% |  |  | 96% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

Table 6. Results of method 2 under common test condition with 1500bytes slice

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class F | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.2% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.3% | 0.1% | 0.2% | 0.0% |  |  |  |
| Class E | 0.0% | 1.4% | -0.6% | -0.1% | 0.2% | 0.5% |  |  |  |
| Class F | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% | -0.7% |  |  |  |
| Overall | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | 0.2% | -0.3% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% |  |  |  |
| Class D | 0.1% | -0.1% | -0.1% | 0.1% | -0.4% | -0.4% |  |  |  |
| Class E | 0.0% | 0.5% | -0.4% | 0.1% | 0.0% | 0.2% |  |  |  |
| Class F | 0.0% | -0.2% | -0.1% | 0.1% | 0.3% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | -0.1% |  |  |  |
|  | 0.0% | -0.1% | -0.2% | 0.1% | 0.2% | -0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusions

In current HEVC software/WD, each slice type (I/P/B) has its own initialization value set. This contribution proposed methods to allow contexts from different slice types sharing the same initialization values to reduce storage. Simulation results show that the proposed sharing has limited impact on coding performance.

# Draft Text

Method 1:

#### 9.2.1.1 Initialisation process for context variables

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P/B | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Error: Reference source not found | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..31 | 32..63 | 64..95 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..23 | 24..47 | 48..71 |

Method 2:

#### 9.2.1.1 Initialisation process for context variables

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 0..2 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..70..3 | 8..110..3 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 10 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 0..3 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 0..2 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 10 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 32 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 10 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 10 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Error: Reference source not found | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..31 | 32..630..31 | 64..950..31 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..23 | 24..470..23 | 48..710..23 |

# References

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).