| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0562 |
| --- | --- |

| Title: | Informal Subjective Quality Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini +, Nader Mahdi +, Hsan Guermazi +, Michael Horowitz +, Shilin Xu + Bin Li † Gary J. Sullivan * Jizheng Xu * | Tel: Email: | faouzi@ebriskvideo.com yhlibin@mail.ustc.edu.cn garysull@microsoft.com jzxu@microsoft.com |
| Source: | +eBrisk Video Inc., †University of Science and Technology of China, and *Microsoft Corp |  |  |

_____________________________

# Abstract

This contribution presents the results of an informal subjective quality comparison between the current state of the HEVC draft standard and the AVC High Profile (HP) for low-delay configurations, using five Class B (1080p) and Class E (720p) video sequences. More specifically, the study compares the high efficiency, low delay configuration of the HEVC reference software against a similarly configured AVC HP JM encoder. These tests are suggested to be complementary to the formal tests performed by AHG22, by focusing on low-delay rather than random-access configurations. The encoding methodology used here was reportedly as described in JCTVC-H0360 (very similar to that previously used for JCTVC-G399), since that methodology was reportedly designed to reflect relevant application scenarios and to enable a fair comparison to the maximum extent feasible, i.e. using comparable quantization settings, reference frame buffering, etc. Relative to JCTVC-G399, the HEVC software version was updated to version HM 5.0, the AVC software version was updated to version JM 18.2, and the JM configuration was updated to maximally reflect the non-normative aspects of the HM 5 common test configuration. When viewing HEVC encoded video side-by-side with AVC video encoded at approximately two times the HEVC average bit rate per sequence, in approximately 75% of the test cases and for four of the five tested encoding pairs, the (mostly non-expert) viewers reportedly indicated either that they had no preference, or that they preferred the HEVC encoded video. For three of the five encoding pairs, the number of people who expressed a preference for the HEVC encoding was substantially greater than (by a ratio of 1.7 to 1 or more) the number of people who expressed a preference for the AVC encoding (and in one of the two remaining cases, the number differed by only 1). The selection of which encoding was displayed on which side for the side-by-side viewing was established in a randomized double-blind fashion. It is suggested that the results generally confirm that the HEVC design (as represented by HM 5.0) exhibits a substantial improvement in compression capability beyond that of AVC (as represented by a similarly-configured JM 18.2) for low-delay video applications.

# Introduction

This contribution presents the results of a subjective quality comparison between the current state of the HEVC draft standard [1] and the AVC High Profile (HP) [5]. It is intended to complement the objective results presented in JCTVC-H0360 [4] (and previously in JCTVC-G399 [3]) by providing subjective quality assessments, to update the experiment to reflect newer software releases (HM 5.0 for HEVC and JM 18.2 for AVC), and to augment the subjective results collected by AHG22 by focusing on low-delay rather than random-access configurations. Here, the low-delay case is examined using the same basic methodology used in JCTVC-G399 [3] and JCTVC-H0360 [4] to ensure a fair comparison to the maximum extent feasible.

# Test conditions

In this section, the software version, encoder configuration and subjective evaluation conditions are discussed.

## Software configuration

The HEVC coded bitstreams were generated using the HM 5.0 software, following the testing conditions specified in JCTVC-G1200 [2], although not experimenting with all test sequences and using modified quantization parameter selections. The AVC HP bitstreams were generated using a modified version of the JM 18.2 software. The JM 18.2 changes, described in [4] and generally as previously described in detail in [3], include:

- P frames with a 1+3 reference frame setting as in HM 5.0 are employed in the JM 18.2 software, and

- The hierarchical QP assignment scheme in the JM 18.2 software was made the same as that used in HM 5.0.

## Subjective Evaluation Conditions

Key elements of the subjective evaluation methodology and video presentation are listed below:

1. One AVC and one HEVC decoded video sequence were presented side-by-side on a 42”, 1080p, Viera-class Panasonic plasma monitor in a darkened room.  The relative position of the AVC and HEVC video was randomized – sometimes the HEVC coded sequence was on the left and sometimes it was on the right.

2. The experiments were executed in a double blind manner (i.e., neither the viewers nor the facilitator knew which sequence was which).

3. Each video sequence was displayed at its native size, and because the two decoded video sequences to be evaluated would not fit on the monitor simultaneously, they were positioned so that the viewable region in each video sequence was identical.

4. The video sequences were played four times in immediate succession with a five-to-ten second pause between presentations and the viewing subjects, who sat between 2 and 5 feet from the plasma screen, were asked whether they had a preference regarding which video sequence had higher quality, and if so, which video sequence they preferred.

5. The viewing subjects were asked if the differences that led to their preference were very noticeable, noticeable, barely noticeable, or not noticeable. They were also offered an additional category of preference expression in which the subjects noted a difference between the two videos, but had no preference.

In the data summary (Table 2), the degree of differences that led to the preferences is not considered and the not-noticeable and no-preference choices were collected into a single no-preference group.

# Subjective Quality Assessment Results

The subjective evaluation was performed on three 720p video sequences (Vidyo1, Vidyo3 and Raven) and two 1080p video sequences (Kimono and ParkScene), all at target bit rates that are suggested to be typical for low delay video applications. For each resolution, a relatively “easy to encode” video sequence and a relatively “difficult to encode” sequence was selected. As shown in Table 1, the achieved bit rate for the HEVC bitstreams was approximately 50% that of the achieved bit rates for the AVC bitstreams.

The selection of which encoding was displayed on which side for the side-by-side viewing was established in a randomized double-blind fashion.

Table 1. Target and Achieved Bit Rates for the Test Bitstreams.

| Configuration | Class | Sequence | AVC Bit Rate (Mbps) | HEVC Bit Rate (Mbps) |
| --- | --- | --- | --- | --- |
|  |  |  | Target/Achieved | Target/Achieved |
| Low Delay | B (1080p) | Kimono | 2.0/2.07 | 1.0/1.02 (49.2% of AVC rate) |
|  |  | ParkScene | 2.0/1.94 | 1.0/0.96 (49.5% of AVC rate) |
|  |  | Vidyo3 | 1.0/1.10 | 0.5/0.56 (50.5% of AVC rate) |
|  |  | Vidyo1 | 1.0/1.08 | 0.5/0.55 (51.1% of AVC rate) |
|  |  | Raven | 2.5/2.54 | 1.25/1.27 (50.0% of AVC rate) |

Table 2. Subjective evaluation results.

| Configuration | Sequence | Number of Viewers | AVC | HEVC | No Preference |
| --- | --- | --- | --- | --- | --- |
| Low Delay | Kimono | 22 | 3 | 18 | 1 |
|  |  |  | 14% | 82% | 5% |
|  | ParkScene | 22 | 4 | 10 | 8 |
|  |  |  | 18% | 45% | 36% |
|  | Vidyo3 | 13 | 5 | 4 | 4 |
|  |  |  | 38% | 31% | 31% |
|  | Vidyo1 | 13 | 7 | 2 | 4 |
|  |  |  | 54% | 15% | 31% |
|  | Raven | 22 | 4 | 7 | 11 |
|  |  |  | 18% | 32% | 50% |
| Total |  | 92 | 23 | 41 | 28 |
|  |  | 100% | 25% | 45% | 30% |

# Observations

The subjective evaluation results in Table 2 show that, for the selected video sequences and bit rates, viewing subjects indicated, on average, a preference for the HEVC encoded video. More specifically, in 75% of the cases, the subjects indicated either that they had no preference or that they preferred an HEVC decoded video sequence.

For four of the five encoding pairs, the majority of the subjects (61% or more) indicated either that they had no preference or that they preferred an HEVC decoded video sequence.

For three of the five encoding pairs, the number of people who expressed a preference for the HEVC encoding was greater than or equal to the number of people who expressed a preference for the AVC encoding. In one of the two remaining cases, the number of test subjects indicating a preference for one encoding versus the other differed by only 1, which seems to suggest rough quality equivalence. For the one remaining sequence pair (the Vidyo1 sequence), the AVC encoded video (at twice the bit rate) seemed to have somewhat better quality.

It is suggested that the results generally confirm that the HEVC design (as represented by HM 5.0) exhibits a substantial improvement in compression capability beyond that of AVC (as represented by a similarly-configured JM 18.2) for low-delay video applications.

# Conclusions

These results suggest that HEVC generally produced better subjective quality when compared with AVC for these low-delay configurations, while maintaining an average bit rate that is approximately 50% of that of AVC. Although formal subjective testing would likely provide additional information, we find these subjective evaluation results encouraging.

# References

- JCTVC-G1103, “WD5: Working Draft 5 of High-Efficiency Video Coding” Geneva, November 2011.

- JCTVC-G1200, “Common test conditions and software reference configurations”, Geneva, November 2011.

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile”, Geneva, November 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile”, San José, CA, February 2012.

- ITU-T Rec. H.264 | ISO/IEC 14496-10, Advanced video coding (High Profile, 2004).