| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0563 |
| --- | --- |

| Title: | Non-CE6: Intra mode coding simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jianle Chen, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this proposal, intra mode coding is modified in order to signal the remaining modes by using fixed length codes. A third MPM is derived and an additional flag is signaled in the MPM modes coding branch. The proposed method shows 0.2% BD-rate saving as well as simplifies the intra mode coding.

# Introduction

In current HEVC design (HM5.0), the available intra prediction modes for Luma component are 18 ( from 0 to 17) for a 4×4 block and 35 for a block which is larger than 4×4. Fig.1 shows all intra mode numbers and associated angular prediction directions.  When signaling a chosen intra mode, 1 flag is coded to indicate whether MPMs or remained mode is used. For a mode in remained mode set an additional syntax rem_intra_luma_pred_mode is coded. In current HEVC design, there are 33 different values in the remaining modes set, a 5-bits fixed length code cannot sufficiently cover all values and the last two modes are represented with the escape code.

4

19

11

20

5

21

12

22

1

23

13

24

6

25

26

14

7

3:

Intra_DC

0:

Intra_Planar

27

15

28

8

29

16

30

2

31

17

32

9

33

18

34

10

Fig.1. Intra mode numbers and associated angular prediction directions.

# Proposed solution

In order to signal the remaining modes by fixed length codes for PU with size larger than 4x4, 3 modes are treated as most probable modes (MPMs) and separately coded from the remained 32 modes. The HOR+6 prediction mode is added to 4x4 since total 19 modes can be represented with 3 MPMs mode plus additional 4 bits for remained mode coding. The 3 MPMs derivation scheme proposed in [1] is also used in this contribution.

Fig. 2 shows the binarization scheme proposed in [1] and the red dots represent the bin that is context coded.

Instead of signal the 3rd MPM with the other remaining modes, it is proposed that all the MPMs are signaled separately from the remaining modes in this contribution. Fig. 3 illustrates the proposed binarization. With the proposed scheme, the longest codeword is reduced from 7 to 6 bins (one bypass coded bin for HM5.0 and one context coded bin for [1]) and thus the parsing throughput is improved.

MPM1

MPM2

MPM3

Remaining mode coding

Fig.2 Binarization of the intra mode coding in [1].

MPM3

Remaining mode coding

MPM1

MPM2

Fig.3 Binarization of the proposed intra mode coding

# Simulation results

The proposed method is implemented on HM 5.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM5.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.1% | 0.0% | -0.3% | -0.1% | -0.1% |
| Class B | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class D | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.3% | -0.1% | -0.1% | -0.3% | 0.0% | -0.1% |
| Overall | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
|  | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3.1 BD-rate saving of the proposed algorithm relative to HM5.0.

# Conclusions

In this contribution, the longest codeword is reduced and the remaining mode is signaled with fixed length codes by signaling additional falg in MPMs mode coding branch. The simulation results show that there is 0.2% BD-rate saving compared to HM5.0.

# Reference

[1] C. Yeo, Y. H. Tan, H. L. Tan, Z. G. Li, “CE6: Subtest 6c (6.2.1) – Intra mode coding simplification”, JCTVC-H0075, San Jose, Feb, 2012.

[2] F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, Dec, 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).