| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0565 WG11 Number: m23446 |
| --- | --- |

| Title: | AHG21: Short-term reference picture range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg | Tel: Email: | +46 761 26 35 91 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a proposal for a minor change in the reference picture set semantics regarding short-term reference pictures.

# Introduction and Problem Statement

In the latest WD of HEVC a reference picture set can contain both long-term reference pictures and short-term reference pictures. There are exact descriptions of how a short-term reference pictures can be changed into a long-term reference pictures. However, there is a problem with the current description. It is described that if there is no long-term reference picture picX in the DPB with pic_order_cnt_lsb equal to PocLtCurr[ i ] but there is a short-term reference picture picY in the DPB with pic_order_cnt_lsb equal to PocLtCurr[ i ] then picY is turned into a long-term picture. The problem is that picY is not necessarily uniquely defined as there might be more than one short term reference pictures in the DPB with pic_order_cnt_lsb equal to PocLtCurr[ i ].

# Proposed solution

We propose to add the restriction that DeltaPocS0[ idx ][ i ] shall be in the range of −1 to −MaxPicOrderCntLsb/2, inclusive and that DeltaPocS1[ idx ][ i ] shall be in the range of 1 to MaxPicOrderCntLsb/2 − 1, inclusive. That way there will only be one short-term reference picture with a specific value of pic_order_cnt_lsb in the DPB.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).