| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0566 WG11 Number: m23447 |
| --- | --- |

| Title: | Temporal layer access pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761 26 35 91 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a proposal to unify the signaling of Clean Random Access (CRA) pictures and Temporal Layer Switching Points in what is referred to as Temporal Layer Access (TLA) pictures. It is proposed to replace the CRA Network Abstraction Layer (NAL) unit type with a TLA NAL unit type. The TLA NAL unit type imposes constraints on the bitstream but does not in this contribution have any impact on the decoding process.

It is stated in the contribution that both random access information and temporal layer switching information is of high value to a network node and thus should be available in NAL unit header, independent of data outside that NAL unit header, specifically the Sequence Parameter Set (SPS) and Picture Parameter Set (PPS). It is further stated that a unified syntax and definition of TLA pictures makes the standard text more readable and comprehensive.

# Introduction and Problem Statement

In the current WD there are three distinct picture types defined; IDR-pictures, CRA-pictures and “Non-IDR and non-CRA”-pictures. While IDR pictures are handled differently in the decoding process, the difference between CRA-pictures and “Non-IDR and non-CRA”-pictures lies only in constraints on the bitstream.

A CRA-picture is used to indicate a “random access” point; a point from which a decoder can start to decode without having access to pictures that precedes the CRA-picture in decoding order. This can also be referred to as an “open-GOP” Intra picture.

It could be argued that information that has no effect on the decoding process should not be in the NAL unit header (and instead in some SEI message) but our opinion is that this information belongs in the NAL unit header since it is valuable information to a network node. It is an advantage if a network node is able to receive this information without having to have capabilities of parsing SPS, PPS or other fields with variable length codes and without having memory to monitor variables.

In this text, a temporal layer consists of all pictures with the same temporal_id and temporal scaling is the process of removing all pictures with temporal_id higher than a specific value.

We have made the following observations:

- We see no need to being able to signal CRA-pictures in any other temporal layer than temporal layer 0 since the first picture in temporal layer 0 that followed the CRA-picture in decoding order and display order would have to be an Intra picture. It is to be preferred to instead make that picture (the Intra picture in temporal layer 0) a CRA-picture, since that picture can not be removed in temporal scaling.

- Another property that is of high value to a network node is information of when it is possible to switch up to a higher temporal layer. This is referred to as a “temporal layer switching-point” and is currently signaled in the PPS.

- The description of a CRA-picture and a switching point picture can be unified by the inclusion of temporal_id in the definition of a CRA-picture; if it is in temporal layer 0 it is a Clean Random Access picture, otherwise it is a temporal layer random access picture.

## Explanation of unification of CRA and temporal layer switching point.

A temporal layer switching point is a picture that represents a point in the bitstream at which it is possible to start decoding a larger number of temporal layers than what was decoded before the switching point. That means, there is no picture following the switching point in both decoding order and display order that uses any picture that preceded the switching point in decoding order or display order.

Temporal layer switching points are included in the current WD with a “switch-from” definition, i.e. they indicate a point at which you can switch from the layer in which the indicator resides to a higher layer. We propose to use a “switch-to” definition of switching points, i.e. they indicate a point at which you can switch to the layer in which the indicator resides. The reason for using a "switch-to" definition is to enable signaling of CRA and switching points using the same NAL unit type as described below.

Figure 1	. Categorization of pictures compared to the current picture X.

Looking at Figure 1, all pictures have been categorized into three different groups of pictures. The horizontal axis in the figure represents output order and the vertical axis represents temporal layers, which means:

A contains all pictures in the same or higher temporal layer as X that precedes X in display order.

B contains all pictures in the same or higher temporal layer as X that follows X in display order.

C contains all pictures that have lower temporal_id than X.

One way to define a temporal layer switching point is to say that both these requirements must be fulfilled:

- X and all pictures in B shall not use any picture in A for prediction.

- No picture in B shall precede X in decoding order.

It would be possible to define a temporal layer switching point without the second rule, but the second rule ensures that a decoder that switches at X and one that already decoded both temporal layers will produce the same output from and including X.

The proposed definition of temporal layer switching points indicates a point at which it is possible to switch up one temporal layer, to the layer that contained the switching point, but it also indicates that it is possible to switch up an arbitrarily number of layers at that point, since none of the pictures in B may use any picture in A. This feature (indication of the possibility to switch an arbitrary number of layers) is also provided by the “switch-from” switching points as defined in the current WD. It is convenient for a network node to only have to check one NAL unit header in order to take decision on how many temporal layers to relay.

The above definition of temporal layer switching point does not depend on C or put any requirements on pictures in C. When X is in temporal layer 0, i.e. set C is empty; the definition of temporal layer switching point becomes exactly the same as the definition of a CRA-picture. This makes sense, as one way to look at a CRA-picture is as a point where it is possible to switch from decoding 0 temporal layers to decoding 1 or more (arbitrary many) temporal layers.

# Proposal

We propose to introduce the temporal layer access picture and change the definition of CRA pictures as follows:

temporal layer access (TLA) unit: An access unit in which the primary coded picture is a TLA picture.

temporal layer access (TLA) picture: A coded picture for which each slice has nal_unit_type equal to 4 that fulfills the requirement that the TLA picture and all coded pictures with temporal_id higher than or equal to the temporal_id of the TLA picture that follow the TLA picture both in decoding order and output order shall not use inter prediction from any picture with temporal_id higher than or equal to the temporal_id of the TLA picture that precedes the TLA picture either in decoding order or output order; and any picture with temporal_id higher than or equal to the temporal_id of the TLA picture that precedes the TLA picture in decoding order also precedes the TLA picture in output order.

clean random access (CRA) picture: A temporal layer access picture containing only I-slices for which temporal_id equals 0.

We propose that both clean random access pictures and temporal layer access pictures should be defined without normative effects on the decoding process.

For the semantics of nal_unit_type, the following is proposed to be added:

When the value of nal_unit_type is equal to 4 for a NAL unit containing a slice of a particular picture, the picture shall not contain NAL units with nal_unit_type equal to 1 or 5. [Ed note - all slices in a TLA must be TLA]

For the semantics of temporal_id, the following is proposed to be added:

When nal_unit_type is equal to 4 (TLA picture) and slice_type is not equal to 2, temporal_id shall not be equal to 0. [Ed note – this is equivalent to say that a TLA with temporal_id equal to 0 must consist of I slices only]

## Proposed semantics changes

No syntax changes are proposed. It is proposed to replace the CRA picture type.

Table 2-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR and non-CRA TLA picture slice_layer_rbsp( ) | VCL |
| 2-3 | Reserved | n/a |
| 4 | Coded slice of a CRA TLA picture slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13-23 | Reserved | n/a |
| 24..31 | Unspecified | non-VCL |

## Removal of current temporal layer switching point syntax and effects on picture marking

In [1] it was proposed to include temporal layer switching point information in PPS and introduce normative actions in the decoding process for temporal layer switching points, i.e. to mark all pictures in higher layers as “unused for reference” when a switching point picture has been decoded. The reference picture sets that were adopted in Geneva [2] made that temporal layer switching point decoding process obsolete. It is now indicated by the reference picture set what pictures should be marked as “unused for reference” and the temporal layer switching point information is purely informative. We propose to remove the current temporal layer switching point information in the picture parameter set. This includes removing the syntax and semantics of num_temporal_layer_switching_point_flags and temporal_layer_switching_point_flag. We propose to keep temporal_id_nesting_flag as is.

# Final remarks

We believe that it is an advantage to provide random access information as well as temporal layer switching point information in the NAL unit header. By replacing the CRA NAL unit type with a TLA NAL unit type the number of picture types is not increased and the additional bit cost for signal temporal layer switching point in the NAL unit header is 0.

We believe that the standard specification text becomes more readable and comprehensive when the concepts of random access and temporal layer switching are unified.

# References

- J. Boyce, D. Hong, S. Wenger, “Extensible High Layer Syntax for Scalability” JCTVC-E279, 5th. JCT-VC Meeting, Geneva, CH, 16-23 March, 2011.

- J. Boyce, R. Sjöberg, Y-K Wang, "JCT-VC break-out report: Reference picture buffering and list construction (AHG21)" JCTVC-G1002, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).