| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0568 WG11 Number: m23449 |
| --- | --- |

| Title: | AHG15: Specification of bitstream subsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg | Tel: Email: | +46 761 26 35 91 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document analyses some high level aspects of the HEVC Working Draft text and proposes to include text regarding bitstream conformance and specification of bitstream subsets. More specifically the following is proposed:

- Copy AVC/H.264 bitstream conformance section into the HEVC WD and split it into two conformance points

  - Output timing conforming bitstreams (copied from H.264/AVC)

  - Output order conforming bitstreams (as H.264/AVC but all parts related to timing removed)

- Copy SVC specification of bitstream subsets for temporal_id with the following changes

  - Separate output timing conformance from output order conformance and make output timing conformance optional

  - Add a sub-bitstream extraction process for removal of individual access units

- Make all information needed for sub-bitstream extraction available in fixed length without dependency on PPS or SPS

  - Temporal Layer Access pictures for defining temporal layer switching points (JCTVC-H0566)

  - Add a fixed length copy of picture order count into the access unit delimiters

# Problem Statement

During the second JCT-VC meeting in Geneva (July 2010) it was decided to inherit temporal_id from SVC to TMuC. In the fifth Working Draft text for HEVC (WD5) temporal_id is not used in the decoding process. Its only usage is for bitstream restrictions, i.e. the current picture can not use a picture with higher temporal_id for reference, and further restrictions might be implied by temporal_id_nesting_flag and temporal_layer_switching_point_flag. However, no specification of bitstream subsets is included in the WD. We propose to include a specification of bitstream subsets to HEVC. Further we propose to copy the bitstream conformance section from AVC/H.264 but separated it into two conformance points; output timing conformance and output order conformance.

We propose to add specification of bitstream subsets consisting of two alternative sub-bitstream extraction processes; one to be used for removal of entire temporal layers and one to be used for removal of individual access units. The first one is used for removal of entire temporal layers and it is basically copied from SVC but with unrelated parts removed (i.e. priority_id, quality_id, dependency_id). The second one is used for removal of individual access units and is targeting network nodes that perform adaptive thinning of a bitstreams depending on network conditions. Since both processes takes as input an output order conforming bitstream and produces as output an output order conforming bitstream the processes can be applied iteratively in arbitrary order still producing an output order conforming bitstream.

# Proposal for bitstream conformance

We propose to copy the AVC/H.264 HRD text for bitstream conformance (shown in black below) separated into two points of conformance (shown in red below).

C.3 Bitstream conformance

A bitstream of coded data conforming to this Recommendation | International Standard fulfils the following requirements.

The bitstream is constructed according to the syntax, semantics, and constraints specified in this Recommendation | International Standard outside of this annex.

There are two types of conformance that can be claimed by a bitstream: output timing conformance and output order conformance.

NOTE – All output timing conforming bitstreams are output order conforming. For each output order conforming bitstream there is at least one configuration of VUI parameters, buffering period SEI messages and picture timing SEI messages for which the bitstream is output timing conforming.

For output timing conformance tThe bitstream is tested by the HRD as specified below:

For Type I bitstreams, the number of tests carried out is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the vcl_hrd_parameters_present_flag. Each of these tests is conducted at the Type I conformance point shown in Error: Reference source not found.

For Type II bitstreams there are two sets of tests. The number of tests of the first set is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination. Each of these tests is conducted at the Type I conformance point shown in Error: Reference source not found. For these tests, only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

The number of tests of the second set, for Type II bitstreams, is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the nal_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the nal_hrd_parameters_present_flag. Each of these tests is conducted at the Type II conformance point shown in Error: Reference source not found. For these tests, all NAL units (of a Type II NAL unit stream) or all bytes (of a byte stream) are counted for the input bit rate and CPB storage.

NOTE 21 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Error: Reference source not found are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Error: Reference source not found for the same values of initial_cpb_removal_delay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found or item Error: Reference source not found of subclause Error: Reference source not found (depending on the profile in use) but also fall within the bounds set for VCL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found or item Error: Reference source not found of subclause Error: Reference source not found (depending on the profile in use), conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item Error: Reference source not found of subclause Error: Reference source not found.

For output timing conforming bitstreams, all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with Dtg,90( n ) specified by

Dtg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )		(C-1)

the value of initial_cpb_removal_delay[ SchedSelIdx ] shall be constrained as follows.

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( Dtg,90( n ) )	(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

Floor( Dtg,90( n ) ) <= 	initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( Dtg,90( n ) )	(C-3)

- NOTE 32 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is larger than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which tr,n( n ) is less than taf( n ). When low_delay_hrd_flag is equal to 0, the CPB shall never underflow.

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order), shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses Error: Reference source not found through Error: Reference source not found for the profile and level specified in the bitstream when decoding a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found.

- Immediately after any decoded picture is added to the DPB, the fullness of the DPB shall be less than or equal to the DPB size as constrained by Annexes Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found for the profile and level specified in the bitstream. At any time instant, there shall be at most max_dec_frame_buffering pictures in the DPB.

- All reference pictures shall be present in the DPB when needed for prediction. Each picture shall be present in the DPB at its DPB output time unless it is not stored in the DPB at all, or is removed from the DPB before its output time by one of the processes specified in subclause Error: Reference source not found.

- The value of Dto,dpb( n ) as given by Equation Error: Reference source not found, which is the difference between the output time of a picture and that of the first picture following it in output order and having OutputFlag equal to 1, shall satisfy the constraint expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream when decoding a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found.

For output order conforming bitstreams, all of the following conditions shall be fulfilled:

- Each access unit shall be smaller than or equal to the CPB size.

- At any time instant, there shall be at most max_dec_frame_buffering pictures in the DPB.

- All reference pictures shall be present in the DPB when needed for prediction.

- Each picture with output_flag equal to 1 shall be output from the DPB by the processes specified in subclause Error: Reference source not found4 in increasing order of PicOrderCntVal.

# Proposed specification of bitstream subsets

- We propose to add the following specification of bitstream subsets to HEVC.

- 10 Specification of bitstream subsets

Subclause 10.1 specifies the sub-bitstream extraction process for temporal layer removal.

Subclause 10.2 specifies the sub-bitstream extraction process for removal of individual access units.

10.1 Sub-bitstream extraction process for temporal layer removal

Input to this process is an output order conforming bitstream and a variable tIdTarget in the range of 0 to max_temporal_layers_minus1, inclusive.

Output of this process is an output order conforming bitstream. 
When the input to this process is an output timing conforming bitstream and output timing conformance of sub-bitstreams is claimed, the output bitstream shall be an output timing conforming bitstream.

The sub-bitstream is derived by applying the following operations in sequential order:

- Mark all VCL NAL units for which temporal_id is greater than tIdTarget as "to be removed from the bitstream".

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units that are marked as "to be removed from the bitstream".

10.2 Sub-bitstream extraction process for removal of individual access units

Input to this process is an output order conforming bitstream and a variable X in the range of 0 to N-1, inclusive where N is the number of access units in the bitstream.

Output of this process is an output order conforming bitstream. 
When the input to this process is an output timing conforming bitstream and output timing conformance of sub-bitstreams is claimed, the output bitstream shall be an output timing conforming bitstream.

Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

The sub-bitstream is derived by applying the following operations in sequential order:

- When there is an access unit containing a TLA picture or IDR picture with temporal_id lower than or equal to the temporal_id of access unit X that follows access unit X in output order, call that access unit Y.

NOTE 2 – access unit Y may precede access unit X in decoding order.

  - If there is an access unit Y, then for each access unit Z with temporal_id higher than or equal to the temporal_id of access unit X that follows access unit X in decoding order and precedes access unit Y in output order, mark all NAL units of access unit Z as "to be removed from the bitstream".

  - Otherwise (there is no access unit Y) then for each access unit Z with temporal_id higher than or equal to the temporal_id of access unit X that follows access unit X in decoding order, mark all NAL units of access unit Z as "to be removed from the bitstream".

- Remove all NAL units marked as "to be removed from the bitstream".

In the proposed sub-bitstream extraction process, output order of an access unit refers to the output order as defined by the PicOrderCntVal of the primary coded picture of that access unit and the temporal_id of an access unit refers to the temporal_id of the primary coded picture of that access unit.

# Information needed for bitstream extraction

To perform sub-bitstream extraction according to the process for removal of individual access units a network node must have access to the following information:

- temporal_id

- picture order count

- temporal layer switching point indication

We believe it would very advantageous if all this information could be exposed to a network node in fixed length code, at fixed bit positions in the NAL units independent of SPS and PPS. That would make it possible for a network node to perform decisions for sub-bitstream extraction based on simple bit masking operations instead of having to parse and store slice headers, PPSs and SPSs. Thus the complexity required by the network node would be vastly reduced.

To enable this we propose the following:

  - Include the Temporal Layer Access (TLA) pictures as proposed in JCTVC-H0566 [1]. This makes the temporal layer switching point information available in the NAL unit header. The TLA picture also provides a compact definition of temporal layer switching points that is used in the sub-bitstream extraction process for removal of individual access units. The TLA pictures also has the advantage that CRA pictures is a special case of TLA pictures and thus bitstreams starting with CRA pictures are output order conforming as long as the original sequence was output order conforming.

  - Mandate access unit delimiters containing a fixed length (32-bit) redundant copy of the picture order count value of the primary coded picture contained in the access unit. Having the picture order count available in the access unit delimiter is also useful for bitstream analysis, picture loss detection and to evaluate dependencies between pictures since picture order count is used for identification of reference pictures.

Proposed syntax:

7.3.2.5 Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| primary_pic_type | u(3) |
| pic_order_cnt | u(32) |
| rbsp_trailing_bits( ) |  |
| } |  |

pic_order_cnt indicates the picture order count for the primary coded picture contained in the access unit. pic_order_cnt is informative and does not affect the decoding process. pic_order_cnt shall be equal to PicOrderCntVal of the primary coded picture contained in the access unit.

# Discussion and examples

In the following examples the colours used to indicate different types of pictures and their corresponding access units are described in Figure 1.

Figure 1	. Colors used to indicate different types of pictures.

## Example 1

An example of sub-bitstream extraction process for removal of individual access units is shown in Figure 2.

Figure 2	. The labels inside the pictures denotes the order of access units in the bitstream (i.e. decoding order of pictures). Access units number 1, 2, 4 and 6 are removed in the bitstream extraction process for removal of individual access units.

If the variable X in the sub-bitstream extraction process for removal of individual access units is equal to 1, the access unit au1 (in dark red in Figure 2) shall be removed from the bitstream. Access units au2, au4 and au6 are also removed since they precede access unit au5 in output order, au5 holding the first TLA picture that follows au1 in output order with temporal_id lower than or equal to the temporal_id of au1 (note that au3 does not hold a TLA picture since it references a picture in the same temporal layer).

The sub-bitstream consisting of access units {0, 3, 5, 7, 8, 9, 10, 11, 12} is an output order conforming bitstream if the original bitstream {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} is an output order conforming bitstream.

## Example 2

Figure 3	. Random access example.

When a picture in the lowest temporal layer is removed from the bitstream in the sub-bitstream extraction process for removal of individual access units, all pictures that precede the next CRA (or IDR) picture in output order shall be removed. This is accomplished with the proposed sub-bitstream extraction process since, with the definition of TLA pictures from [1], CRA pictures is just a special case of TLA pictures (with temporal_id equal to 0).

# References

- J. Samuelsson, R. Sjöberg "AHG15: Temporal layer access pictures" JCTVC-H0566, 8th. JCT-VC Meeting, San Jose, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).