| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0568 WG11 Number: m23449 |
| --- | --- |

| Title: | AHG15: Specification of bitstream subsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg | Tel: Email: | +46 761 26 35 91 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

# Introduction / Problem Statement / …

During the second JCT-VC meeting in Geneva (July 2010) it was decided to inherit temporal_id from SVC to TMuC. In the fifth Working Draft text for HEVC (WD5) temporal_id is not used in the decoding process. Its only usage is for bitstream restrictions, i.e. the current picture can not use a picture with higher temporal_id for reference, and further restrictions might be implied by temporal_id_nesting_flag and temporal_layer_switching_point_flag. However, no process for sub-bitstream extraction is included in the WD. We propose to include a sub-bitstream extraction process to HEVC and present two options for that in this document of which the second option is our preference.

## Option 1: Sub-bitstream extraction process similar to SVC

SVC contains both priority_id, temporal_id, dependency_id and quality_id. HEVC does (at this point) only contain temporal_id. A sub-bitstream extraction process in line with the SVC solution could be specified like this for HEVC:

- G.8.8.1 Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with tIdTarget equal to any value in the range of 0 to 7, inclusive, shall be conforming to this Recommendation | International Standard.

NOTE – A conforming bitstream contains one or more coded slice NAL units with temporal_id equal to 0.

Input to this process  is a variable tIdTarget.

Output of this process is a sub-bitstream.

The sub-bitstream is derived by applying the following operations in sequential order:

- Mark all VCL NAL units for which temporal_id is greater than tIdTarget as "to be removed from the bitstream".

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units that are marked as "to be removed from the bitstream".

### Problems with Option 1.

There are some problems with Option 1;

- sub-bitstream extraction is limited to entire sequences

- encoder must evaluate bitstream conformance of each sub-bitstream

- encoder must hold different DPB statuses for the different sub-bitstream to ensure correct operation of the "decoder-in-the-encoder", including picture marking

- decoder must select (know) how many layers it decodes and that must be the same throughout the whole sequence*

- a network node that trims the bitstream must evaluate conformance of the trimmed bitstream since there is no guarantee of bitstream conformance unless the entire sequence is trimmed to a specific temporal layer according to the sub-bitstream extraction process.

* Assuming that it is possible to define different values of e.g. level_idc for different layers/sub-bitstreams, the decoder must know at each time instant how many layers it decodes (receives) to retain the correct DPB status.

## Option 2: A flexible sub-bitstream extraction process

We propose that different temporal layers can have different values of max_num_ref_frames and max_dec_frame_buffering for different layers/sub-streams as proposed in JCTVC-H0567 [1]. This allows for a decoder with limited DPB size to decode only the lower temporal layers.

To address the Option 1 problems and to provide a flexible solution for sub-bitstream extraction process, we propose the following sub-bitstream extraction process:

- 9 Sub-bitstream extraction process

Input to this process is an ordered list of integers au_num, in increasing order, with N elements, where N is in the range of 0 to the number of access units in the bitstream, inclusive. Each element au_num[i] in the list, for i in the range from 0 to N-1, has a value in the range from 0 to the number of access units in the bitstream.

Output of this process is a sub-bitstream that conforms to this Recommendation | International Standard.

NOTE 1 – Consider a hypothetical Recommendation | International Standard consisting of this Recommendation | International Standard with the Sub-bitstream extraction process subclause removed. All sequences that conforms to that hypothetical Recommendation | International Standard also conforms to this Recommendation | International Standard.

The sub-bitstream is derived by applying the following operations in sequential order:

- For each i in the range of 0 to N-1 the following applies:

  - Let integer j = au_num[i], let auX be the j'th access unit in decoding order and, when there is an access unit with a TLA picture or IDR picture with temporal_id lower than or equal to the temporal_id of auX that follows auX in output order, call that access unit auY..

NOTE 2 – auY may precede auX in decoding order.

  - If there is an access unit auY, then for each access unit auZ with temporal_id higher than or equal to the temporal_id of auX that follows auX in decoding order and precedes auY in output order, mark all NAL units of auZ as "to be removed from the bitstream".

  - Otherwise (there is no access unit auY) then for each access unit auZ with temporal_id higher than or equal to the temporal_id of auX that follows auX in decoding order, mark all NAL units of auZ as "to be removed from the bitstream".

- Remove all NAL units marked as "to be removed from the bitstream".

In the proposed sub-bitstream extraction process output order of access units refers to the output order as defined by the PicOrderCntVal of the primary coded picture of that access unit and the temporal_id of an access unit refers to the temporal_id of the primary coded picture of that access unit.

In order to support this sub-bitstream extraction process, we further propose the following:

- Include the Temporal Layer Access (TLA) pictures as proposed in JCTVC-H0566 [1] in order to have a single simple definition for dependency between pictures available at NAL unit header level.

- Include the syntax controlled output process as proposed in [1] in order to have an output process that does not depend on the max_dec_frame_buffering, which may be different for different layers/sub-bitstreams. A syntax controlled output process as proposed in [1], or something similar is needed to guarantee that each possible sub-bitstream generated by the sub-bitstream extraction process is a conforming bitstream. Example 3 below is the same as example 3 in [1] and it shows how it is possible to create a bitstream which would not be conforming if the "bumping" process is used instead of the proposed syntax controlled output process.

# Discussion and examples

In the following examples the colours used to indicate different types of pictures and their corresponding access units are described in Figure 1.

Figure 1	. Colors used to indicate different types of pictures.

## Example 1

The sub-bitstream extraction process is exemplified in Figure 2.

Figure 2	. The labels inside the pictures denotes the order of access units in the bitstream (i.e. decoding order of pictures). Access units number 1, 2, 4 and 6 are removed in the bitstream extraction process.

If the value 1 is in the au_num list, the access unit au1 (in dark red in Figure 2) shall be removed from the bitstream. Access units au2, au4 and au6 are also removed since they precede access unit au5 in output order, au5 holding the first TLA picture that follows au1 in output order with temporal_id lower than or equal to the temporal_id of au1 (note that au3 does not hold a TLA picture since it references a picture in the same temporal layer).

The sub-bitstream consisting of access units {0, 3, 5, 7, 8, 9, 10, 11, 12} is a conforming bitstream if the original bitstream {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} is a conforming bitstream.

## Example 2

Figure 3	. Random access example.

When a picture in the lowest temporal layer is removed from the bitstream, all pictures that precede the next CRA (or IDR) picture in output order shall be removed. This is accomplished with the proposed sub-bitstream extraction process since, with the definition of TLA pictures from [1], CRA pictures is just a special case of TLA pictures (with temporal_id equal to 0).

## Example 3

Figure 4	. Case where "bumping" fails.

Figure 4 shows a case where temporal layer switching does not work if "bumping" is used and different layers have different values of max_dec_frame_buffering. If the decoder starts out by only decoding the lowest temporal layer which uses only 2 reference frame, the picture in au2 will have been outputted before au4 is decoded. If the encoder then decides to switch to decoding more temporal layers (which is possible since that is a valid temporal layer switching point) au5 will be decoded but since the picture in au2 has already been outputted the sub-bitstream will not be conforming. Note that this problem is independent of what definition of temporal layer switching points is used ("switch-to" or "switch-from").

When the output process proposed in [1] is used the coding structure in example 3 will not cause a problem for temporal switching since the encoder will have to decide when the picture in au2 is output and the output status will be consistent among the layers.

# References

- J. Samuelsson, R. Sjöberg "AHG15: Syntax controlled output process" JCTVC-H0567, 8th. JCT-VC Meeting, San Jose, CA, USA, 1–10 February, 2012.

- J. Samuelsson, R. Sjöberg "AHG15: Temporal layer access pictures" JCTVC-H0566, 8th. JCT-VC Meeting, San Jose, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).