| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0570 |
| --- | --- |

| Title: | AHG14/AHG15/non-CE9: Loss robustness issue in TMVP disabling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Viktor Wahadaniah ChongSoon Lim Jizheng Xu Bin Li | Tel: Email: | (65) 6550 5415 viktor.wahadaniah@sg.panasonic.com |
| Source: | Panasonic Corporation Microsoft Corporation University of Science and Technology of China |  |  |

_____________________________

# Abstract

This contribution proposes to explicitly signal the enable_temporal_mvp_flag at every P and B slice header. It is purported that the benefits include improved error robustness and reduced decoder-side complexity. It is suggested that the JCT-VC consider this proposal in refining the design of TMVP disabling process.

# Introduction

The 7th JCT-VC meeting (Geneva, CH) adopted proposal JCTVC-G398 which provides a complete design to the TMVP disabling flag agreed previously in the 6th JCT-VC meeting (Torino, IT).

The adopted scheme introduces a enable_temporal_mvp_flag syntax element to the PPS. When a slice (or picture) refers to a PPS with enable_temporal_mvp_flag value equal to 0, the temporal motion vector predictor (TMVP) is not included in the set of motion vector predictors of every PU in the slice. In addition, all pictures in the DPB are marked as “unused for temporal motion vector prediction”. As a result of this marking process, temporal motion vector prediction dependency is reset to start from the last non-TMVP picture.

# Problem Statement

When a slice/picture which triggers the above marking process is lost or contains errors, a video decoder may not know that the marking operation should be done and the last non-TMVP picture should be updated. This leads to a mismatch between the encoder and decoder.

# Proposal

It is proposed to send the enable_temporal_mvp_flag at the slice headers of all P or B slices. This explicit method is conceptually similar to DPB management scheme using reference picture set, where DPB configuration is explicitly signalled in the bitstream instead of relying on frame-by-frame update of a buffer state.

TMVP is derived from the collocated picture, which is either RefPicList0[0] or RefPicList1[0] depending on the value of collocated_from_l0_flag. Therefore, a video encoder should keep track of the intended non-TMVP frame and should set the enable_temporal_mvp_flag accordingly to ensure that there is no TMVP dependency on the pictures preceding the non-TMVP frame in coding and display order.

The proposed method removes the need for a marking process and improves the error robustness of the TMVP disabling mechanism. The processing/intelligence burden is placed on the encoder for correctly setting the enable_temporal_mvp_flag, while the complexity of decoder implementation is reduced.

# Conclusion

This contribution proposes to explicitly signal the enable_temporal_mvp_flag at every P and B slice header. It is purported that the benefits include improved error robustness and reduced decoder-side complexity. It is suggested that the JCT-VC consider this proposal in refining the design of TMVP disabling process.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCTVC-G1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

- B. Li, J. Xu, H. Li, “High-level Syntax: Marking process for non-TMVP pictures,” JCTVC-G398, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

- B. Li, J. Xu, H. Li, “High-level Syntax: Temporal Information Decoding Refresh,” JCTVC-F201, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, IT, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed changes for HEVC WD5 (d9)

Proposed working draft text is included below using the following text conventions:

- Unchanged text that is not shown here is indicated with "…"

- Removed text is shown with strikethrough.

- Newly added text is shown in yellow highlight.

…

- 7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| enable_temporal_mvp_flag | u(1) |
| … |  |
| } |  |

…

- 7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| 5_minus_max_num_merge_cand | ue(v) |
| enable_temporal_mvp_flag | u(1) |
| } |  |
| … |  |
| } |  |

…

- 7.4.3	Slice header semantics

…

enable_temporal_mvp_flag specifies whether temporal motion vector predictor can be used or not. If enable_temporal_mvp_flag is equal to 1, temporal motion vector predictors are used; otherwise (enable_temporal_mvp_flag is equal to 0), the temporal motion vector predictors are not used. enable_temporal_mvp_flag equals to 0 together with temporal_id equals to 0 also indicates the marking process will occur before decoding the first picture with an output order greater than the current non-TMVP picture.   enable_temporal_mvp_flag equal to 0 together with temporal_id equal to 0 also indicate that any pictures that follow the current picture both in decoding order and output order shall not use temporal motion vector prediction from any picture that precedes the current picture either in decoding order or output order.

NOTE – a picture with enable_temporal_mvp_flag equal to 0 and temporal_id equal to 0 represents a refresh point where the propagation of temporal motion vector prediction is terminated. When a slice of a picture that follows this refresh point both in decoding order and output order has a collocated picture that precedes the refresh point either in decoding order or output order, enable_temporal_mvp_flag of the slice shall be set to 0.

…

- 8.2.4	Marking of reference pictures before decoding

This process is invoked once per picture, after decoding of a slice header and the decoding process for reference picture set as specified in Error: Reference source not found but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause Error: Reference source not found.

When enable_temporal_mvp_flag of the current picture is equal to 0 and temporal_id of the current picture is equal to 0, all the reference pictures in the DPB are marked as “unused for temporal motion vector prediction”.

…

- 8.4.2.1.8	Derivation process for temporal luma motion vector prediction

…

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If one of the following conditions is true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - colPu is coded in an intra prediction mode.

  - colPu is unavailable.

  - colPic is marked as “unused for temporal motion vector prediction”.

  - enable_temporal_mvp_flag is equal to 0.

…