| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0574_r1 WG11 Number: m23456 |
| --- | --- |

| Title: | CE10.4: deblocking parameters signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Kenneth Andersson, Jack Enhorn, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document is a proposal for signaling deblocking filter parameters in the slice header. It is proposed to signal three deblocking parameters: two that control which pixels are being filtered and one clipping parameter that determines the largest possible modification of the pixels in the weak filtering mode.

# Introduction

The output of the BoG G1035, which contained parameters signaling for the deblocking filter, has been adopted at Geneva meeting. The parameters can be signaled in either the slice header or APS (or both of them). It has been decided to signal two parameters,

# Proposed solution

The current signaling of the deblocking filter parameters in Working Draft 5.0 [1] in the slice header is shown in Table 1:

Table 1  Current syntax for deblocking parameters signaling in APS

| if (aps_deblocking_filter_flag) { |  |
| --- | --- |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |

Table 2 Current syntax for deblocking parameters signaling in slice header

| inherit_dbl_params_from_APS_flag | u(1) |
| --- | --- |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |

It is proposed to change signaling of deblocking filter parameters as in the following (the changes to the current signaling are marked in yellow):

Table 3  Current syntax for deblocking parameters signaling in APS

| if (aps_deblocking_filter_flag) { |  |
| --- | --- |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset | se(v) |
| tc_offset_div2 | se(v) |
| side_beta_offset_div4 | se(v) |
| } |  |

‘

Table 4 Current syntax for deblocking parameters signaling in slice header

| inherit_dbl_params_from_APS_flag | u(1) |
| --- | --- |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset | se(v) |
| tc_offset_div2 | se(v) |
| side_beta_offset_div4 | se(v) |
| } |  |
| } |  |

It is proposed to use a parameter  beta_offset  instead beta_offset _div2. This is done in order to allow for a better granularity of beta changes. It is also proposed a separate offset to side threshold parameter (side_beta_offset_div4). The side threshold is obtained from beta as approximately beta/6.  However, one can see that in order to change the side threshold parameter by one, the beta parameter should be increased by 6. Therefore, it is suggested to send a separate offset divided by 4 for the side beta value. This would also allow more flexibility for adapting the deblocking filtering since the beta parameter determines what edges are being filtered and the side beta parameter determines how many pixels from the block boundary are filtered. In

So, the following semantics of the deblocking filter control parameters is suggested (changes to the current semantics are marked in yellow):

IndexTC = Clip3(0, MAX_QP+2, QP + Tc_IntraOffset + tc_offset_div2<<1 );

IndexB = Clip3(0, MAX_QP, QP + beta_offset );

IndexSideBeta = Clip3(0, MAX_QP, iQP + side_beta_offset_div4<<2);

Beta = betatable[IndexB]*iBitdepthScale;

BetaSide = betatable_8x8[iIndexSideBeta]*iBitdepthScale;

iTc = tctable [iIndexTC]*iBitdepthScale;

SideThreshold = (iBetaSide+(iBetaSide>>1))>>3;

The divisors for the offsets are chosen based on the form of beta and tc tables in order to minimize the cost of signaling the offset while still allowing fine granularity of threshold values.

# Experimental results

Simple encoder control for deblocking parameter estimation has been implemented. The estimation of parameters is performed for every frame and is signaled in the slice header. The rate-distortion optimization has been implemented minimizing the cost, where Cost = SSD + λ*bits. The BDR results are obtained by comparing the rate-distortion performance of the proposal with the performance of the code distributed by Toshiba, where the deblocking parameters signaling adopted from G1035 was implemented on the base of HM5.0. This code is used as a reference for proposals in CE10.4.

The BDR results have been obtained for common conditions and two additional configurations mandatory for CE10[2], namely LD-P HE and LD-P LC. The decoding time has been obtained by decoding the anchor and the reference of a single computer. The encoding time was obtained on a computational cluster under different time periods with different load and therefore is not reliable.

Table 5 BDR performance of proposal relative to the CE10.4 reference (implementation of deblocking parameters signaling based on HM5.0).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | -0.1% | 0.0% | -0.6% | 0.0% | -0.1% |
| Class B | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.1% |
| Class D | -0.2% | -0.1% | 0.0% | -0.2% | 0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.0% | 0.0% | -0.3% | 0.1% | 0.0% |
|  | -0.2% | -0.1% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class F | -0.2% | 0.0% | -0.1% | -0.2% | 0.1% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.0% | 0.3% | -0.5% | -0.1% | 0.3% |
| Class C | -0.2% | -0.1% | 0.0% | -0.3% | 0.1% | 0.0% |
| Class D | -0.3% | -0.1% | 0.2% | -0.4% | 0.6% | 0.3% |
| Class E | -0.2% | -0.4% | 0.3% | -0.3% | 0.1% | 0.2% |
| Overall | -0.2% | -0.1% | 0.2% | -0.4% | 0.2% | 0.2% |
|  | -0.2% | -0.1% | 0.1% | -0.4% | 0.2% | 0.1% |
| Class F | -0.2% | 0.1% | -0.2% | -0.2% | -0.7% | -0.5% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.1% | 0.0% | -1.3% | 0.2% | -0.4% |
| Class C | -0.1% | 0.0% | -0.2% | -0.7% | 0.1% | 0.2% |
| Class D | -0.1% | 0.0% | 0.0% | -0.5% | -0.2% | 0.1% |
| Class E | -0.2% | -0.3% | -0.1% | -1.9% | -0.4% | -0.8% |
| Overall | -0.2% | 0.0% | -0.1% | -1.1% | 0.0% | -0.2% |
|  | -0.2% | 0.0% | -0.1% | -1.0% | 0.0% | -0.2% |
| Class F | 0.0% | 0.5% | 0.1% | -0.2% | 0.2% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| Random Access HE-10 |  |  |
| --- | --- | --- |
| Y | U | V |
| -0.2% | 0.2% | 0.1% |
| -0.2% | 0.0% | 0.0% |
| -0.2% | 0.1% | 0.0% |
| -0.2% | 0.1% | 0.0% |
| N/A |  |  |
| 100% |  |  |

# Conclusions

Two modifications to deblocking filter parameters in slice header and APS have been proposed. We propose to adopt this signaling to the Working Draft text and implement it in the reference software.

We also propose to include the encoder control for finding the deblocking filter parameters into the next version of HM.

# References

- A. Norkin, R. Sjöberg, K. Andersson, J. Enhorn, “Non-CE12: deblocking parameters signalling in slice header”, JCTVC-G619, Geneva, Nov. 2011.

- A. Norkin, X. Guo, B. Jeon, M. Narroschke, “Description of Core Experiment 10 (CE10): Deblocking Filtering”, JCTVC-G1210, Geneva, Nov. 2011.

- A. Norkin et al, “BoG report on resolving deblocking filter description issues”, JCTVC-G1035, Geneva, Nov. 2011.

# Working Draft

##### Filtering process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	variables dE[0], dE[1], dEp[0], dEp[1], dEq[0] and dEq[1] containing decisions,

–	a variable bSL,

–	a variable tCL,

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div_2<<1) )  as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*(bS – 1) + ( tc_offset_div2 << 1 ) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-2)

- For each m=0..1, if dE[m] is not equal to 0, for each sample location ( xC + xB, yC + yB + k + 4*m ), k = 0..3, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the decision dE[m], variables dEp[m] and dEq[m], the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary, and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +j, yC + yB + k + 4*m ] = qj’		(8-3)

s’[ xC + xB – i – 1, yC + yB + k + 4*m ] = pi’	(8-4)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -3 and 4 respectively. If xPOS is equal to 2, the parameters ks and ke are set to 0 and 4 respectively. Otherwise ks and ke are set to 0 and 7 respectively.

- The sample values pi,k and qi,k with i = 0..3 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-5)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-6)

- If xPOS is less than 0 [Ed. (WJ): maybe wrong condition] and dEL is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = -3..-1, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision arrays dEL, dEpL and dEqL [Ed. (WJ): dEpL and dEqL do not exist. Needs to be stored], the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + j ] = qj’		(8-7)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-8)

- For each m=0..1, if dE[m] is not equal to 0, for each sample location ( xC + xB + k + 4*m, yC + yB ), k = 0.. ke-4 , the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dE[m], variables dEp[m] and dEq[m], the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB + k + 4*m, yC + yB + j ] = qj’		(8-9)

s’[ xC + xB + k + 4*m, yC + yB – i – 1 ] = pi’

##### Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE[0], dE[1], dEp[0], dEp[1], dEq[0], and dEq[1] containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-10)

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

–	A variable  β1 is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( side_beta_offset_div_4<<2) )  as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS – 1) + ( tc_offset_div2 << 1) ) as input.

All elements of a one-dimensional array of size (4), dL are initialized to dL[0] = 0, dL[1] = 3, dL[2] = 4 and dL[3] = 7.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3, 4, 7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-11)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-12)

- The elements of one dimensional arrays of size (),dp, dq and dpq are derived as follows with i=0..3:

dp[i] = | p2,dL[i] – 2*p1,dL[i] + p0,dL[i] |		(8-13)

dq[i] = | q2,dL[i] – 2*q1,dL[i] + q0,dL[i] |		(8-14)

dpq[i] = dp[i] + dq[i]		(8-15)

- The elements of the one dimensional array d are derived as follows with i=0..1:

d[i] = dpq[ 2*i ] + dpq[ 2*i + 1 ]		(8-16)

- All elements of the one dimensional arrays dE, dEp and dEq are set equal to 0.

- For each j=0..1, if d[j] is less than β, the following ordered steps apply:

- For each sample location ( xC + xB, yC + yB + dL[k] ), k = 0..1, the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,dL[k], qi,dL[k] with i = 0..3, the variables 2*dpq[ 2*j + k ], β and tC as inputs and a decision dSam[k] as output.

- The variable dE[j] is set equal to 1.

- If dSam[0] is equal to 1 and dSam[1] is equal to 1, the variable dE[j] is set equal to 2.

- If dp[ 2*j ] + dp[ 2*j + 1 ] is less than ( β1 + ( β1 >> 1 ) ) >> 3, the variable dEp[j] is set equal to 1.

- If dq[ 2*j ] + dq[ 2*j + 1 ] is less than ( β1 + ( β1 >> 1 ) ) >> 3, the variable dEq[j] is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3, 4, 7 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-17)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-18)

- The elements of one dimensional array of size (4), dp, dq and dpq are derived as follows with i=0..3:

dp[i] = | p2,dL[i] – 2*p1,dL[i] + p0,dL[i] |		(8-19)

dq[i] = | q2,dL[i] – 2*q1,dL[i] + q0,dL[i] |		(8-20)

dpq[i] = dp[i] + dq[i]		(8-21)

- The elements of the one dimensional array d are derived as follows with i=0..1:

d[i] = dpq[ 2*i ] + dpq[ 2*i + 1 ]		(8-22)

- All elements of the one dimensional arrays dE, dEp and dEq are set equal to 0.

- For each j=0..1, if d[j] is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + dL[k] + 4 * j, yC + yB ), k = 0..1, the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,dL[k], qi,dL[k] with i = 0..3, the variables 2*dpq[ 2*j + k ], β and tC as inputs and a decision dSam[k] as output.

- The variable dE[j] is set equal to 1.

- If dSam[0] is equal to 1 and dSam[1] is equal to 1, the variable dE[j] is set equal to 2.

- If dp[ 2*j ] + dp[ 2*j + 1 ] is less than ( β1 + ( β1 >> 1 ) ) >> 3, the variable dEp[j] is set equal to 1.

- If dq[ 2*j ] + dq[ 2*j + 1 ] is less than ( β1 + ( β1 >> 1 ) ) >> 3, the variable dEq[j] is set equal to 1.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).