| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0574_r1 WG11 Number: m23456 |
| --- | --- |

| Title: | CE10.4: deblocking parameters signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Kenneth Andersson, Jack Enhorn, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document is a proposal for signaling deblocking filter parameters in the slice header. It is proposed to signal three deblocking parameters: two that control which pixels are being filtered and one clipping parameter that determines the largest possible modification of the pixels in the weak filtering mode.

# Introduction

The output of the BoG G1035, which contained parameters signaling for the deblocking filter, has been adopted at Geneva meeting. The parameters can be signaled in either the slice header or APS (or both of them). It has been decided to signal two parameters,

# Proposed solution

The current signaling of the deblocking filter parameters in Working Draft 5.0 [1] in the slice header is shown in Table 1:

Table 1  Current syntax for deblocking parameters signaling in APS

| if (aps_deblocking_filter_flag) { |  |
| --- | --- |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |

Table 2 Current syntax for deblocking parameters signaling in slice header

| inherit_dbl_params_from_APS_flag | u(1) |
| --- | --- |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |

It is proposed to change signaling of deblocking filter parameters as in the following (the changes to the current signaling are marked in yellow):

Table 3  Current syntax for deblocking parameters signaling in APS

| if (aps_deblocking_filter_flag) { |  |
| --- | --- |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset | se(v) |
| tc_offset_div2 | se(v) |
| side_beta_offset_div4 | se(v) |
| } |  |

‘

Table 4 Current syntax for deblocking parameters signaling in slice header

| inherit_dbl_params_from_APS_flag | u(1) |
| --- | --- |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset | se(v) |
| tc_offset_div2 | se(v) |
| side_beta_offset_div4 | se(v) |
| } |  |
| } |  |

It is proposed to use a parameter  beta_offset  instead beta_offset _div2. This is done in order to allow for a better granularity of beta changes. It is also proposed a separate offset to side threshold parameter (side_beta_offset_div4). The side threshold is obtained from beta as approximately beta/6.  However, one can see that in order to change the side threshold parameter by one, the beta parameter should be increased by 6. Therefore, it is suggested to send a separate offset divided by 4 for the side beta value. This would also allow more flexibility for adapting the deblocking filtering since the beta parameter determines what edges are being filtered and the side beta parameter determines how many pixels from the block boundary are filtered. In

So, the following semantics of the deblocking filter control parameters is suggested (changes to the current semantics are marked in yellow):

IndexTC = Clip3(0, MAX_QP+2, QP + Tc_IntraOffset + tc_offset_div2<<1 );

IndexB = Clip3(0, MAX_QP, QP + beta_offset );

IndexSideBeta = Clip3(0, MAX_QP, iQP + side_beta_offset_div4<<2);

Beta = betatable[IndexB]*iBitdepthScale;

BetaSide = betatable_8x8[iIndexSideBeta]*iBitdepthScale;

iTc = tctable [iIndexTC]*iBitdepthScale;

SideThreshold = (iBetaSide+(iBetaSide>>1))>>3;

The divisors for the offsets are chosen based on the form of beta and tc tables in order to minimize the cost of signaling the offset while still allowing fine granularity of threshold values.

# Experimental results

Simple encoder control for deblocking parameter estimation has been implemented. The estimation of parameters is performed for every frame and is signaled in the slice header. The rate-distortion optimization has been implemented minimizing the cost, where Cost = SSD + λ*bits. The BDR results are obtained by comparing the rate-distortion performance of the proposal with the performance of the code distributed by Toshiba, where the deblocking parameters signaling adopted from G1035 was implemented on the base of HM5.0. This code is used as a reference for proposals in CE10.4.

The BDR results have been obtained for common conditions and two additional configurations mandatory for CE10[2], namely LD-P HE and LD-P LC. The decoding time has been obtained by decoding the anchor and the reference of a single computer. The encoding time was obtained on a computational cluster under different time periods with different load and therefore is not reliable.

Table 5 BDR performance of proposal relative to the CE10.4 reference (implementation of deblocking parameters signaling based on HM5.0).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | -0.1% | 0.0% | -0.6% | 0.0% | -0.1% |
| Class B | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.1% |
| Class D | -0.2% | -0.1% | 0.0% | -0.2% | 0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.0% | 0.0% | -0.3% | 0.1% | 0.0% |
|  | -0.2% | -0.1% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class F | -0.2% | 0.0% | -0.1% | -0.2% | 0.1% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.0% | 0.3% | -0.5% | -0.1% | 0.3% |
| Class C | -0.2% | -0.1% | 0.0% | -0.3% | 0.1% | 0.0% |
| Class D | -0.3% | -0.1% | 0.2% | -0.4% | 0.6% | 0.3% |
| Class E | -0.2% | -0.4% | 0.3% | -0.3% | 0.1% | 0.2% |
| Overall | -0.2% | -0.1% | 0.2% | -0.4% | 0.2% | 0.2% |
|  | -0.2% | -0.1% | 0.1% | -0.4% | 0.2% | 0.1% |
| Class F | -0.2% | 0.1% | -0.2% | -0.2% | -0.7% | -0.5% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.1% | 0.0% | -1.3% | 0.2% | -0.4% |
| Class C | -0.1% | 0.0% | -0.2% | -0.7% | 0.1% | 0.2% |
| Class D | -0.1% | 0.0% | 0.0% | -0.5% | -0.2% | 0.1% |
| Class E | -0.2% | -0.3% | -0.1% | -1.9% | -0.4% | -0.8% |
| Overall | -0.2% | 0.0% | -0.1% | -1.1% | 0.0% | -0.2% |
|  | -0.2% | 0.0% | -0.1% | -1.0% | 0.0% | -0.2% |
| Class F | 0.0% | 0.5% | 0.1% | -0.2% | 0.2% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| Random Access HE-10 |  |  |
| --- | --- | --- |
| Y | U | V |
| -0.2% | 0.2% | 0.1% |
| -0.2% | 0.0% | 0.0% |
| -0.2% | 0.1% | 0.0% |
| -0.2% | 0.1% | 0.0% |
| N/A |  |  |
| 100% |  |  |

# Conclusions

Two modifications to deblocking filter parameters in slice header and APS have been proposed. We propose to adopt this signaling to the Working Draft text and implement it in the reference software.

We also propose to include the encoder control for finding the deblocking filter parameters into the next version of HM.

# References

- A. Norkin, R. Sjöberg, K. Andersson, J. Enhorn, “Non-CE12: deblocking parameters signalling in slice header”, JCTVC-G619, Geneva, Nov. 2011.

- A. Norkin, X. Guo, B. Jeon, M. Narroschke, “Description of Core Experiment 10 (CE10): Deblocking Filtering”, JCTVC-G1210, Geneva, Nov. 2011.

- A. Norkin et al, “BoG report on resolving deblocking filter description issues”, JCTVC-G1035, Geneva, Nov. 2011.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).