| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0576 WG11 Number: m23458 |
| --- | --- |

| Title: | CE10.3: Cross-verification of proposal on deblocking Bs simplification from LG (H0108) by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution is a cross-verification of LG’s proposal H0108 on Bs simplification. The cross-verifier confirms the BDR and decoding time results are similar.

# Software and cross-verification

The software has been compiled successfully with gcc 4.3.2 on Linux and Visual Studio for Windows. The software has been studied and found to correspond to the description provided in H0108.

Currently, in HM5.0 the deblocking decisions are done for each 4-pel part of the block boundary. However, the Bs is determined for the 8-pel boundary as a maximum of Bs of the first 4-pel part of the block boundary and Bs of the second 4-pel part of the block boundary. It is proposed in H0108 to use the Bs of the first part of the block boundary for the whole 8-pel edge (for both the 4-pel part and the second 4-pel part). It is proposed therefore to skip calculation of Bs for the second 4-pel part of the block boundary.

The changes in the software are turned on by setting the following macro-definition:

#define BS_SIMPLIFICATION					1

The software has been studied and found to correspond to the description in H0108.

# Objective performance

The objective performance of the cross-verified method is shown in the table below. The scheme has been compared against the HM5.0 anchor. The comparison results are reported in Table 1. The obtained BDR results match the results for common conditions [2] provided by the proponents.

In order to evaluate the complexity changes of G086, the decoding time on a single computer has been measured and compared to that of the anchor. The time has been obtained without writing decoded pictures on the disk in order to minimize the influence of the I/O time. The results for the decoding time on a single computer are also provided in Table 1.

Table 1 Comparison of H0108 with HM5.0 as anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | -0.4% | 0.2% | 0.0% | 0.7% | 0.0% |
| Class E | 0.0% | 0.1% | -0.1% | 0.1% | -0.2% | -0.1% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class F | -0.1% | 0.0% | -0.2% | -0.3% | -0.3% | -0.5% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% | 0.1% | 0.3% | -0.3% |
| Class C | 0.1% | 0.1% | -0.1% | 0.1% | 0.3% | 0.2% |
| Class D | 0.0% | 0.1% | -0.3% | 0.0% | -0.1% | 0.2% |
| Class E | 0.0% | -0.2% | 0.3% | 0.1% | -0.9% | -0.7% |
| Overall | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% |
|  | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% | -0.1% |
| Class F | 0.0% | 0.2% | 0.2% | -0.1% | -0.2% | -0.3% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

| Random Access HE-10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.0% | -0.1% | -0.2% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | -0.1% |
| 0.0% | 0.0% | 0.0% |
| N/A |  |  |
| 99% |  |  |

# Subjective performance

The subjective quality of the proposed method has been tested against the HM5.0 anchor. An informal blind test was used so that the identities of the sequences shown in motion one after another were not known before the decision is taken, which sequence is better. The sequences from the subjective viewing test were used. When watching the sequences in motion, one can notice differences between the anchor and the proposal. However, it is difficult to say whether the proposal is consistently worse than the anchor.

# References

- S. Park, B. Jeon, “CE10 Subtest3: Simplified BS calculation process in deblocking filter”, JCTVC-G0108, Geneva, November 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F1200, Geneva, Nov. 2011.

- A. Norkin, X. Guo, B. Jeon, M. Narroschke, “Description of Core Experiment 10 (CE10): Deblocking Filtering”, JCTVC-G1210, Geneva, Nov. 2011.

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.