| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0578 |
| --- | --- |

| Title: | AHG6: Inspection report on deblocking of IPCM Blocks containing reconstructed samples (JCTVC-H0448) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

- Abstract

JCTVC-H0448 introduces the following two modifications into the current qp derivation process for deblocking filter:

- If pcm_loop_filter_disable_flag is 0, use the reconstructed qp value instead of 0 for I_PCM block, like the other blocks coded by non I_PCM.

- If pcm_loop_filter_disable_flag is 1 and only one of given a neighboring two blocks is I_PCM block, use the qp value of the non I_PCM block.

Through the inspection of the software and WD text provided by the proponent, it is confirmed that they correctly reflect the two modifications.

- Reference

- Geert Van der Auwera et al., "AHG6: Deblocking of IPCM Blocks Containing Reconstructed Samples," JCTVC-H0448, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1 - 10 February, 2012.